3GPP TSG RAN meeting #97-e	 RP-222561

Online, September 12-16, 2022

## Status Report to TSG

Agenda item: 10.2.1 IoT (Internet of Things) NTN (non-terrestrial network) enhancements

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | IoT_NTN_enh |  |  |  |  |
| Unique ID | 941004 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220979 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2023 | Performance part: 12/2023 | Testing part: - |  |
| Overall Completion level | Study Item: | Core part: Overall: 10% For information RAN1: 20% RAN2: 10% RAN3: 5% RAN4: 0% | Performance Part: 0% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Abhishek Roy |
|  | Company | MediaTek |
|  | Email | abhishek.roy@mediatek.com |
| Secondary WG |  | RAN2 |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

###### RAN1#110, August 22nd – 26th, 2022, Toulouse, France

Agreements on “9.12.3 Disabling of HARQ feedback for IoT NTN”

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options:

- Option 1: per HARQ process via UE specific RRC signaling.

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field).

- Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS)

- Option 6: combinations of some options above.

Agreement

- For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options:

- Option 1: per HARQ process via UE specific RRC signaling

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

- Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS)

- Option 6: combinations of some options above

Agreement

- For a DL HARQ process with disabled HARQ feedback in NB-IoT, at least the following UE behavior(s) can be considered:

- Option 1: UE is not expected to receive another NPDCCH carrying a DCI scheduling a NPDSCH for a given HARQ process that starts until X(ms) after the end of the reception of the last NPDSCH for that HARQ process.

  - X =12

- Option 2: UE is not required to monitor NPDCCH in a period of Y(ms) from the end of reception of the last NPDSCH

  - Y=12

- Note: it may be different UE behaviors for different UE categories (e.g., UE with single/multiple HARQ processes)

Agreements on “9.12.4	Improved GNSS operations for IoT NTN”

Agreement

GNSS assistance information that UE reports to eNB at least consists of:

- GNSS position fix time duration for measurement

- GNSS validity duration

Agreement

When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix

- FFS details of signalling

- FFS how UE reports GNSS assistance information after eNB trigger and the detailed content

- Note: further discuss whether a UE is expected to handle all eNB triggers

#### Remaining Open issues

Objective 1 (IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17):

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates.

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption.

## 2.2	RAN2

#### 2.2.1	Agreements

###### RAN2#119-e, August 17th – 26th, 2022, e-meeting

Agreements on “8.6.2 Performance Enhancements”

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

- For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

Agreements on “8.6.3 Mobility Enhancements”

Agreements:

- IoT NTN can use the mechanism for neighbor cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g., triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g., the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- CHO enhancements for eMTC NTN (i.e., time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

#### 2.2.2	Remaining Open issues

Objective 1 (IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17):

- RAN2 aspects for Disabling of HARQ feedback in NB-IoT NTN to mitigate impact of HARQ stalling.

- Further details on disabling HARQ feedback in DL for both NB-IoT and eMTC based NTN.

- Further details on HARQ mode A and HARQ mode B for UL HARQ operation in IoT NTN.

Objective 2 (mobility enhancements):

- Possible enhancements (e.g., triggers) for neighbour cell measurements in connected mode (for both NB-IoT and eMTC).

- Possible CHO enhancements for eMTC-based IoT-NTN.

Objective 3 (further enhancement to discontinuous coverage): No discussion in RAN2 119bis-e (Oct) meeting, waiting for some progress in SA2 Study.

## 2.2	RAN3

#### 2.3.1	Agreements

###### RAN3#117-e, August 15th – 24th, 2022, e-meeting

Agreements on “18.2 Support discontinuous coverage”

- There is no need to provide the ephemeris info over S1.

- Whether MME shall be aware of the coverage should be decided by SA2.

- Paging enhancement for power saving should wait for the progress of RAN2 or SA2.

- The mobility management enhancements for discontinuous coverage should be pending to the progress in RAN2.

#### 2.3.2	Remaining Open issues

- UE context release procedure due to discontinuous coverage

- The issues of discontinuous coverage finalized by SA2 if any.

## 2.4	RAN4

#### 2.4.1	Agreements:

The work has not started yet.

#### 2.4.2	Remaining Open issues:

NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements

- Specify UE RRM performance requirements to support the agreed mobility enhancements for NB-IoT/eMTC [RAN4]

- Specify UE and Base Station demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC [RAN4]

NOTE: The need for Performance requirements for improved GNSS operations will be established once the need for specification work has been decided.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

RAN1#109-e, May 9th – 20th, 2022, e-meeting

Submitted TDocs to AI 9.12.2

- R1-2205831	Remaining Issues for HARQ	Lockheed Martin

- R1-2205857	Discussion on disabling of HARQ feedback for IoT NTN	Huawei, HiSilicon

- R1-2206010	Discussion on disabling of HARQ feedback for IoT NTN	Spreadtrum Communications

- R1-2206022	Discussion on disabling of HARQ feedback for IoT-NTN	ZTE

- R1-2206135	On disabling HARQ feedback for IoT-NTN	Sony

- R1-2206139	Disabling of HARQ for IoT NTN	MediaTek Inc.

- R1-2206312	Discussion on disabling of HARQ feedback for IoT NTN	OPPO

- R1-2206388	Discussion on disabling of HARQ feedback for IoT NTN	CATT

- R1-2206480	Disabling of HARQ feedback for IoT NTN	NEC

- R1-2206632	Discussion on HARQ operation for IoT NTN	Xiaomi

- R1-2206850	Disabling of HARQ feedback for IoT NTN	Samsung

- R1-2206881	Disabling of HARQ feedback for IoT NTN	Nordic Semiconductor ASA

- R1-2206933	Discussion on disabling of HARQ feedback for IoT NTN	CMCC

- R1-2207080	On disabling HARQ feedback for IOT-NTN	Mavenir

- R1-2207144	Discussions on disabling of HARQ feedback for IoT NTN	Sharp

- R1-2207150	Disabling HARQ feedback in IoT-NTN	InterDigital, Inc.

- R1-2207257	Disabling HARQ Feedback for IoT-NTN	Qualcomm Incorporated

- R1-2207291	Disabling of HARQ feedback for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

- R1-2207295	Disabling of HARQ feedback for IoT NTN	Lenovo

- R1-2207355	HARQ Feedback Disabling for IoT NTN	Apple

- R1-2207570	On disabling HARQ feedback for IoT NTN	Ericsson

- R1-2207772	FLS#1 on disabling of HARQ feedback for IoT NTN	Moderator (Lenovo)

- R1-2207949	FLS#1 on disabling of HARQ feedback for IoT NTN	Moderator (Lenovo)

Submitted TDocs to AI 9.12.3

- R1-2205858	Discussion on improved GNSS operations for IoT NTN	Huawei, HiSilicon

- R1-2206011	Discussion on improved GNSS operations for IoT NTN	Spreadtrum Communications

- R1-2206023	Discussion on improved GNSS operation for IoT-NTN	ZTE

- R1-2206140	Improved GNSS operations for IoT NTN	MediaTek Inc.

- R1-2206313	Discussion on improved GNSS operations for IoT NTN	OPPO

- R1-2206389	Improved GNSS operations for IoT NTN	CATT

- R1-2206633	Discussion on improved GNSS operation for IoT NTN	Xiaomi

- R1-2206851	Improved GNSS operations for IoT NTN	Samsung

- R1-2206882	Improved GNSS operations for IoT NTN	Nordic Semiconductor ASA

- R1-2206934	Discussion on improved GNSS operations for IoT NTN	CMCC

- R1-2207151	On improved GNSS operation for IoT-NTN	InterDigital, Inc.

- R1-2207258	Improved GNSS Operations for IoT-NTN	Qualcomm Incorporated

- R1-2207259	On improved GNSS operation in IoT NTN	Ericsson Limited

- R1-2207292	Enhancements for long connections in NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

- R1-2207296	Improved GNSS operations for IoT NTN	Lenovo

- R1-2207356	On Improved GNSS Operations for IoT NTN	Apple

- R1-2207736	Feature lead summary#1 of AI 9.12.4 on improved GNSS operations	Moderator (MediaTek)

- R1-2207737	Feature lead summary#2 of AI 9.12.4 on improved GNSS operations	Moderator (MediaTek)

## 4.2	RAN2

RAN2#119-e, August 17th – 26th, 2022, e-meeting

Submitted TDocs to AI 8.6.2

- R2-2207060	Discussion on HARQ enhancement for IoT NTN			OPPO

- R2-2207075	Discussion on GNSS operation in connected mode		OPPO

- R2-2207300	On Disabling HARQ Feedback in IoT-NTN				MediaTek Inc.

- R2-2207354	HARQ process enhancements						Qualcomm Incorporated

- R2-2207484	Discussion on HARQ feedback disabling				Huawei, HiSilicon

- R2-2207647	Discussion on performance enhancement for IoT NTN		Transsion Holdings

- R2-2207710	Considerations on reducing UE GNSS operations in long connection time	Lenovo

- R2-2207841	Consideration on HARQ and GNSS enhancements		ZTE Corporation, Sanechips

- R2-2208187	Disabling HARQ feedback for IoT-NTN				Interdigital, Inc.

- R2-2208388	Discussion on the HARQ disabling in IoT NTN			CATT

- R2-2208448	Discussion on the performance enhancement for IoT-NTN	CMCC

- R2-2208565	Discussion on HARQ feedback disabling for IoT NTN		Nokia, Nokia Shanghai Bell

- R2-2208585	Discussion on disabling of HARQ feedback				Xiaomi

Submitted TDocs to AI 8.6.3

- R2-2207061	Discussion on mobility enhancement for IoT NTN			OPPO

- R2-2207275	Discussion on neighbour cell measurements in IoT NTN		Intel Corporation

- R2-2207299	On Mobility Enhancements in IoT-NTN				MediaTek Inc.

- R2-2207355	Connected mode mobility enhancements				Qualcomm Incorporated

- R2-2207500	Discussion on mobility enhancements for IoT NTN		Huawei, HiSilicon

- R2-2207648	Discussion on mobility enhancement for IoT NTN			Transsion Holdings

- R2-2207682	Discussion on triggering neighbour cell measurement before RLF	Spreadtrum Communications

- R2-2207711	Considerations on neighbour cell measurement for NB-IoT in NTN scenario		Lenovo

- R2-2207842	Consideration on mobility enhancements				ZTE Corporation, Sanechips

- R2-2207913	Discussion on mobility enhancements to IoT NTN			Xiaomi

- R2-2207931	Mobility Enhancement for IoT NTN 					Samsung R&D Institute UK

- R2-2207939	Neighbour cell measurements before RLF	Apple			IoT_NTN_enh

- R2-2208037	Changes to current mobility enhancement procedures for IoT-NTN	Nokia, Nokia Shanghai Bel

- R2-2208146	Discussion on Mobility Enhancements				TURKCELL

- R2-2208188	IoT-NTN mobility enhancements	Interdigital, Inc.

- R2-2208389	Discussion on the mobility enhancements in eMTC		CATT

- R2-2208449	Discussion on the mobility enhancement for IoT-NTN		CMCC

- R2-2208518	Use of Elevation Angle Threshold for IoT NTN Neighbour Cell Measurements 	SHARP Corporation

- R2-2208673	R18 IoT NTN Mobility enhancements				Ericsson

Submitted TDocs to AI 8.6.4

- R2-2207301	Enhancements to discontinuous coverage in IoT-NTN			MediaTek Inc.

- R2-2207356	RRC release procedure in discontinuous coverage			Qualcomm Incorporated

- R2-2207483	Discussion on the discontinuous coverage					Huawei, HiSilicon

- R2-2207649	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings

- R2-2207683	Discussion on power saving mechanism for supporting discontinuous coverage	Spreadtrum Communications

- R2-2207712	Considerations on mobility management and power saving for discontinuous coverage	Lenovo

- R2-2207778	Power Saving Enhancement for Discontinuous Coverage		Google Inc.

- R2-2207843	Consideration on discontinuous coverage enhancements		ZTE Corporation, Sanechips

- R2-2207914	Discussion on enhancements to discontinuous coverage		Xiaomi

- R2-2208023	Enhancements to discontinuous coverage					Samsung R&D Institute UK

- R2-2208115	Power Saving Enhancement for Discontinuous Coverage		Samsung R&D Institute UK

- R2-2208189	IoT-NTN discontinuous coverage enhancements				Interdigital, Inc.

- R2-2208450	Discussion on the discontinuous coverage for IoT-NTN			CMCC

- R2-2208566	Discussion on Discontinuous Coverage for IoT NTN			Nokia, Nokia Shanghai Bell

- R2-2208663	Discussion on Enhancements related to discontinuous coverage	Rakuten Mobile, Inc

- R2-2208672	R18 IoT NTN Enhancements to discontinuous coverage			Ericsson

## 4.3	RAN3

RAN3#117-e, August 15th – 24th, 2022, e-meeting

- R3-224385	Discussion on the support for discontinuous coverage	Nokia, Nokia Shanghai Bell

- R3-224386	Support for IOT NTN enhancements	Nokia, Nokia Shanghai Bell

- R3-224575	Support for Discontinuous Coverage in IoT NTN	Ericsson

- R3-224591	Discussion on power saving enhancements regarding discontinuous coverage	Huawei

- R3-224596	Mobility management enhancements for discontinuous coverage	Huawei

- R3-224643	Discussion on Discontinuous Coverage	CATT

- R3-224708	Initial consideration on mobility management enhancements for discontinuous coverage	ZTE

- R3-224709	Initial consideration on power saving enhancements for discontinuous coverage	ZTE

- R3-225025	CB: # IoTNTN_Coverage - Summary of email discussion	ZTE - moderator

# 5	Others

END