3GPP TSG RAN WG1 #100			R1-2000685

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.7.2

Source: 	LG Electronics

Title: 	Remaining issues on procedure of cross-slot scheduling power saving techniques

Document for:	Discussion and decision

- Introduction

In a previous meeting, an agreement for defining exceptional cases of minimum applicable K0 was made as follows;

Agreements: The adaptation on the minimum applicable value of K0 does not apply to C/CS/MCS-RNTI monitored in any common search space (of type 0/0A/1/2) associated with CORESET 0 if default TDRA table is applied. - FFS the case of CSS of type 3 - FFS other cases if default TDRA table is applied

In this contribution, we discuss remaining issues on the exceptional case.

- Discussion

The power saving gain from cross-slot scheduling comes from micro sleep or PDCCH processing relaxation in the duration guaranteed by minimum K0. Meanwhile, for some cases, those power saving schemes may be inefficient or data buffering should be performed. To be specific, if a UE monitors (SI, RA, P)-RNTI on Type(0,0A,1,2) CSS using default PDSCH TDRA table, the adaptation on the minimum applicable value of K0 cannot be applied because mostly all K0 values in default TDRA table is 0. For this reason, it was agreed that the adaptation on the minimum applicable value of K0 is not applied for following cases;

- SI-RNTI monitored in Type0/0A CSS, RA-RNTI & TC-RNTI monitored in Type1 CSS, and P-RNTI monitored in Type2 CSS (agreement in RAN1#96bis)

- C-/CS-/MCS-C-RNTI monitored in any CSS (of Type 0/0A/1/2) associated with CORESET0 if default TDRA table is applied (agreement in RAN1#99)

Table 1 shows applicable PDSCH time domain resource allocation for DCI formats 1_0 and 1_1, and it is defined in TS38.214 Section 5.1.2.1. In the table, the highlighted region should be discussed to finalize exceptional case of minimum applicable K0, the rest of the table was agreed as exceptional cases of minimum applicable K0. If a RRC-configured TDRA table (i.e., pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon, pdsch-TimeDomainAllocationList provided in pdsch-Config) is provided to a UE, a gNB can select minimum applicable K0 value in the table. However, if a UE assumes default TDRA table for PDSCH time domain resource allocation, the minimum applicable K0 value (which is larger than 0) cannot be applied because default TDRA table is applied. (For reference, it was agreed in RAN1#97 that an entry in the active DL (UL) TDRA table with K0 (K2) value smaller than the indicated minimum K0 (K2) is not expected by or not valid for the UE for the TDRA indication(s).)

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | Default A for normal CP |
|  |  | 2 | - | - | Default B |
|  |  | 3 | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | Default A |
|  |  | 2 | No | - | Default B |
|  |  | 3 | No | - | Default C |
|  |  | 1,2,3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| RA-RNTI, MsgB-RNTI, TC-RNTI | Type1 common | 1, 2, 3 | No | - | Default A |
|  |  | 1, 2, 3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | Default A |
|  |  | 2 | No | - | Default B |
|  |  | 3 | No | - | Default C |
|  |  | 1,2,3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | Default A |
|  |  | 1, 2, 3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | Default A |
|  |  | 1,2,3 | Yes | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | pdsch-TimeDomainAllocationList provided in pdsch-Config |

Table 1. Applicable PDSCH time domain resource allocation (Table 5.1.2.1.1-1 in TS38.214)

In conclusion, we propose that the adaptation on the minimum applicable value of K0 does not apply to C-/CS-/MCS-C-RNTI monitored in any search space set associated with any CORESET if default TDRA table is applied.

Proposal: The adaptation on the minimum applicable value of K0 does not apply to C-/CS-/MCS-C-RNTI monitored in any search space set associated with any CORESET if default TDRA table is applied.

- Conclusion

In this contribution, we discuss exceptional cases of minimum applicable K0, and following is proposed;

Proposal: The adaptation on the minimum applicable value of K0 does not apply to C-/CS-/MCS-C-RNTI monitored in any search space set associated with any CORESET if default TDRA table is applied.