3GPP TSG RAN WG1#100 Meeting	R1-2000875

e-Meeting, 24th February – 6th March, 2020

Source:	Sharp

Title:	Remaining issues on wide-band operation for NR-U

Agenda Item:	7.2.2.2.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues on wide-band operation for NR-U.

- Remaining issues for CORESET

  - CORESET offset

In RAN1#99 meeting [1], we made following agreements:

Agreement: For the frequency domain resource allocation that is provided with frequencyDomainResources in CORESET configuration, - Introduce a new RRC parameter rb-Offset (with the value range of 0,1,…,5) in ControlResoureSet IE. - If rb-Offset is not configured, rb-Offset is 0 - The bits of the 45-bit bitmap frequencyDomainResources have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the BWP with the starting PRB position as {the first PRB index in the BWP + rb-Offset} for a CORESET. - FFS: For multi-cluster CORESET configuration, rb-Offset also applies to the RB offset between the starting PRB index of the first 6 PRB group and the first PRB index in each RB set. Full 6 PRB groups are counted till the end of the RB set. The bits in frequencyDomainResources sequentially maps to the 6 RB groups in all RB sets in the BWP. - Note: Cluster above implies a group of resource blocks that are not contiguous in frequency

In the agreement, the new RRC parameter rb-Offset is introduced so that the CORESET can be suited for the RB sets in the BWP. However, the agreement is not well-reflected into the current specification of TS38.213. The agreement says that the rb-Offset is 0 if the rb-Offset is not provided, but the case when the rb-Offset is not provided can be interpreted as 6∙NBWPstart6 for the current specification. In other words, the case when the rb-Offset is not provided, has a same meaning that the UE behaviour should follow the Rel-15 instead rb-Offset is 0 as in the agreement, since the RRC parameter rb-Offset does not exist in the Rel-15. Anyhow, the agreement is needed to modify. From our point of view, if the rb-Offset is not provided, it might be a straightforward solution that a UE should follow Rel-15 behaviour, otherwise the rb-Offset plays a role to have an offset from the starting common RB position defined in Rel-15 so that it exactly reflects what the agreement intends to. Therefore, we propose that the UE should follow Rel-15 UE behaviour if rb-Offset is not provided as given in Text proposal #1.

Proposal 1: The UE should follow Rel-15 UE behaviour if rb-Offset is not provided as given in Text proposal #1.

Text proposal #1 --------- beginning of text proposal for TS 38.213 ## 10.1 UE procedure for determining physical downlink control channel assignment <omitted> For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. The bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of PRBs with starting common RB position where the first common RB of the first group of 6 PRBs has common RB index 6∙NBWPstart6-rb_Offset, where rb_Offset is 0 if not provided. NBWPstart+rb_Offset <omitted> --------- end of text proposal

- Conclusion

In this contribution, we propose

Proposal 1: The UE should follow Rel-15 UE behaviour if rb-Offset is not provided as given in Text proposal #1.

- References

  - 3GPP RAN1#99 Chairman’s note, November 2019.