3GPP TSG RAN WG1 Meeting #100-e	R1-2001114

February 24-March 6, 2020

Agenda Item:	6.2.2.7

Source:	Huawei, HiSilicon

Title:	Further considerations on RRC parameter lists for NB-IoT

Document for:	Discussion and Decision

- Introduction

In NB-IoT Rel-16, the agreements are summarized in [1] and related RRC parameters list is endorsed in [2]. In this contribution, we provide our further considerations on RRC parameter lists for NB-IoT.

- Transmission in preconfigured resource

In RAN1#99, the following agreement about CFS PUR was made. Two UE-specific cyclic shifts for DMRS were defined.

Agreement The following previous working assumption is confirmed with modification: For NB-IoT allocation with 12 tones, for PUR with R>= 64ms: - Allow for UE-specific cyclic shift for DMRS: - For 12 tone allocation, following 2 cyclic shifts are supported in TS 36.211 Table 5.2.2.1.1-2. - The cyclic shifts are {0, 6}*(2pi/12) - For NPUSCH scrambling, the current c_init is used by using the configured PUR_RNTI.

The above agreements have been captured in TS 36.211. It is assumed that higher layers will provide the value of ncs. And the value of ncs is 0 or 6 according to above agreements.

Extracted from 10.1.4.1.2 in TS 36.213 The cyclic shift for and is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 10.1.4.1.2-3. For , if pur-NPUSCH-cyclic-shift is configured it provides the value of ncs and the cyclic shift α in a slot is given as α=2πncs/12, otherwise .

But in the endorsed RRC parameters list [2], the value range of higher layer parameter for UE-specific cyclic shift is {0, 6}*(2*pi/12). It is not in consistent with current description of TS 36.211.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect (see note) | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | TDD/FDD | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NB_IOTenh3-Core | PUR | 36.213 36.211 |  |  |  | pur-NPUSCH-cyclic-shift | New | PUR NPUSCH cyclic shift | PUR NPUSCH cyclic shift for the DMRS | FFS {0, 6}*(2pi/12) |  |  | UE-specific | FDD | 36.331 |

Proposal 1: For CFS PUR, the value range of higher layer parameter pur-NPUSCH-cyclic-shift is {0, 6}.

- Multiple TB scheduling

According to the endorsed RRC parameters list [2] for NB-IoT, multiple DL or UL TB scheduling for unicast is enabled by the same higher layer parameter.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect (see note) | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | TDD/FDD | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NB_IOTenh3-Core | Multi-TB scheduling | 36.211, 36.212, 36.213 |  |  |  | multi-TB-Unicast-config | New | Multi-TB scheduling enabled for unicast | If enabled, one DCI can be used to schedule multiple TBs for unicast. | enabled | disabled |  | UE-specific | FDD | 36.331 |

But in eMTC, multiple DL or UL TB scheduling for unicast is enabled by separate higher layer parameters.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect (see note) | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | TDD/FDD | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE_eMTC5-Core | Multi-TB scheduling | 36.212, 36.213 |  |  |  | multi-TB-DL-config | New | Multi-TB scheduling enabled for unicast in downlink | If enabled, a single DCI can schedule multiple (up to 8 in CE mode A and up to 4 in CE mode B) transport blocks for DL unicast in CE mode A/B in RRC_CONNECTED. | Enabled |  |  | UE specific |  | 36.331 |
| LTE_eMTC5-Core | Multi-TB scheduling | 36.212, 36.213 |  |  |  | multi-TB-UL-config | New | Multi-TB scheduling enabled for unicast in uplink | If enabled, a single DCI can schedule multiple (up to 8 in CE mode A and up to 4 in CE mode B) transport blocks for UL unicast in CE mode A/B in RRC_CONNECTED. | Enabled |  |  | UE specific |  | 36.331 |

And in RAN2#106, the following agreement for NB-IoT and eMTC was made.

UE capability for multiple TB is indicated separately for uplink and downlink.

Since the UE capabilities for multiple DL or UL TB scheduling are separate, the configuration of multi-TB for downlink and uplink should also be separate. Otherwise, the eNB would be unable to configure UEs that support multi-TB only in downlink or uplink to use this feature.

Proposal 2: Multiple TB scheduling for unicast is configured separately for DL and UL.

Proposal 3: Send LS to RAN2 to take above agreements into consideration.

- Conclusion

In this contribution, our further considerations on RRC parameter lists for NB-IoT are provided. The following proposals are made.

Proposal 1: For CFS PUR, the value range of higher layer parameter pur-NPUSCH-cyclic-shift is {0, 6}.

Proposal 2: Multiple TB scheduling for unicast is configured separately for DL and UL.

Proposal 3: Send LS to RAN2 to take above agreements into consideration.

- References

R1-1913595, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT”, Futurewei, RAN1#99, Reno, USA, 18th – 22nd November 2019.

R1-1913673, ” Updated consolidated parameter list for Rel-16 LTE”, Qualcomm, RAN1#99, Reno, USA, 18th – 22nd November 2019.