3GPP TSG RAN WG1 #100 Meeting						R1-2001189

e-Meeting, February 24th – March 6th, 2020

Source:	vivo

Title:	Summary#2 of UE adaptation to maximum number of MIMO layers

Agenda Item:	7.2.7.3

Document for:	Discussion and Decision

- Introduction

For RAN1#100e meeting, it is planned as follows,

- Preparation phase (2/17-2/21):

  - By the end of 2/17: Feature leads to summarize critical issues for each sub-agenda (FL contributions are to provide a list of issues/proposals and an indication of priority)

- 2/18-2/21: Email discussion to finalize the set of critical issues for each sub-agenda

  - feature leads to request a revised Tdoc# for their summary once all critical issues have been identified and the group agrees the topics to be discussed over email

The contribution is a summary of the email discussion outcome, and it is proposed as follows,

Proposal: No high priority issues need to be discussed in RAN1#100 e-meeting.

- Summary of the issues proposed in RAN1#100e

The following is a summary of the issues proposed with high/low priority flag.

| Email discussion thread | category | # | Issues | Ref. | Priority |
| --- | --- | --- | --- | --- | --- |
| [100e-Prep-NR- NR_UE_pow_sav-MIMO-A] | Spec impact | A | Shorter MIMO layer adaption delay for DCI/timer based BWP switching | [0211][0339] | Low |
| [100e-Prep-NR- NR_UE_pow_sav-MIMO-B] | Clarifications | B1 | Clarification on initial BWP, which can also be configured by network of its maximum number of DL MIMO layers | [0339] | Low |
|  |  | B2 | UE shall follow the configuration given for SRS-ResourceSet with usage set as 'antennaSwitching' regardless of the maxMIMO-Layers parameter of the DL BWP. No spec impact is expected. | [1050] | Low |

  - [100e-Prep-NR- NR_UE_pow_sav-MIMO-A]

When only number of maximum MIMO layer adaptation or antenna port is changed between the BWPs before and after MIMO layer adaptation, e.g., the BWP center frequency, bandwidth and SCS will keep unchanged during MIMO layer adaptation, further study the switching delay and interruption requirements is needed. Two companies proposed the followings,

Proposal 1: The MIMO layer adaption delay for DCI/timer based BWP switching in case 2 shall be defined as below: [R1-2000211]

|  | NR Slot length (ms) | BWP switch delay TMIMOswitchDelay (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 6 |

Proposal 2: The application delay for the case only the number of max#M.L. is changed in the BWP before and after max#M.L. adaptation can be considered to be shorter than normal BWP switching. [R1-2000339]

  - [100e-Prep-NR- NR_UE_pow_sav-MIMO-B]

The following proposals are for clarification of supporting maximum number of MIMO layer adaptations. These issues may not have specification impacts.

B1: MIMO layer adaptation and initial BWP [R1-2000339]

- Proposal 1: The network can also configure the max number of MIMO layer for initial BWP.

B2: Antenna Switching and MIMO layer adaptation [R1-2001050]

- Observation: SRS resource set(s) for DL CSI acquisition can be configured as normally, independently of BWP specific maximum number of DL MIMO layers parameter.

- Proposal: UE shall follow the configuration given for SRS-ResourceSet with usage set as 'antennaSwitching' regardless of the maxMIMO-Layers parameter of the BWP.

Reference

- R1-2000211  Corrections on UE adaptation to maximum MIMO layers	Huawei, HiSilicon

- R1-2000339  Maintenance of UE adaptation to maximum number of MIMO layers	vivo

- R1-2000886  Maintenance issues for MIMO layer signaling per BWP	Ericsson

- R1-2001050  On open issues for per-BWP DL MIMO layers	Nokia, Nokia Shanghai Bell

Annex 1 : Previous agreements

Agreements:

- Support per-DL-BWP configuration of maximum number of DL MIMO layers

  - Signalling details up to RAN2

Conclusion:

- No additional spec impact to support adaptation of UL Max #MIMO layers for UL NCB and CB MIMO in Rel-16 power saving WI

Agreements:

- The configured per-BWP DL max MIMO layer value is expected to be less than or equal to the per-cell configured DL Max MIMO layer value (if configured).

Send an LS to RAN2 to inform the above two decisions (& the DL part decision from last meeting) – Xiaodong (vivo), R1-1911479, which is approved (by also CC-ing RAN4) with final LS in R1-1911528

Agreements:

- DL-SCH TBSLBRM is invariant across all the DL BWPs, when DL max MIMO layer adaptation is per BWP

  - FFS details

Agreements:

- No spec change for TS38.212 is needed for determining DL-SCH TBSLBRM when downlink max MIMO layer adaptation is configured.

  - Note: DL-SCH TBSLBRM is not dependent on the per-BWP configured maximum number of DL MIMO layers value.

- When maximum number of DL MIMO layers per BWP is configured for all configured DL BWPs for a serving cell,

  - At least one BWP is configured with per-BWP configured maximum number of DL MIMO layers value equal to the per-cell configured maximum number of DL MIMO layers value (if configured).

Agreements:

The capability of Rel-16 per-BWP configuration of maximum number of DL MIMO layers is indicated only when the Rel-15 maxLayersMIMO-Indication capability is indicated.

Annex 2: WID

For UE adaptation to the maximum number MIMO layers, the following is agreed in updated power saving WID in RAN#84,

Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2] - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation