3GPP TSG RAN WG1#100-e			R1-2001348

e-Meeting, 24th February – 6th March, 2020

Agenda item:	7.2.6.2

Source: 	OPPO

Title: 	Text Proposal 1 for [100e-NR-eMIMO-multiTRP-02]

Document for:	Discussion and Decision

# Text Proposal

The following TP is proposed for TS38.213-g00.

-------- start of the TP------- #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = JointFeedback where - a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and - a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for the set S0 and the set S1 of serving cells separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1 to obtain a total number of HARQ-ACK information bits.S0∪S1NcellsDL=NcellsDL,0+ NcellsDL,1 For the set of slot timing values , the UE determines a set of occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases. < Unchanged parts are omitted > Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell; If for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, lower indexes correspond to lower RRC indexes of corresponding cell in the set S0 or the set S1. Set - HARQ-ACK information bit index Set to the number of serving cells configured by higher layers for the UE or to NcellsDL,0 or to NcellsDL,1 while Set – index of occasion for candidate PDSCH reception or SPS PDSCH release while < Unchanged parts are omitted > -------- end of the TP-------