3GPP TSG RAN WG1 Meeting #100bis 	R1-2001832

e-Meeting, April 20th – 30th, 2020

Agenda Item: 7.2.11.9

Source: MediaTek Inc.

Title: 	Views on Rel-16 UE features for NR mobility enhancements

Document for: Discussion

- Introduction

This paper provides our views on Rel-16 UE features for NR mobility enhancements. Detailed proposals are described in the following sections.

- Views on UE features for NR mobility enhancements

Agreement:

- In RAN1 understanding, if UE indicates that it supports DAPS HO for a specific feature set, it implies (without additional capability signaling) that UE supports simultaneous reception of DL signals/channels in overlapping OFDM symbols for DAPS HO.

- FFS: Whether, a UE that indicates DAPS-HO capability is capable of simultaneously receiving PDCCH/PDSCH from source and target cells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time.

For the current RAN1 UE feature table of mobility enhancement, FL suggested that feature 21-2 could be modified to state the support of simultaneous DL reception when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time, but current feature 21-2 in the UE feature table is for “UL power sharing for DAPS HO”, while the RAN1 #99 FFS shown below is discussing DL reception. Hence, it’s better to state the FFS in FG 21-1a, rather than FG 21-2.

Since “simultaneously receiving PDCCH/PDSCH from source and target cells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time” only happens in intra-band case, it should be a per band signalling.

Proposal 1: Add additional capability FG 21-1a to indicate per band support of simultaneously DL reception with both source and target PCells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time to address the FFS in RAN1 #99.

Besides, according to RAN2 guidance, the basic feature 21-1 is indicated per band for intra-frequency DAPS-HO and per band combination for inter-frequency DAPS-HO. Therefore, FGs 21-1/21-2/21-2a should also indicated per band for intra-frequency DAPS-HO and per band combination for inter-frequency DAPS-HO.

Proposal 2: For FGs 21-1/21-2/21-2a, they should be indicated per band for intra-frequency DAPS-HO and per band combination for inter-frequency DAPS-HO.

Agreement:

- If a UE is configured with DAPS HO operation, the UE performs transmission power control based on Section 7.6.2 of 38.213 replacing the MCG with target MCG and SCG with source MCG.

  - Note: This assumes the latest draft CR in R1-1913214 as the baseline

For FG 21-2: UE power sharing for DAPS HO, it is agreed in RAN1 #99 agreement to reuse NR-DC UL power sharing mechanism as shown below. Hence, ALT 2 in the UE feature table should be used.

However, due to the reason that the signallng structure of NR-DC UL power sharing is still under discussion (Ex. the function of look-ahead), ALT 2 also needs to be revised after the signallng structure of NR-DC UL power sharing is finished.

Proposal 3: Choose ALT 2 for FG 21-2 and revise it later after the signallng structure of NR-DC UL power sharing is finished.

# Proposed changes on current RAN1 UE features table

# Mobility Enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21. Mobility Enhancement | 21-1 | DAPS HO | For the Indicated support of intra-frequency DAPS-HO for a given [band] and indicated support of inter-frequency DAPS-HO for a given [band combination]. 1) Indicates support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Indicates support of PDCCH blind decoding capability in the first MCG and second MCG. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO | [Per BC] for inter-frequency case, Per Band for intra-frequency case | No | N/A | N/A |  | [Optional with capability signalling] |
|  | 21-1a | Simultaneous DL reception with both source and target PCells druing resource collision | Indicate support of simultaneous DL reception with both source and target PCells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The UE does not expect simultaneous DL reception with both source and target PCells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time | Per Band | No | N/A | N/A |  | Optional with capability signalling |
|  | 21-2 | UE power sharing for DAPS HO | Indicates support of UL power sharing mode during DAPS-HO operation. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The UE is only able to to drop the transmission to the source. | [Per BC] for inter-frequency case, Per Band for intra-frequency case | No | N/A | N/A |  | Optional with capability signalling {values: Semi-static-mode1, Semi-static-mode2, Dynamic} |
|  | 21-2a | UL transmission cancellation | Indicates support of cancelling UL transmission to the source cell |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The UE does not expect UL transmissions to source cell and target cell overlap. The indication is signalled per pair of bands per band combination. | Per BC for inter-frequency case, Per Band for intra-frequency case | No | Yes | N/A |  | Optional with capability signalling |
|  | 21-3 | PDCCH blind detection for MCG1 and MCG2 | Indicates PDCCH blind decoding capability supported in the first MCG (intended for target cell in HO) and second MCG (intended for source cell in HO) when in DAPS-HO. Value range for both cells should be {1, …, 15}. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network may not able to configure UE with DAPS HO, as parameters required for PDCCH monitoring is essential for defining UE behaviour | Per UE | No | Yes | N/A | This capability (although optional) parameter value must be conveyed to the network in order to correctly define UE behaviour | Optional with capability signalling For first MCG {values: 1, 3, … 15} For second MCG {values: 1, 3, … 15} |
|  | 21-4 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |