3GPP TSG RAN WG1 #100bis	R1-2001877

e-Meeting, April 20th –30th, 2020

Agenda Item:	7.2.4.2.2

Source: 	Fujitsu

Title:	Remaining details on mode 2 resource allocation for NR V2X

Document for:	Discussion/Decision

Introduction

In the previous RAN1 meetings, some agreements have been achieved on mode 2 resource allocation for NR V2X. In this contribution, we give our views on several remaining details related to mode 2, i.e. details of step 2 (Resource selection from the identified candidate resources), the number of resources indicated in each selected resource’s corresponding 1st SCI, the UE behavior on the resource re-selection procedure triggered by re-evaluation and pre-emption, the UE behavior on the released un-used resource, and some other aspects of pre-emption, etc.

Discussion

Details of step 2

According to [1] and [2], in LTE V2X, the maximum number of HARQ (re-)transmissions is 2, and only if there are available resources left in the identified candidate resources set which meet the conditions below, the retransmission can be performed; otherwise, it should be dropped.

TS 36.213, 14.1.1.7

When a set of subframes  for  have been selected for a set of transmission opportunities of PSSCH, a set of subframes  for  another set of transmission opportunities of PSSCH shall meet the conditions  and  where  and  is the maximum number of transmission opportunities of PSSCH in a selected subframe set. Here,  is the resource reservation interval provided by higher layers.

In the RAN1#98bis meeting, the following agreements have been achieved w.r.t. the maximum number of HARQ (re-)transmissions for NR V2X.

Agreements:

- Maximum number of HARQ (re-)transmissions is (pre-)configured per priority per CBR range per

transmission resource pool

  - The priority is the one signaled in SCI

  - This includes both blind and feedback-based HARQ (re)-transmission

- The value range is any value from 1 to 32

  - If the HARQ (re)transmissions for a TB can have a mixed blind and feedback-based approached (FFS whether or not to support this case), the counter applies to the combined total

In the RAN1#99 meeting, the following agreement has been achieved. It points out that, similar to LTE, the randomized resource selection is still the baseline of step 2 in NR V2X. However, the conditions which should be met may need to be re-defined in some cases, since the number of HARQ (re-)transmissions can be more than 2 in NR V2X.

Agreements:

- In Step 2, randomized resource selection from the identified candidate resources in the selection window is supported

  - FFS if CSI can be used for resources selection

In the RAN1#100e meeting, the following agreements have been reached, which have limited the timing of resource selection to avoid the PSFCH transmitting collision and freed up the PSFCH processing time.

Agreements:

- In Step 2, a UE ensures a minimum time gap Z = a + b between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected

  - ‘a’ is a time gap between the end of the last symbol of the PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception determined by

resource pool configuration and higher layer parameters of MinTimeGapPSFCH and

periodPSFCHresource

  - ‘b’ is a time required for PSFCH reception and processing plus sidelink retransmission preparation including multiplexing of necessary physical channels and any TX-RX/RX-TX switching

time and is determined by UE implementation

In NR V2X, when the determined number of HARQ (re-)transmissions (Mtarget) is not larger than 2, the LTE V2X mechanism in [1] and [2] can be reused for step 2, in which only the range in the conditions needs to be expanded to [-31, 31] slots since the time position of the second resource is within 32 slots of the first resource..

Observation 1: In NR V2X, when the determined number of the HARQ (re-)transmissions is not larger than 2, the LTE V2X step 2 mechanism can be reused.

Proposal 1: In the case where the determined number of the HARQ (re-)transmissions is not larger than 2, the LTE V2X step 2 mechanism should be reused in NR V2X,  and only the limited range in the condition should be expanded to [-31, 31] slots.

Otherwise, when the determined number of the HARQ (re-)transmissions (Mtarget) is larger than 2, the resources for HARQ (re-)transmissions should be randomly selected one by one from the identified candidate resources.

Proposal 2: In the case where the determined number of HARQ (re-)transmissions is larger than 2, the resources for the HARQ (re-)transmissions should be randomly selected one by one from the identified candidate resources in NR V2X.

In LTE V2X, the condition which needs to be met is  and, which means the second selected resource cannot be in the same sub-frame with the first resource, and the interval b/w them must be within a defined range. Based on this condition, two resources can indicate each other by the corresponding SCIs, so that the reliability can be enhanced even more.

Similarly, in NR V2X, whether to form a chain during the resource selection procedure is a remaining and critical issue, and no agreement has been reached in the last meeting. In our point of view, since the maximum number of the HARQ (re-)transmissions of a TB is 32, the transmitter UE may send packets by non-reserved resources many times if there is no such a chain. As a result, the probability of the resource collision may obviously increase, especially in the case that most of the other UEs around the transmitter UE are sending non-periodic services, and the PRR performance may also decrease in this case. On this basis analysis, we support a chain should be formed during the step 2 in resource selection procedure, i.e. the selected resource for a retransmission of a TB should be reserved at least once by a SCI corresponding to a previous (re)transmission.

Proposal 3: A chain should be formed during the step 2 in resource selection procedure, i.e. the selected resource for a retransmission of a TB should be reserved at least once by an SCI corresponding to a previous (re)transmission.

Based on above proposal, the following scheme can be deployed for resource selection: for an nth resource selection with the purpose of selecting resource Rn, all already selected resources {R1, R2 ... Rn-1}, and all the candidate resources which are in the same slot with {R1, R2 ... Rn-1}, should be precluded from the identified candidate resource set. For HARQ-ACK enabled (re)transmission; with regard to the agreements in RAN1 100e, all the candidate resources which are within the minimum time gap ‘Z’ with at least one resource from {R1, R2 ... Rn-1}, can also be precluded from the identified candidate resource set, to avoid PSFCH transmitting collision (the associated PSFCHs appear in the same slot, case 3). Here, when starting the nth resource selection, the remaining set of available resources after the resource exclusion procedure are defined as Sn-1. This set should be iterated when the resource selection procedure proceeds.

Proposal 4: When starting the nth resource selection to select resource Rn, all already selected resources {R1, R2 ... Rn-1}, and all the candidate resources which are in the same slot with {R1, R2 ... Rn-1}, should be precluded from the identified candidate resource set.

Furthermore, the selected resource Rn should meet the new conditions in NR V2X: it should be within [-32, 32] slots of at least one resource from {R1, R2 ... Rn-1}, to form a chain and ensure that each selected resource except for the first one in time can be reserved by at least one SCI corresponding to the previous resources. For the nth resource selection with the purpose of selecting resource Rn, all the candidate resources meeting the above condition are defined as S’n-1.

Proposal 5: The selected resource Rn should be within [-32, 32] slots of at least one resource from {R1, R2 ... Rn-1}.

Finally, the step 2 of resource selection procedure stops and can be regarded as completed when the number of the already selected resources has reached Mtarget or Sn-1∩Sn-1,=∅. M is defined as the number of selected resources after the resource selection procedure is completed, M≤Mtarget. After the resource selection procedure has been completed, we consider the first resource in time domain as the new transmission opportunities and the rest set of transmission opportunities as the retransmission opportunities.

Proposal 6: The step 2 of the resource selection procedure stops and can be regarded as completed when the number of the already selected resources has reached Mtarget or Sn-1∩Sn-1,=∅.

An example for the step 2 procedure described above is shown in the following figures. In the following figures, we have Mtarget=6. In figures 1(a) and 1(b), for the selection of Rn (n= 1, 2,…6), all the candidate resources which are in the same slot with {R1, R2 ... Rn-1} are precluded; in figures 1(c) and 1(d), all the candidate resources whose associated PSFCH resources are in the same slot with the associated PSFCH resources of with {R1, R2 ... Rn-1} are additionally precluded. In all of the following figures, W=32, then the selected resource Rn should be within [-32, 32] slots of at least one resource from {R1, R2 ... Rn-1}. In figures 1(a) and 1(c), the step 2 of resource selection procedure stops due to the number of already selected resources has reached Mtarget, where M=Mtarget=6; in figures 1(b) and 1(d), the step 2 of resource selection procedure stops due to Sn-1∩Sn-1,=∅, where M=4, Mtarget=6, M<Mtarget.

Figure 1 Example for the step 2 of resource selection procedure

The number of resources indicated in an SCI

According to the summary of RAN1# 98bis meeting, the following agreements have been achieved w.r.t. the number of maximum resources for a same TB indicated by an SCI (Note: all SCIs mentioned in this section are 1st SCIs).

Agreements:

- When reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is disabled, NMAX is 3

  - SCI signaling is designed to allow to indicate 1 or 2 or 3 resources at least of the same number of

sub-channels with full flexibility in time and frequency position in a window W of a resource pool

    - FFS: if full flexibility is limited in some cases

  - Value 2 or 3 is (pre-)configured per resource pool

  - FFS size of window W

In the RAN1# 99bis meeting, from the following agreement, we know that in an SCI, the number of resources should be less than Nmax, and can be flexibly indicated.

Agreements:

- For mode 1 and mode 2, for the time-frequency resource indication in the SCI:

  - NMAX = 2

    - Frequency

      - m=1S(S+1-m)=SS+12 code-points, indicating starting sub-channel of the

second resource and number of sub-channels of both resources

      - log2(SS+12) bit

    - Time

      - 1 code-point indicates no reserved resource

      - 31 code-points indicate different time position of the second resource within 32 slots

      - 5 bit

  - NMAX = 3

    - Frequency

      - Option 2-f-a: joint coding

        - m=1SS+1-m2=SS+12S+16 code-points indicating starting

sub-channel of the second resource, starting sub-channel of the third

resource, and the number of sub-channels of all resources

        - log2(SS+12S+16) bit

    - Time

      - Option 2-t-a: joint coding

        - 1 code-point indicates no reserved resource

        - 31 code-points indicate different time position of the second resource

within 32 slots, when no third resource is reserved

        - 30 + 29 +…+ 1 = 465 code-points indicate different time position of

two resources within 32 slots

        - 9 bit

Based on the mechanism in section 2.1, after all resources have been selected and sorted in a chronological order, another remaining issue is on how to determine the number of resources indicated in each selected resource’s corresponding SCI. In our point of view, each of selected resource except for the first one in time should be reserved by at least one SCI corresponding to the previous resources. Like LTE, each resource which has been sorted in a chronological order can be defined as a transmission opportunity. Thus, in the case where Nmax=2, for an mth (1≤m≤M-1) transmission opportunity in time, the corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity, especially, the last (Mth)  opportunity only indicates the resource for itself; in the case where Nmax=3, for an transmission mth (1≤m≤M-2) opportunity in time, if the gap b/w the opportunity m and m+2 is larger than 31 (illustrated in figure 2b), the corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity; else, the corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity and (m+2)th opportunity (illustrated in figure 2a). Especially, the last (Mth) opportunity only indicates the resource for itself, the (M-1)th opportunity indicates the resources for itself and the Mth opportunity.

Figure 2 Illustration of Nmax=3, W=32, the number of resources indicated in the corresponding SCI of the mth opportunity

Proposal 7:

- In the case where Nmax=2,

  - For an mth (1≤m≤M-1) transmission opportunity in time,

    - The corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity;

  - The last (Mth) opportunity only indicates the resource for itself.

Proposal 8:

- In the case where Nmax=3,

  - For an mth (1≤m≤M-2) transmission opportunity in time,

    - If the gap b/w the opportunity m and m+2 is larger than 31, the corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity;

    - Else, the corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity and (m+2)th opportunity;

  - The last (Mth) opportunity only indicates the resource for itself, the (M-1)th opportunity indicates the resources for itself and the Mth opportunity.

Resource re-selection triggered by re-evaluation and pre-emption

In the last meeting, the following agreements have been achieved on re-evaluation and pre-emption, and some remaining open issues are treated as FFS.

Agreements:

- For re-evaluation of a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’, where k ≥ m,

  - Step 1 of the resource (re-)selection procedure is performed at least at the moment ‘m-T3’, and if the pre-selected resource is not in the identified candidate resource set, Step 2 is triggered for reselection of the resource

    - Re-evaluations before the moment ‘m-T3’ or after ‘m-T3’ but before ‘m’ are not precluded and are up to UE implementation

      - FFS whether to mandate a UE to perform Step 1 checking every slot before ‘m-T3’

    - FFS whether evaluation of Step 2 has to ensure any introduced timing restrictions between pre-selected and re-selected resources when re-evaluation is triggered, and whether it is allowed to change the pre-selected but not reserved resources which are still in the candidate resource set in order to ensure the timing restrictions

- FFS whether for the case of enabled periodic reservation, already reserved resources in upcoming periods can be re-evaluated

Agreements:

- For pre-emption, both full and partial frequency domain overlap in the same slot are considered as the overlapping condition to trigger resource reselection, wherein the whole resource is reselected even if the partial overlap happened

- (Re-)selection procedure for an already reserved but pre-empted resource to be used for transmission in a slot ‘m’ is not required to be triggered at moment > ‘m – T3’

  - T3 here is identical to T3 introduced for the re-evaluation

- FFS whether re-selection of the already-reserved, but pre-empted resource applies only to the resource transmitted in slot ‘m’ or to other already-reserved and pre-empted resource(s) signaled in the SCI in slot ’m’ as well

Until now, the resource re-selection should be triggered by re-evaluation/pre-emption is agreed but some details of the resource re-selection procedure triggered by these two operations need to be clarified.

The re-evaluation in NR V2X means, when a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’, where k ≥ m, is not in the identified candidate resource set, the resource re-selection procedure should be triggered and the re-evaluated resource which is not in the identified candidate resource set needs to be re-selected. Moreover, the resource which is in the same slot and indicated by the corresponding PSCCH in slot ‘m’ should be regarded as the 1st selected resource for the re-selection procedure, since it has been reserved and signaled by a previous SCI. On this basis analysis, the remaining steps and timing restrictions of the resource re-selection are same to the initial resource selection described in the previous section. Compared to the initial selected resource, whether other pre-selected but not reserved resources which are still in the candidate resource set is changed depends on UE implementation and does not need to be specified.

Proposal 9: For resource re-selection of a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’ triggered by re-evaluation,

- The resource which is in the same slot and indicated by the corresponding PSCCH in slot ‘m’ should be regarded as the 1st selected resource;

- Whether other pre-selected but not reserved resources which are still in the candidate resource set is changed depends on UE implementation.

An example is shown by the figure below. In this figure, the re-evaluation is done in the time of T3 before the slot of resource #m k+1, resource #m k+2 which is signaled by the SCI corresponding to #m k+1 is not in the identified candidate resource set, then this resource should be reselected; resource #m k+1 should be regarded as the 1st selected resource and whether the pre-selected resource of #m k+3 and #m k+4 is changed should be based on the implementation when all the timing restrictions are met; whether the pre-selected resource of #m k+5 which is also not in the identified candidate resource set is changed should also be based on the implementation, since it can be re-evaluated again in the time of T3 before the slot of resource #m k+3 or #m k+4.

Figure 3 Illustration of resource re-selection triggered by re-evaluation

The pre-emption in NR V2X means, for a UE, a resource which has been signaled by a previous SCI, is occupied by another UE which has a higher priority packet to be transmitted, and the pre-empted resource needs to be re-selected. The steps and timing restrictions of the resource re-selection are same to the initial resource selection described in the previous section. A special case is, if there is another resource is signaled by a same SCI with the pre-empted resource in case of Nmax=3, this “another resource” should be regarded as the 1st selected resource for the re-selection procedure, since it has been reserved and signaled.

Proposal 10: For resource re-selection of a pre-empted resource contained in a slot ‘m’,

- If there is another resource is signaled by a same SCI with the pre-empted resource in case of Nmax=3, this “another resource” should be regarded as the 1st selected resource for the resource re-selection procedure;

- Else, the 1st resource is randomly selected in the selection window of the resource re-selection triggered by the pre-emption.

An example is shown by the figure below. In this figure, resource #m k+2 is pre-empted, then this resource should be reselected; resource #m k+3 should be regarded as the 1st selected resource during performing resource re-selection since it has also been reserved and signaled by the SCI corresponding to resource #m k+1.

Figure 4 Illustration of resource re-selection triggered by pre-emption

Furthermore, since sensing procedure is always performed in physical layer, when physical layer found one or more pre-selected resources is not in the identified resource set any more, these resources should be re-selected. Then, physical layer needs to report sensing result/ (re)selection indication to MAC layer, and MAC layer decide whether to proceed the resource re-selection procedure based on the contents of this report.

Proposal 11: The resource re-selection procedure should be triggered by MAC layer, on the basis of the reporting contents from physical layer.

Release of unused resource

According to the summary of RAN1# 97 meeting, the reservation of resources for feedback-based HARQ retransmission(s) has been accepted by RAN1 as follows.

Agreements:

- NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB

  - FFS impact on subsequent sensing and resource selection procedures

  - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported

    - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

    - FFS the behavior of the receiver UE(s) of this TB and other UEs

In the following subsection, we give our opinions on the following opening aspect:

- FFS the behavior of the receiver UE(s) of this TB and other UEs

It is possible the other UEs notify the using or un-using of the reserved resource by monitoring the ACK/NACK of the sending UE. If the feedback to the sending UE is ACK, the other UEs can start to use the released resource. If the feedback to the sending UE is NACK, the other UEs can avoid to select the reserved resource. It means the UE needs to decoding both the SCI and SFCI (sidelink feedback control information) for its resource selection. Since no additional signaling is defined for the purpose of release of unused resources by the transmitting UE, only the HARQ is used for the receiver UE(s) releasing the unused resource. The behavior of the receiver UE(s) is as follows: it will not use the reserved resource when the receiver UE receives the ACK. it will use the reserved resource when the receiver UE receives the NACK.

In order to let other UE(s) use the released unused resource, the other UEs needs to decode the HARQ which is not related to its own link. Then, the other UEs could check the using status of the resource reserved not by its own and has the ability to use the released resource.

The behavior of the other UE(s) is as follows: it will use the unused resource when it decodes the ACK related to the link in which the resource is not reserved by itself. It will not use the unused resource when it decodes the NACK related to the link in which the resource is not re-served by itself.

Proposal 12: The resources that have previously been reserved for the UE(s) can be used or released by using HARQ feedback. The released resource could be used by other UEs after the other UEs monitoring the HARQ of the reserved UE(s).

Proposal 13: The other UEs need to monitor the HARQ ACK/NACK feedback when perform their own resource selection.

- If the feedback to the sending UE is ACK, the other UEs can start to use the released resource.

- If the feedback to the sending UE is NACK, the other UEs can avoid selecting the reserved resource.

Other aspects of pre-emption related design

In RAN1 #98bis meeting, the following agreements were achieved:

Agreements:

- Support a resource pre-emption mechanism for Mode-2

  - A UE triggers reselection of already signaled resource(s) as a resource reservation in case of overlap with resource(s) of a higher priority reservation from a different UE and, SL-RSRP measurement associated with the resource reserved by that different UE is larger than an associated SL-RSRP threshold

    - Only the overlapped resource(s) is/are reselected

    - FFS

      - the timeline for reselection

      - other details

    - FFS whether or not to support other potential UE behaviour (e.g, power boosting/reduction)

  - This mechanism can be enabled or disabled, per resource pool

    - FFS details

Power boosting or reduction

It is possible for a high priority UE to increase the power and for a low priority UE to reduce power during the pre-emption. However, it may result in the poor system performance considering the possibility of the inaccurate sensing results. In our opinion, the power boosting or reduction for pre-emption scheme is not supported.

Proposal 14: The power reduction/boosting during the pre-emption operation is not supported.

PSCCH used to carry pre-emption information

The UE with the critical V2X packet can send the SCI with its selected resource information to other UEs. The low priority UE receives this SCI will trigger it to pre-empt its own selected resource which is conflict with the high priority UE’s selection.

Proposal 15: The preemption is trigged to the low priority UE by the SCI of higher priority UE.

Conclusions

In this contribution, we give our views on some remaining details on mode 2 resource allocation for NR V2X. Some design aspects are highlighted. The proposals are summarized as follows.

Observation 1: In NR V2X, when the determined number of the HARQ (re-)transmissions is not larger than 2, the LTE V2X step 2 mechanism can be reused.

Proposal 1: In the case where the determined number of the HARQ (re-)transmissions is not larger than 2, the LTE V2X step 2 mechanism should be reused in NR V2X,  and only the limited range in the condition should be expanded to [-31, 31] slots.

Proposal 2: In the case where the determined number of HARQ (re-)transmissions is larger than 2, the resources for the HARQ (re-)transmissions should be randomly selected one by one from the identified candidate resources in NR V2X.

Proposal 3: A chain should be formed during the step 2 in resource selection procedure, i.e. the selected resource for a retransmission of a TB should be reserved at least once by an SCI corresponding to a previous (re)transmission.

Proposal 4: When starting the nth resource selection to select resource Rn, all already selected resources {R1, R2 ... Rn-1}, and all the candidate resources which are in the same slot with {R1, R2 ... Rn-1}, should be precluded from the identified candidate resource set.

Proposal 5: The selected resource Rn should be within [-32, 32] slots of at least one resource from {R1, R2 ... Rn-1}.

Proposal 6: The step 2 of the resource selection procedure stops and can be regarded as completed when the number of the already selected resources has reached Mtarget or Sn-1∩Sn-1,=∅.

Proposal 7:

- In the case where Nmax=2,

  - For an mth (1≤m≤M-1) transmission opportunity in time,

    - The corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity;

  - The last (Mth) opportunity only indicates the resource for itself.

Proposal 8:

- In the case where Nmax=3,

  - For an mth (1≤m≤M-2) transmission opportunity in time,

    - If the gap b/w the opportunity m and m+2 is larger than 31, the corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity;

    - Else, the corresponding SCI of the mth opportunity indicates the resource for itself and the reservation resource of the (m+1)th opportunity and (m+2)th opportunity;

  - The last (Mth) opportunity only indicates the resource for itself, the (M-1)th opportunity indicates the resources for itself and the Mth opportunity.

Proposal 9: For resource re-selection of a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’ triggered by re-evaluation,

- The resource which is in the same slot and indicated by the corresponding PSCCH in slot ‘m’ should be regarded as the 1st selected resource;

- Whether other pre-selected but not reserved resources which are still in the candidate resource set is changed depends on UE implementation.

Proposal 10: For resource re-selection of a pre-empted resource contained in a slot ‘m’,

- If there is another resource is signaled by a same SCI with the pre-empted resource in case of Nmax=3, this “another resource” should be regarded as the 1st selected resource for the resource re-selection procedure;

- Else, the 1st resource is randomly selected in the selection window of the resource re-selection triggered by the pre-emption.

Proposal 11: The resource re-selection procedure should be triggered by MAC layer, on the basis of the reporting contents from physical layer.

Proposal 12: The resources that have previously been reserved for the UE(s) can be used or released by using HARQ feedback. The released resource could be used by other UEs after the other UEs monitoring the HARQ of the reserved UE(s).

Proposal 13: The other UEs need to monitor the HARQ ACK/NACK feedback when perform their own resource selection.

- If the feedback to the sending UE is ACK, the other UEs can start to use the released resource.

- If the feedback to the sending UE is NACK, the other UEs can avoid selecting the reserved resource.

Proposal 14: The power reduction/boosting during the pre-emption operation is not supported.

Proposal 15: The preemption is trigged to the low priority UE by the SCI of higher priority UE.

- References

[1] 3GPP TS 36.213, “Physical layer procedures”, V15.7.0, September 2019.

[2] 3GPP TS 36.321, “Medium Access Control (MAC) protocol specification”, V15.7.0, September 2019.