3GPP TSG RAN WG1 #100bis							R1-2001927

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.5

Source: 	LG Electronics

Title: 	Discussion on UE features for URLLC/IIoT

Document for:	Discussion and decision

- Introduction

In this contribution, we provide our view on UE feature/capability for URLLC/IIoT.

NR_L1enh_URLLC

On FG 11-1a, there is a FFS in “Note” column. Regarding the above FFS, monitoring DCI format 1_2 with 1_1 and 0_2 with 0_1 seems no significant difference from UE capability perspective. Furthermore, there is no separation from gNB configuration perspective. In other words, there is no case where a UE is configured to monitor DCI format 1_1(1_2) but not DCI format 0_1(0_2) for a given search space set. In this context, we do not see a clear need to separate this FG between DL DCI format and UL DCI format.

On FG 11-2, regarding “FFS: Whether to merge component 1) and 3), and accordingly merge component 2) and 4)”, we do not have a strong view but there is no significant need to merge them. This should be decided after the value of M and C for each span combination is decided.

For another FFS point on 3 PDSCH/PUSCH per slot, it would be necessary for efficient scheduling under (4, 3) pair.

On FG 11-2b, we are fine to have separate capability between Rel-16 only (FG 11-2) and mixed capabilities (FG 11-2b). As pointed out by others, it is possible to configure FG 3-1 for some serving cells and FG 3-5b for other serving cells and no separate capability are defined in Rel-15 to indicate whether particular combinations are supported or not. In this context, even the note ([Rel-15 monitoring capability here is subjected to the capability of FG 3-1, FG 3-2 and FG 3-5b.]) may not be needed.

On FG 11-3, the component 3) has been added but it is unclear whether this component is really necessary. For instance, for a UE capable of 2-symbol subslot configuration, what is the difference for supporting between (4,2) and (2,2)? Our suggestion is to remove the component 3) and the corresponding note.

On FG 11-4, the description of FG needs to be updated as there is no definition on priority of “PDSCH reception”. It can be updated as “Two HARQ-ACK codebooks simultaneously constructed supporting PDSCH reception with different priorities of the corresponding HARQ-ACK feedback at a UE”. Regarding the separation of UE capability for different combinations based on slot-based HARQ-ACK codebook and sub-slot based HARQ-ACK codebook, our first preference is to set the following combinations: (1) One is slot-based and one is sub-slot-based, (2) Both are slot-based, and (3) Both are sub-slot-based. However, if the test efforts really need to be considered, we are fine with the compromised option from Rapporteur: (1) At least one is slot-based, and (2) Both are sub-slot-based.

For the component 4), we think the bracket needs to be removed. For the component 6), considering this FG 11-4 would entail multiple PUCCHs with different priorities in a slot, it would be reasonable to have the component.

Regarding “FFS: Whether and how to combine FG 11-4 and FG 12-1”, it can be understood that two HARQ-ACK codebook construction is related to intra-UE prioritization. In fact, the component 3) of FG 11-4 may be a part of FG 12-1. However, the other components of FG 11-4 would be just to support two HARQ-ACK codebooks with different priorities itself rather than only intra-UE prioritization. In this context, the benefit and methodology are a bit questionable to merge two FGs into a FG.

Regarding “FFS: For component 4), whether to separate DL priority and UL priority, and whether to separate DCI format 0_1/1_1 and DCI format 0_2/1_2”, we think there is no need for separation between DL and UL priorities.

On FG 11-4a, we think this FG can be further separated into two FGs: one for DCI format 0_1/1_1 and another for DCI format 0_2/1_2. With this, more flexibility can be ensured such that DCI format 0_1/1_1 can schedule two priorities while DCI format 0_2/1_2 can schedule only one priority.

For the prerequisite FG of FG 11-4a, considering FG 11-4a is not related to whether a UE is capable of monitor both DCI format 0_1/1_1 and 0_2/1_2 in the same search space, our suggestion would be 11-1 and 11-4 rather than 11-1a and 11-4.

Regarding the FFS on separation between DL and UL (11-4x/11-4y), we do not see a essential need for further separation.

On FG 11-5, it should be clear whether component 8 is for single TB or for different TBs. As Rapporteur’s view, we would have UE capability on the total number of unicast PUSCHs for different TBs per slot per CC. If component 8 restricts the total number of actual repetiton for different TB per slot, the total number of unicast PUSCH would be restricted by the smaller of those two capability. If component 8 restricts the total number of actual repetiton for a TB per slot, number of actual repetition may not be restricted by the number of unicast PUSCH in a slot. We don’t have strong view on that, we think that following questions need to be clarified:

- Are two actual repetitions in a slot counted as two unicast PUSCH in a slot?

- Is component 8 for different TB or only for single TB?

Regarding FFS on the separation between DG and CG, we think it is not necessary to have separated FG for dynamic grant and configured grant. We support Rapporteure suggestion, to have a simple note “PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20”.

Regarding the FFS on the seperation between DCI format 0_1 and 0_2, we are fine with majority view, i.e., no separation.

On FG 11-7, Tproc, 2 is in absolute time unit, however, d would be a value in symbol level. For simplicity, we would like to add unit. For example, “after Tproc, 2 +d symbol” or “after d symbol after Tproc, 2”.

Regarding FFS on more than one monitoring occasion, we support raportuer’s suggestion. If UE can monitor UL grant with <1 slot periodicity, UE should be able to monitor UL CI with <1 slot periodicity as well. It is not necessary to make specific monitoring capabilty only for UL CI. Moreover, UL CI already has restrcition in terms of the number of BD. If UL CI has same restriction on type 3 CSS like other DCI format, there won’t be a problem.

On FG 11-9/9a, though we don’t have a strong view, we are fine to add both components 2 and 3.

Regarding the number of PUSCHs for different TBs in a slot, we think that the total number of unicast PUSCH for different TB in a slot per configuration can re-use the capability of 11-5(if any. Otherwise, 5-12, 5-12a, 5-12b, 5-13d, 5-13e, 5-13f can be re-used). Among CG configuration, it is up to UE implementation which CG PUSCH is actually used within capability. In Rel-16, UE can be configured with multiple CG configurations and these can be overlapped. In our point of view, even if a UE has a capabilty for DG like 5-13, i.e., up to two unicast PUSCH, the UE can be configured with more than two CG configurations as long as each of CG configuration with proper configured periodicity and the number of repetitions is complied with FG 5-13. Among multiple configurations, the UE will use up to two CG PUSCH resources for actual transmission in a slot under the given UE capability, and if another PUSCH is scheduled in the slot by UL grant in a DCI, then in this case the UE will use up to one CG PUSCH resource due to given UE capability.

On FG11-10/11, we are fine to combine FG 11-10 and FG 11-11 as one feature group for simplicity. It may be necessary to add notes “Type 2 configured grant release by DCI format 0_2 is subjected to the capability reported by FG 11-1”.

NR_IIOT

On FG 12-1, as noted above in the comment on FG 11-4, the benefit and methodology are a bit questionable to merge two FGs into a FG.

In addition to the current FGs, it is necessary to further discuss on whether to define the following FGs:

- Support of SPS periodicity shorter than 10 ms

  - Although this is just a straightforward extension of Rel-15 SPS operation, this is not included anywhere so better to clarify.

- Support of SPS activation by DCI format 1_2

  - Similarly, it would be good to clarify whether there is a need to include this functionality somewhere.