3GPP TSG RAN WG1 #100bis			R1-2002007

e-Meeting, April 20th – 30th, 2020

Source:	Intel Corporation

Title:	Low PAPR Reference Signals

Agenda item:	7.2.6.5

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss corrections to Low PAPR Reference Signals that has been introduced in Rel-16 as part of MIMO enhancement WID [1].

# Discussion

As part of normative work on low PAPR reference signals in Rel-16 NR, enhancements to PUCCH DM-RS for PUCCH format 3 and 4 were introduced for the case when transform precoding is used in conjunction with π/2-BPSK modulation. It was agreed that for sequences of length 30 and larger, Gold sequences generated in pre-DFT time domain will be used, whereas for sequences less than length 30, computer generated sequences (CGS) were designed. Both the designs yield non-constant modulus sequences when mapped to the frequency domain after transform precoding. It was also agreed that a single PUCCH port is supported for both PF3 and PF4.

In Rel-15 NR, DM-RS sequences were based on frequency domain constant modulus CGS and ZC sequences and cyclic shifts α were specified yielding orthogonal sequences from the same root sequence enabling port multiplexing. Interference averaging by ensuring different sequences across cells for the case of CGS sequences was achieved by ensuring good cross correlation properties as well as sequence and group hopping mechanisms which have been carried forward in Rel-16.

However, current specification indicates that cyclic shift values can be used even for π/2-BPSK DM-RS for PUCCH formats 3 and 4. Since the sequence design was undertaken for single port, cross correlation across different root sequences was optimized. The existence of sequence and group hopping mechanisms make the use of cyclic shift somewhat redundant and therefore current sequences do not depend on the value of α. This needs to be captured in current specification and to this end, the following TP for TS 38.211 is proposed:

### 5.2.3 Low-PAPR sequence generation type 2 The low-PAPR sequence ru,v(α,δ)n is defined by a base sequence ru,vn according to ru,v(α,δ)n=ru,vn, 0≤n<M where M=mNscRB2δ is the length of the sequence. Multiple sequences are defined from a single base sequence through different values of α and δ.

##### 6.4.1.3.3 Demodulation reference signal for PUCCH formats 3 and 4 ###### 6.4.1.3.3.1 Sequence generation The reference-signal sequence rlm shall be generated according to where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration: - if the higher-layer parameter dmrsUplinkTransformPrecodingPUCCH-r16 is configured, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1. and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2. - otherwise, ru,vα,δ(m) is given by clause 6.3.2.2. The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

# References

[1] RP-182863, Revised WID: Enhancements on MIMO for NR, Samsung, RAN#82, Sorrento, December 2018.

[2] TS 38.211, “Physical Channels and Modulation”, v16.1.0