3GPP TSG RAN WG1 #100bis                              	                  R1- 2002277

e-Meeting, 20th April – 30th April, 2020

Agenda Item:	7.2.2.2.5

Source:	Spreadtrum Communications

Title:	Remaining issues in Wide-band operation

Document for:	Discussion and decision

Introduction

In RAN1#100e meeting, the following had been agreed [1]:

Agreement: - If CORESET p is not configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16, - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart, where the first common RB of the first group of 6 consecutive RBs has common RB index 6∙NBWPstart6, i.e., same as in Rel-15. - If CORESET p is not configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16, - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset, where the UE assumes the default value rb-Offset = 0. - If CORESET p is configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16, - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset. - If CORESET p is configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16, - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset. - Note: A bits in above bullets is defined as floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6), as per previous agreement. - TS 38.213 editor to implement this agreement

In this contribution, we provide our views on CORESET and search space set configuration.

Discussion

CORESET and Search space set configuration

At the last meeting, the UE behavior of PDCCH monitoring in wide-band shared spectrum had been agreed. A CORESET associated with a search space set configured with freqMonitorLocations-r16 could be also configured with another search space set not configured with freqMonitorLocations-r16. However, there is still ambiguity that a CORESET is mapped to first RB-set 0 in BWP through frequencyDomainResources, but bitmap of freqMonitorLocation-r16 may indicate that CORESET is not in RB-set 0. Current spec could work if the CORESET associated with only one search space set. For example, the CORESET replication can be assigned to other available RB-set which is indicated through freqMonitorLocations-r16. However, if the same CORESET is associated with another SS set that is not configured with freqMonitorLocations-r16, there will be an issue with the UE behavior of CORESET reception. For example, for a search space set configured with freqMonitorLocations-r16, which indicates that its associated CORESET is not in RB-set 0, the UE could expect to receive the CORESET replication in other available RB-set. For search space set configured without freqMonitorLocations-r16, the UE expect to receive the CORESET through the configuration of frequencyDomainResources. Then, the gNB should transmit the CORESET and its replication in RB-set 0 and other RB-set respectively, if these RB-sets pass LBT. When all the RB-sets except RB-set 0 fail LBT, then UE could not receive CORESET associated with search space set configured with freqMonitorLocations-r16. A simple solution to this problem is that the UE should ignore the configuration of freqMonitorLocations-r16 in RB-set 0 and follow the configuration of frequencyDomainResources in RB-set 0.

Proposal 1: if the configuration of freqMonitorLocations-r16 contradicts with the configuration of frequencyDomainResources in RB-set 0:

- UE should ignore the configuration of freqMonitorLocations-r16 in RB-set 0 and follow the configuration of frequencyDomainResources in RB-set 0.

------------------------------------------ Text Proposal for 38.213, Section 10.1 --------------------------------------------- *** Unchanged text omitted *** For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset. - if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. - If a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, and the search space set configured with freqMonitorLocation-r16 indicates that CORESET is not in RB-set 0, the UE ignores the configuration of freqMonitorLocations-r16 in RB-set 0 and follows the configuration of frequencyDomainResources in RB-set 0. *** Unchanged text omitted *** ------------------------------------------------------ End Text Proposal -------------------------------------------------------

Conclusion

In this contribution, we have discussed the CORESET and search space set configuration for NR-U wide-band operation. Based on the discussion in section 2, we provide the following proposals.

Proposal 1: if the configuration of freqMonitorLocations-r16 contradicts with the configuration of frequencyDomainResources in RB-set 0:

- UE should ignore the configuration of freqMonitorLocations-r16 for RB-set 0 and follow the configuration of frequencyDomainResources in RB-set 0.

Reference

- RAN1#100e Chairman notes