3GPP TSG-RAN WG1 #100-bis-e		 	R1-2002377

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.4.2.1

Title:	On Resource Allocation Mode 1 for NR V2X

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Agreements:

- The mapping between the values of HPN signaled in DCI and HPN signaled in SCI is fixed for a TB, and is up to UE implementation.

- For dynamic grant, the toggling of NDI in DCI is used as the toggling of NDI in SCI for the first SL transmission scheduled by the DCI. The SCI for the remaining transmissions scheduled by the DCI, if any, have the NDI untoggled with respect to the first SL transmission.

- FFS NDI in SCI for PUCCH ACK-NACK error cases

Agreements:

- The HARQ process ID for each transmission in a resource corresponding to a SL configured grant is determined based on the formula used for UL configured grants.

  - The mapping with the values of HPN in SCI is fixed for a TB, and is up to UE implementation.

Note: This corresponds to the HARQ process ID for the interaction between gNB and UE, if any distinction is made.

Note: if the principle itself is agreeable, we can then discuss any need for modifications, possibly as part of the TP drafting phase.

Agreements:

- Only one new TB can be transmitted in one period of the configured grant.

  - FFS any issue with retransmission spanning multiple periods

- The DCI scheduling the retransmissions uses the first HARQ process ID corresponding to the first transmission of the TB, as agreed for Q2.

Agreements:

The specification supports having multiple HARQ ID processes for a given SL configured grant.

Note: this is issue is tightly coupled with Q2, so all configuration and specification details can be discussed together.

The following agreements for resource allocation in mode 1 for NR were made in RAN1 #100-e [1]:

In this paper, we discuss some details of DCI design in particular DCI contents for NR V2X mode 1.

# DCI Contents

For NR V2X mode 1, a new DCI format, DCI format 3_0 [2], is defined to be used for scheduling the NR PSCCH and PSSCH. The DCI format 3_0 is used to schedule a dynamic grant or activate/deactivate type-2 configured grant when the CRC is scrambled by SL-RNTI or SL-CS-RNTI respectively.

The following downlink control information fields have been specified for DCI format 3_0 [2]:

- Time gap

- HARQ process ID

- New data indicator

- Lowest index of the subchannel allocation to the initial transmission

- SCI format 0-1 fields

  - Frequency resource assignment.

  - Time resource assignment.

- PSFCH-to-HARQ feedback timing indicator

- PUCCH resource indicator

- Configuration index

During RAN #86 meeting, a list of the tasks that RAN1 needs to focus on for 5G V2X was summarized in RP-193198 [3]. On the task list, one remaining issue of the NR V2X mode 1 is the details on DCI contents for dynamic grant and Type-2 configured grant. In this section, we discuss the details on this aspect.

## DCI contents for dynamic grant

In NR V2X mode 1, for dynamic grant scenario, gNB sends a DCI to schedule a sidelink transmission with the resources to be used. At RAN1 #98bis, it had been agreed that a UE-specific SL-RNTI will be introduced for the DCI for dynamic grant. Since it is supported that the DCI can dynamically schedule broadcast, groupcast and unicast transmission on the sidelink, to reduce the burden of the blind decoding of the DCI on the UE side, it is preferable that gNB sends the DCI using the same DCI format for all these three cases.

Proposal 1. Same DCI format is used for resource allocation for broadcast, groupcast and unicast in mode 1.

Regarding the contents of information fields of the DCI, in addition to the above downlink control information already specified for DCI format 3_0 [2], the following downlink control information may also be carried by a DCI format 3_0 to allow gNB to flexibly schedule a sidelink transmission with dynamic signalling on downlink:

- Cast type indicator: Since the DCI format 3_0 can be used for scheduling broadcast, groupcast or unicast, this field is used to indicate the communication type of the scheduled sidelink transmission.

- Resource pool index: In V2X mode 1, a UE may be configured with multiple transmit resource pools, this field is used to indicate the resource pool to be used for the scheduled sidelink transmission.

- Destination ID: In some cases, a gNB may receive multiple SR from the same UE requesting grants for sidelink transmissions to different destination UEs. This field may be used to indicate the destination of the scheduled sidelink transmission.

- PSFCH resource indicator: Different from the V2X mode 2, gNB may have full control of sidelink resource allocation in V2X mode 1. It is beneficial for resource utilization if gNB can flexibly schedule the feedback resources on the sidelink. Similar to the PUCCH resource indicator, this field is used to indicate the PSFCH resources to be used for the feedback of a scheduled sidelink transmission.

Proposal 2. In addition to the already specified downlink control information for dynamic scheduling on sidelink, the following control information may also be indicated by a DCI format 3_0:

- Cast type indicator

- Resource pool index

- Destination ID

- PSFCH resource indicator

## DCI contents for Type-2 configured grant

When a DCI format 3_0 is used for activating/deactivating a type-2 configured grant, in addition to the above specified control information in DCI format 3_0 [2], the following information may also be included in the DCI:

- Activation/deactivation indicator: This field is used for activation or deactivation of a resource configuration for type-2 configured grant.

- Cast type indicator: Since the DCI format 3_0 can be used for activating type-2 configured grant for broadcast, groupcast or unicast, this field may also be used to indicate the communication type of a resource configuration for an activated type-2 configured grant, if necessary.

- Destination/Zone ID: A UE may be connected with multiple other UEs on the sidelink and may communicate with them using type-2 configured grant. This field may be used to indicate the destination or zone for a resource configuration for the activated type-2 configured grant.

Proposal 3. In addition to the already specified downlink control information, the following control information may also be indicated by the DCI format 3_0 for a type-2 configured grant:

- Activation/deactivation indicator

- Cast type indicator

- Destination/Zone ID

# Conclusion

In this contribution, we discussed the details on DCI contents and resource allocation for NR V2X mode 1. The following proposals were made:

Proposal 1. Same DCI format is used for resource allocation for broadcast, groupcast and unicast in mode 1.

Proposal 2. In addition to the already specified downlink control information for dynamic scheduling on sidelink, the following control information may also be indicated by a DCI format 3_0:

- Cast type indicator

- Resource pool index

- Destination ID

- PSFCH resource indicator

Proposal 3. In addition to the already specified downlink control information, the following control information may also be indicated by the DCI format 3_0 for a type-2 configured grant:

- Activation/deactivation indicator

- Cast type indicator

- Destination/Zone ID

# References

- Draft Report of 3GPP TSG RAN WG1 #100-e v0.2.0 (Online meeting, 24th February – 6th March 2020).

- TS 38.212 “Multiplexing and channel coding (Release 16)”, V16.0.0 (2019-12)

- RP-193198, “Task list for 5G V2X in RAN1#100”, RAN #86, Sitges, ES, December 9th – 12th, 2019