3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002387

e-Meeting, April 20th – 30th, 2020

Source:	Sharp

Title:	Remaining issues on resource allocation mode 1 for NR sidelink

Agenda Item:	7.2.4.2.1

Document for:	Discussion and Decision

Introduction

In this document, we share our views on a few remaining issues on resource allocation mode 1 for NR sidelink.

Discussion

Size alignment for DCI format 3_0 and 3_1

The following were agreed regarding handling of the sizes of DCI format 3_0 and 3_1.

RAN1#98bis agreements: - Two different UE-specific SL RNTIs are introduced for Mode-1 scheduling: one for CRC scrambling in DCI for a dynamic grant and the other one for CRC scrambling in DCI for a configured grant type-2. - The two above DCIs have the same size RAN1#99 agreements: - Existing DCI size budget is maintained when the UE is configured with SL - (working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned. RAN1#100-e agreements - If UE is configured to monitor DCI 3_0, the sizes of DCI 3_1 and DCI 3_0 are is aligned by zero padding to DCI 3_0 size. - If UE is not configured to monitor DCI 3_0, the mechanism for size alignment between DCI 3_0 and Uu DCI is reused for size alignment between DCI 3_1 and Uu DCI

The above agreements on size alignment between DCI format 3_0 and 3_1 were specified in TS 38.212 as follows,

##### 7.3.1.4.1 Format 3_0 DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell. The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI: - Time gap – [x] bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause x.x.x of [6, TS 38.214] […] If the UE is configured to monitor DCI format 3_1 and the number of information bits in DCI format 3_0 is less than the payload of DCI format 3_1, zeros shall be appended to DCI format 3_0 until the payload size equals that of DCI format 3_1. ##### 7.3.1.4.2 Format 3_1 DCI format 3_1 is used for scheduling of LTE PSCCH and LTE PSSCH in one cell. The following information is transmitted by means of the DCI format 3_1 with CRC scrambled by SL-L-CS-RNTI: - Timing offset – 3 bits determined by higher layer parameter sl-TimeOffsetEUTRA, as defined in clause 16.6 of [5, TS 38.213] […] If the UE is configured to monitor DCI format 3_0 and the number of information bits in DCI format 3_1 is less than the payload of DCI format 3_0, zeros shall be appended to DCI format 3_1 until the payload size equals that of DCI format 3_0.

However, it remains to be specified in TS 38.212 how to perform size alignment between DCI format 3_0/3_1 and other DCI formats. As can be seen in sub clause 7.3.1.0 of TS 38.212, after the size alignment procedure for NR Uu DCI formats there are a total of up to 4 DCI sizes for these DCI formats in a cell. In order not to increase the DCI size budget in a cell, DCI formats 3_0/3_1 should be padded to align with one of these DCI sizes. On the other hand, in order to perform the DCI size alignment in a future-proof manner, it would be better not to fix the DCI format to align to. Instead, the size of DCI formats 3_0/3_1 should be aligned to the smallest value among NR Uu DCI sizes larger than the size of DCI formats 3_0/3_1 prior to padding

Proposal 1: The size of DCI formats 3_0/3_1 are aligned to the smallest value among the NR Uu DCI sizes larger than the size of DCI formats 3_0/3_1 prior to padding.

A TP is also provided below for sub clause 7.3.1.0 of TS 38.212.

#### < Unchanged parts are omitted > #### 7.3.1.0 DCI size alignment If necessary, padding or truncation shall be applied to the DCI formats according to the following steps executed in the order below: Step 0: - Determine DCI format 0_0 monitored in a common search space according to clause 7.3.1.1.1 where is the size of the initial UL bandwidth part. - Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. - If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0. - If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to truncation is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals the size of the DCI format 1_0. Step 1: - Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where is the size of the active UL bandwidth part. - Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where is the size of the active DL bandwidth part. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in DCI format 0_0 in UE-specific search space for the SUL is not equal to the number of information bits in DCI format 0_0 in UE-specific search space for the non-SUL, a number of zero padding bits are generated for the smaller DCI format 0_0 until the payload size equals that of the larger DCI format 0_0. - If DCI format 0_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0. - If DCI format 1_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 1_0 prior to padding is less than the payload size of the DCI format 0_0 monitored in UE-specific search space for scheduling the same serving cell, zeros shall be appended to the DCI format 1_0 until the payload size equals that of the DCI format 0_0 Step 2: - Determine DCI format 0_1 monitored in a UE-specific search space according to clause 7.3.1.1.2. - Determine DCI format 1_1 monitored in a UE-specific search space according to clause 7.3.1.2.2. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1. - If the size of DCI format 0_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1. - If the size of DCI format 1_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1. Step 2A: - Determine DCI format 0_2 monitored in a UE-specific search space according to clause 7.3.1.1.3. - Determine DCI format 1_2 monitored in a UE-specific search space according to clause 7.3.1.2.3. - For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_2 for the SUL is not equal to the number of information bits in format 0_2 for the non-SUL, zeros shall be appended to smaller format 0_2 until the payload size equals that of the larger format 0_2. - [If the size of DCI format 0_2 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_2.] - [If the size of DCI format 1_2 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_2.] - If DCI format 3_0 and/or DCI format 3_1 is monitored in a UE-specific search space and the size of the DCI format 3_0/3_1 does not equal that of any other DCI format configured to monitor for the cell, zeros shall be appended to the DCI format 3_0 and/or DCI format 3_1 until the payload size equal the smallest value among the sizes of other DCI formats configured to monitor for the cell which are larger than that of the DCI format 3_0/3_1 prior to padding. Step 3: - If both of the following conditions are fulfilled the size alignment procedure is complete - the total number of different DCI sizes configured to monitor is no more than 4 for the cell - the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell Step 4: - Otherwise Step 4A: - Remove the padding bit (if any) introduced in step 2 and step 2A above. - Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. - Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where is the size of the initial UL bandwidth part. - If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 monitored in a UE-specific search space until the payload size equals that of the DCI format 1_0 monitored in a UE-specific search space. - If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to truncation is larger than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 monitored in a UE-specific search space equals the size of the DCI format 1_0 monitored in a UE-specific search space. Step 4B: - If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps - If the number of information bits in the DCI format 0_2 prior to padding is less than the payload size of the DCI format 1_2 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_2 until the payload size equals that of the DCI format 1_2. - If the number of information bits in the DCI format 1_2 prior to padding is less than the payload size of the DCI format 0_2 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_2 until the payload size equals that of the DCI format 0_2. Step 4C: - If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps - If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1. - If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1. Step 4D: - If DCI format 3_0 and/or DCI format 3_1 is monitored in a UE-specific search space and the size of the DCI format 3_0/3_1 does not equal that of any other DCI format configured to monitor for the cell, zeros shall be appended to the DCI format 3_0 and/or DCI format 3_1 until the payload size equal the smallest value among the sizes of other DCI formats configured to monitor for the cell which are larger than that of the DCI format 3_0/3_1 prior to padding. The UE is not expected to handle a configuration that, after applying the above steps, results in - the total number of different DCI sizes configured to monitor is more than 4 for the cell; or - the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or - the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or - the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space; or - [the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_2 in another UE-specific search space; or] - [the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_2 in another UE-specific search space.] #### < Unchanged parts are omitted >

HARQ process ID determination for sidelink configured grants

The principle on HARQ process ID determination for configured grants was agreed in RAN1#100e as follows,

Agreements (Q2): - The HARQ process ID for each transmission in a resource corresponding to a SL configured grant is determined based on the formula used for UL configured grants. - The mapping with the values of HPN in SCI is fixed for a TB, and is up to UE implementation. Note: This corresponds to the HARQ process ID for the interaction between gNB and UE, if any distinction is made. Note: if the principle itself is agreeable, we can then discuss any need for modifications, possibly as part of the TP drafting phase.

We noticed that the potential modifications to the HARQ process ID determination in NR UL configured grant remain undecided. From our perspective, there would be mainly two aspects for consideration.

- Considering that multiple configured grants with potential different periodicities are supported in NR sidelink, to avoid the HARQ process ID collision between different configured grant, we propose to maintain the (pre-)configured parameter of HARQ process offset (sl-HARQ-ProcID-offset-r16) and accordingly introduce the parameter into the formula of determining HARQ process ID.

- The other point lies mainly on the (pre-)configuration of the configured grant. On top of the determined resource pool(slots within a resource pool denoted as tqSL), it would be better to define the periodicity of the configured grant in unit of logical slots, i.e. slots within the resource pool.

Thus, we might get the modified formula to determine the HARQ process ID for the sidelink configured grants as follows,

- HARQ Process ID=floorqperiodicitymodulo nrofHARQ-Processes+(HARQ-ProcID-offset).

  - q is the subscript of the slot within the (pre-)configured resource pool.

Therefore, we have the following proposal.

Proposal 2: For SL configured grants, the HARQ process ID associated with the first symbol of a SL transmission is derived from the following equation:

- HARQ Process ID=floorqperiodicitymodulo nrofHARQ-Processes+(HARQ-ProcID-offset).

  - q is the subscript of the slot within the (pre-)configured resource pool.

TDRA/FDRA field setting in SCI

In Rel-14 LTE V2X, if the field “Time gap between initial transmission and retransmission” in DCI format 5A for resource allocation mode 3 is set as non-zero, it would imply that eNB schedules two sidelink resources for initial transmission and blind retransmission, respectively. The scheduled UE sets the field “Retransmission index” as ‘0’ and ‘1’ respectively representing the initial transmission and blind retransmission of a TB.

For mode 1 in NR sidelink, it is also supported that gNB might schedule multiple sidelink resources(at most 3) and accordingly indicate the resources in DCI format 3_0. The agreement on the interpretation of TDRA in both DCI format 3_0 and SCI format 0-1 was reached in RAN1#100e as follows,

Agreements: - Time resource assignment in SCI uses an extended time domain RIV mechanism as follows: if N=1 TRIV=0 elseif N=2 TRIV=T1 else if T2-T1-1≤15 TRIV=30T2-T1-1+T1+31 Else TRIV=3031-T2+T1+62-T1 end if end if where - N denotes the actual number of resources indicated - Ti denotes i-th resource time offset - for N=2, 1≤T1≤31 - for N=3, 1≤T1≤30, T1<T2≤31

In NR sidelink, there would be no restriction on how the scheduled UE utilizes the sidelink resources, which means e.g. UE transmits initial transmission of a TB in the 1st scheduled SL resource and might transmit initial transmission of another TB in the 2nd scheduled SL resource. Nevertheless, the scheduled UE should never indicate SL resource in the SCI format 0-1 transmission beyond the SL resource(s) indicated in the corresponding DCI format 3_0, which means e.g. if gNB indicates two actual SL resources, i.e.N=2 and RIV=T1 in DCI format 3_0, when UE is transmitting PSCCH/PSSCH in the 2nd scheduled SL resource, the UE should set TRIV=0 as the UE only needs to indicate the resource of the current transmission and applies accordingly N=1 case, i.e. TRIV=0. Similar mechanism applies for FDRA field setting in SCI format 0-1. Therefore, we have the following proposal.

Proposal 3: For NR sidelink mode 1, if gNB schedules N(1≤N≤3) SL resources in DCI format 3_0, the scheduled UE sets TDRA/FDRA field in SCI format 0-1 corresponding to the ith(i≤N) SL resource such that indicates the ith, (i+1)th,…, Nthscheduled SL resource(s).

Conclusion

In this contribution, we discuss some remaining issues on resource allocation mode 1 for NR sidelink, make the following proposals and provide corresponding TPs in section 5.

Proposal 1: The size of DCI formats 3_0/3_1 are aligned to the smallest value among the NR Uu DCI sizes larger than the size of DCI formats 3_0/3_1 prior to padding.

Proposal 2: For SL configured grants, the HARQ process ID associated with the first symbol of a SL transmission is derived from the following equation:

- HARQ Process ID=floorqperiodicitymodulo nrofHARQ-Processes+(HARQ-ProcID-offset).

  - q is the subscript of the slot within the (pre-)configured resource pool.

Proposal 3: For NR sidelink mode 1, if gNB schedules N(1≤N≤3) SL resources in DCI format 3_0, the scheduled UE sets TDRA/FDRA field in SCI format 0-1 corresponding to the ith(i≤N) SL resource such that indicates the ith, (i+1)th,…, Nthscheduled SL resource(s).

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.