3GPP TSG RAN WG1 #100-Bis-e	R1-2002425

e-Meeting, April 20th – April 30th, 2020

Agenda Item:	7.2.11.7

Source:	Ericsson

Title:	Discussion on UE features for UEPS

Document for:	Discussion and Decision

# 1	Introduction

We provide our input for UE PS related features.

# 2	Discussion

We are fine with features for UE power savings (in R1-2001484) with following suggested updates.

- 19-1

- Minor typo in note should be fixed (3_0 -> 2_6).

- Our preference is to have per-UE indication, while allowing indicating differentiation between FRx/xDD.

- 19-2 :

- To reduce capability signalling overhead, it is highly preferable to avoid having cross-slot scheduling support indicated at feature set level, since the offset indicator in the DCI is purely a baseband function, there is no need to have very granular support indication, and UE can anyways choose to not take advantage of micro-sleep opportunities provided by the bit in the DCI.

- Our preference is to have per-UE indication, while allowing indicating differentiation between FRx/xDD.

19-3 :

  - The UE capability to support per-BWP configuration of maximum MIMO layers is signaled to the network. Here it would be preferred that the capability is indicated per UE and not per BC. The RRC parameter for per-BWP max-MIMO layers indicates the PDSCH scheduling restriction that UE can expect for that BWP, a form of NW assistance for the UE to save power. It is then up to the UE implementation how to use it for power savings. Signaling per UE is enough, whereas signaling per BC would also lead to a larger signaling overhead. Additionally, differentiation based on frequency range/xDD seems OK.

- Our preference is to have per-UE indication, while allowing indicating differentiation between FRx/xDD.

- 19-4a:

- UE preferred K0/K2 for Cross-carrier scheduling is FFS in RAN1 -  it should be reflected in the feature list. Can add LS to RAN2 (R1-1911586) in note  for detailed description.

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | - Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset K2 configuration for PUSCH |  |  |  |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |  |  |  |  | N/A |  | [Per BC] | No | No |  | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication | Optional with capability signalling |
|  | 19-4a | UE assistance information | - Preferred k0/k2 [for same carrier scheduling] - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted | Optional with capability signalling |