3GPP TSG RAN WG1 #100bis-e		                                                 R1-2002481

e-Meeting, April 20th – 30th, 2020

Agenda item: 	7.2.11.7

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for UE Power Savings

Document for:	Discussion and Decision

# Introduction

The Rel-16 UE feature e-mail discussions have gone through two phases in RAN1 e-mail reflector, with the current proposals available in [1]. In this contribution we present our further views regarding the UE features for UE Power Savings.

# Discussion

The detailed comments are provided below following the numbering of features in [1], and which can be found in the Annex for reference:

- 19-3: Type definition should be per UE.

Justification:

RAN1 made following agreements in RAN1#98:

Agreements: - Support per-DL-BWP configuration of maximum number of DL MIMO layers - Signalling details up to RAN2 Agreements: - No spec change for TS38.212 is needed for determining DL-SCH TBSLBRM when downlink max MIMO layer adaptation is configured. - Note: DL-SCH TBSLBRM is not dependent on the per-BWP configured maximum number of DL MIMO layers value. - When maximum number of DL MIMO layers per BWP is configured for all configured DL BWPs for a serving cell, - At least one BWP is configured with per-BWP configured maximum number of DL MIMO layers value equal to the per-cell configured maximum number of DL MIMO layers value (if configured).

As the capability 19-3 informs whether UE supports the new BWP specific parameter, maxMIMO-Layers-r16, and does not require other UE specific behaviour e.g. in terms of TBSLBRM determination, there does not seem to be any band specific dependency. Possible use of the parameter to adjust UE receiver algorithm, in network transparent manner, is up to the UE.

# Summary

In this contribution we presented our proposals on UE features for UE Power Savings.

# References

[1] R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO, April 2020

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | - Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset K2 configuration for PUSCH |  |  |  |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |  |  |  |  | N/A |  | [Per BC] | No | No |  | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication | Optional with capability signalling |
|  | 19-4a | UE assistance information | - Preferred k0/k2 [for same carrier scheduling] - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted | Optional with capability signalling |