3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002579

E-meeting, April 20 - April 30, 2020

Agenda Item:	7.2.10.5

Source:	Huawei, HiSilicon

Title:	Remaining issues on intra-band CA with unaligned frame boundary

Document for:	Discussion and Decision

Introduction

After RAN1#100-e meeting [1], most remaining issues on intra-band CA with unaligned frame boundary are completed and endorsed in the specifications. In this contribution, some remaining details on non-SFN aligned CA will be discussed and corresponding TPs will be proposed.

Discussion

For the lowest SCS combinations of 60kHz/60kHz and 120kHz/120kHz, the current specification 38.211 [2] does not define the start slot 0 SCS for the cell whose point A has a lower frequency which is not aligned with the agreed TP in the last meeting and is not clear for the start slot 0 SCS. It is proposed to adopt the following text proposal.

Proposal 1: To align with the agreements in RAN1#100-e, adopt the following TP in TS 38.211.

q=-1q=-1

-------------------------------------- Start TP for TS 38.211 sub-clause 4.5 ---------------------------------------

<Unchanged parts omitted>

4.5	Carrier aggregation

Transmissions in multiple cells can be aggregated. Unless otherwise noted, the description in this specification applies to each of the serving cells.

For carrier aggregation of cells with non-aligned frames, the slot offset Nslot, offsetCA between a PCell/PScell and an SCell is determined by higher-layer parameter CA-slot-offset for the SCell. The quantity μoffset is defined as the maximum of the lowest subcarrier spacing configuration among the subcarrier spacings given by the higher-layer parameters SCS-SpecificCarrierList configured for PCell/PSCell and the SCell, respectively. The slot offset Nslot, offsetCA fulfills

-	when the lowest subcarrier spacing configuration among the subcarrier spacings configured for the cell is μ=2 for both cells or μ=3 for both cells, the start of slot 0 corresponding to the lowest subcarrier spacing configuration given by SCS-SpecificCarrierList for the cell whose point A has a lower frequency coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell where  q=-1 if point A of the PCell/PSCell has a frequency lower than the frequency of point A for the SCell, otherwise q=1;

-	otherwise, the start of slot 0 for the cell with the lower subcarrier spacing of the lowest subcarrier spacing given by the higher-layer parameters SCS-SpecificCarrierList configured for the two cells, or the Pcell/PSCell if both cells have the same lowest subcarrier spacing given by the higher-layer parameters SCS-SpecificCarrierList configured for the two cells, coincides with the start of slot qNslot, offsetCA mod Nslotframe,μ for the other cell where  q=-1 if the lowest subcarreier spacing configuration given by SCS-SpecificCarrierList of the PCell/PSCell is smaller than or equal to the lowest subcarrier spacing given by SCS-SpecificCarrierList for the SCell, otherwise q=1.

<Unchanged parts omitted>

-------------------------------------- End TP for TS 38.211 sub-clause 4.5 ---------------------------------------

Conclusion

According to the above discussions, we propose to adopt the above mentioned TP.

- References

- 3GPP RAN1 Chairman’s notes, RAN1#100-e, Feb.24– Mar.06, 2020.

- 3GPP TS 38.211: “NR; Physical channels and modulation”.