3GPP TSG RAN WG1 Meeting #100bis-e 	 R1-2002593

E-meeting, April 20 - April 30, 2020

Agenda Item:	7.2.11.7

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for UE power saving

Document for:	Discussion and Decision

Introduction

In this paper, we share our views on Rel-16 UE features for UE power saving. The discussions here are in reference to the latest draft in R1-2001484.

Corrections to existing UE features

We itemized our suggested corrections to the existing UE features as follows.

- FG 19-1:

- For component (1): it is not mentioned that minimum time gap is reported by the UE. Therefore, it is proposed to add ‘reported’ before the minimum time gap;

- For the note: The ‘DCI format 3_0’ should be corrected to ‘DCI format 2_6’ in the column of ‘Note.

- Add a new component: it is agreed and captured in the TS 38.213 (See following description) that if the UE does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle, the UE shall start the drx-onDurationTimer for the next DRX cycle. It is proposed to capture this as a new component.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle the UE shall start the drx-onDurationTimer for the next DRX cycle.

- FG 19-2:

- Add a new component: it is one important component of cross-slot scheduling based power saving to jointly indicate the scheduling offset restriction for minimumSchedulingOffset K0 and minimumSchedulingOffset K2. It is proposed to reflect the joint indication as a new component.

According to the above comments, the following revision is proposed for UE power saving.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 19-1:

  - For component (1): Add the wording of ‘reported’ before the ‘the minimum time gap’;

  - For the note:  The ‘DCI format 3_0’ should be corrected to ‘DCI format 2_6’ in the column of ‘Note.

  - Add a new component: “When the UE does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle, the UE shall start the drx-onDurationTimer for the next DRX cycle. ”

- FG 19-2:

- Add a new component: “Joint indication of the scheduling offset restriction for minimumSchedulingOffset K0 and minimumSchedulingOffset K2 by DCI format 0_1 and 1_1.”

Summary of proposals

The proposals in this paper are summarized as follows.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 19-1:

  - For component (1): Add the wording of ‘reported’ before the ‘the minimum time gap’;

  - For the note:  The ‘DCI format 3_0’ should be corrected to ‘DCI format 2_6’ in the column of ‘Note.

  - Add a new component: “When the UE does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle, the UE shall start the drx-onDurationTimer for the next DRX cycle. ”

- FG 19-2:

- Add a new component: “Joint indication of the scheduling offset restriction for minimumSchedulingOffset K0 and minimumSchedulingOffset K2 by DCI format 0_1 and 1_1.”