3GPP TSG-RAN WG1 Meeting #100bis	R1-2002618

# e-Meeting, April 24th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TBS determination when 3824<Ninfo<3825 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TBS determination, the Ninfo may be interpreted as a floating point or an integer value. This may lead to a different TBS determination when before quantization 3824<Ninfo<3825. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that Ninfo is not quantized and remove the misleading reference to number of bits. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ambiguity remains and some implementations may interpret Ninfo as a quantized value and while others implement the equation as specified without any quantization leading in some isolated cases to a different understanding of the correct TBS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: If the UE is implemented according to the CR and the gNB considers the Ninfo as a quantized value, or the gNB is implemented according to the CR and the UE considers the Ninfo as a quatized value: if the unquantized Ninfo calculation is in the range of 3824…3825, the TBS sizes are determined differently by the UE and the gNB and the packet is lost. This CR may be implemented by earlier release UEs/gNBs |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

#### 5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0, format 1_1 or format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1 or format 1_2 or as described for format 1_0 in Clause 5.1.6.2, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 0, 6, 12, or 18), the  is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI,  is assumed to be 0.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Unquantized intermediate variable (Ninfo) TBStemp) is obtained by TBStemp= NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-1 find the closest TBS that is not less than .

<Unchanged parts are omitted>