3GPP TSG-RAN WG1 Meeting #100bis		    				R1-2002625

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Discussion on Rel-16 UE power saving UE features

Agenda Item:	7.2.11.7

Document for:	Discussion and Decision

- Introduction

This contribution addresses our further comments  on Rel-16 power saving UE features. The discussion is based on the latest version R1-2001484[1].

- Discussion on UE Power saving features

19-1 DRX Adaptation

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |
| --- | --- | --- |

So far the feature includes (1) ‘Configured PS_offset for the detection of  DCI format 2_6  with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration’. However, the UE capability of minimum gap for each band can be different considering different realization. It can be caused by different RF paths to different bands. To solve this issue, we think the UE reported capability for (1) of 19-1 shall be per band.

For example, each band reports a SCS dependent minimum time gap, either a smaller gap (type 1) or a lager gap (type 2) in a per SCS basis, such that the network can decide how to configure the monitoring occasions for WUS, i.e., WUS is monitoring before DRX ON no later than the reported minimum time gap for the band. Meanwhile, when CA is configured, the network may choose to configure the WUS monitoring before DRX ON no later than any of the minimum time gap among configured CCs. As 60KHz SCS can be used for both FR1 and FR2, per band reporting allows UE to indicate different minimum gaps with 60KHz for bands in FR1 and FR2, respectively. It can provide better implementation flexibility for the UE.

Therefore,in the UE feature discussion, we propose to further decide 19-1 (1) is based on granularity of per band, in order to reflect the fact as mentioned above. For the other components, it is based on granularity of per UE

Proposal 1: for 19-1,

- Component (1) is based on granularity of per band.

- Component (2) ~ (5) is based on granularity of per UE.

19-3 Maximum MIMO Layer Adaptation

| 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |
| --- | --- | --- |

So far the feature 19-3 has not yet decided on its granularity. In Rel-16 power saving, this feature is designed to save UE power by reducing baseband processing, reducing RF path  and other implementation oriented  approach. Furthermore, maximum MIMO layer configuration is band-specific configuration.

Therefore, considering potential different RF implementations across different bands, we propose to confirm 19-3 is based on the granularity of per-band

Proposal 2: 19-3 is based on granularity of per band.

- Conclusion

This contribution addresses our further comments  on Rel-16 power saving UE features. And it is proposed that,

Proposal 1: for 19-1,

- Component (1) is based on granularity of per band.

- Component (2) ~ (5) is based on granularity of per UE.

Proposal 2: 19-3 is based on granularity of per band.

# Reference

- R1-2001484, 	RAN1 UE features list for Rel-16 NR after RAN1#100-E, 	AT&T, NTT DOCOMO, INC.

# Annex: UE feature for Rel-16 power saving

# 12. UE Power Saving

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | - Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset K2 configuration for PUSCH |  |  |  |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |  |  |  |  | N/A |  | [Per BC] | No | No |  | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication | Optional with capability signalling |
|  | 19-4a | UE assistance information | - Preferred k0/k2 [for same carrier scheduling] - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  |  |  |  | N/A |  | Per UE | No | No |  | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted | Optional with capability signalling |