3GPP TSG RAN WG1 Meeting #100bis-e	R1-200xxxx

# E-meeting, April 20 – April 30, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUSCH with a Type 1 configured grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Clause 6.1 in TS 38.214, it is specified that the higher layer parameter pathlossReferenceIndex provided in rrc-ConfiguredUplinkGrant for initial PUSCH transmission with a Type 1 configured grant is also applied for PUSCH retransmission scheduled by DCI with CRC scramble by CS-RNTI and NDI set to 1. However, the following spec in Clause 7.1.1 in TS 38.213 only captures the case for initial PUSCH transmission with a Type 1 configured grant but misses the case for retransmission: - For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig , a RS resource index is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking Therefore, specification change is needed for TS 38.213 to complete the power control related procedure for PUSCH retransmission corresponding to a Type 1 configured grant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “a PUSCH transmission” to “a PUSCH (re)transmission” to capture both initial transmission and retransmission corresponding to a Type 1 configured grant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification on power control related procedure for PUSCH retransmission correpsponding to a Type 1 configured grant is incomplete, and the specifications are not aligned between TS 38.213 and TS 38.214. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR impacts PUSCH transmission with a Type 1 configured grant. According to Clause 6.1 of TS 38.214, both UE and gNB know the higher layer parameter pathlossReferenceIndex is also used for power control of PUSCH retransmission corresponding to a Type 1 configured grant. However, the corresponding UE behavior is not specified in TS 38.213, which makes the power control related procedure for Type 1 configured grant incomplete and also leads to alignment issue between TS 38.213 and TS 38.214. |  |  |  |  |  |  |  |  |

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP  of carrier  of serving cell  using parameter set configuration with index  and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power  in PUSCH transmission occasion  as

[dBm]

where,

< Unchanged parts are omitted >

-	is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index  for the active DL BWP, as described in Subclause 12, of carrier  of serving cell

-	If the UE is not provided PUSCH-PathlossReferenceRS or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource from the SS/PBCH block that the UE uses to obtain MIB

< Unchanged parts are omitted >

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, the UE determines a RS resource index  from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission. If the DCI format activating the PUSCH transmission does not include a SRI field, the UE determines a RS resource index  with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

< Unchanged parts are omitted >