3GPP TSG RAN WG1 Meeting #100-e-bis   	          	     	R1-2002677

E-meeting, April 20th – April 30th, 2020

Title:	[Draft] Reply LS on UL-SL prioritization

Response to:	R2-1916468

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Huawei [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Yangfan

#### E-mail Address:	james.yangfan@huawei.com

Attachments: 	    None

1. Overall Description:

RAN1 would like to thank RAN2 for the reply LS on UL-SL prioritization.

According to the reply LS in R2-1916468, RAN1 identified a number of remaining issues for handling UL-SL prioritization.

- PUCCH or PUSCH transmission without SL HARQ-ACK vs SL transmission

A priority index of PUCCH or PUSCH transmission without SL HARQ-ACK is indicated by the Priority indicator field in the associated DCI or associated configured grant. In this case, UL transmission is prioritized if the value of priority index of the PUCCH or PUSCH as indicated by the Priority indicator field in the associated DCI or provided by the associated configured grant is 1. Otherwise, LTE-V solution is used.

- SL HARQ-ACK on PUCCH or PUSCH without UL-SCH vs SL transmission

It is based on direct comparison between priority of SL HARQ-ACK on PUCCH or PUSCH without UL-SCH and SL transmission, where the priority of SL HARQ-ACK on PUCCH or PUSCH without UL-SCH is same as corresponding SL-SCH.

- SL HARQ-ACK on PUSCH with UL-SCH vs SL transmission

In this case, SL transmission is prioritized when following conditions are met：

- SL transmission is prioritized over UL-SCH according to prioritization rule between NR UL data and NR SL data transmission; and

- The priority of SL transmission is higher than that of SL HARQ-ACK on PUSCH.

- SRS vs SL transmission

RAN1 would like to reuse LTE-V solution for this case.

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above into account in the related work.

3. Date of Next TSG-RAN1 Meetings:

3GPP RAN1#101e	25th May – 5th June 2020   	e-Meeting