3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002977

Online, April 13th-30th, 2020

Agenda Item:	7.2.6.5

Source:	Moderator – (Ericsson)

Title:	Text proposals for issue #1 of low PAPR RS

Document for:	Decision

# 1	Introduction

Based on the email discussion on low PAPR issue #1, these text proposals, identified as TP#0 and TP#4 in the discussion, were endorsed.

# 2 Text proposal for TS 38.211:

Reason for change:  The Rel.16 DMRS for low PAPR shall only be used when pi/2 BPSK is used for PUCCH, and not when QPSK is used.

Summary of change:  Clarify the description to indicate that this is for pi/2-BPSK only

Consequences if not approved:  There is a risk that it is interpreted that he low-PAPR DMRS is used also for QPSK modulation based PUCCH.

Clauses affected:   Section 6.4.1.3.3.1

-----------------------------------------------------Start of text proposal---------------------------------------------------------

###### 6.4.1.3.3.1	Sequence generation

< Unchanged parts are omitted >

-	if the higher-layer parameter dmrsUplinkTransformPrecodingPUCCH-r16 is configured, and π/2-BPSK modulation is used for PUCCH,  ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2.

< Unchanged parts are omitted >

-------------------------------------------------------End of text proposal---------------------------------------------------------

# 3 Text proposal for TS 38.211:

Reason for change:  The current specification indicates that cyclic shift hopping for Rel.16 DMRS is used for PUCCH Format 3 and 4.  However, this is incorrect since the reference signal sequence in this case does not depend on the cyclic shift alpha.

Summary of change:  Remove redundant text and indicate that alpha=0 (always) in this case. Re-arrange the text so it’s clear the cyclic shift description only applies to Rel.15 behaviour of PUCCH DMRS.

Consequences if not approved:  Risk of confusion. Incomplete specifications.

Clauses affected:   Section 6.4.1.3.3.1

-----------------------------------------------------Start of text proposal---------------------------------------------------------

< Unchanged parts are omitted >

##### 6.4.1.3.3	Demodulation reference signal for PUCCH formats 3 and 4

###### 6.4.1.3.3.1	Sequence generation

The reference-signal sequence rlm shall be generated according to

where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration:

-	if the higher-layer parameter dmrsUplinkTransformPrecodingPUCCH-r16 is configured, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α=0 depends on the cyclic shift hopping in clause 6.3.2.2.2.

-	otherwise, ru,vα,δ(m) is given by clause 6.3.2.2. The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

< Unchanged parts are omitted >

-------------------------------------------------------End of text proposal---------------------------------------------------------