3GPP TSG RAN WG1 Meeting #100b-e

e-Meeting, April 20th – 30th, 2020

Title:	Session notes for 7.2.6 (Maintenance of Enhancements on MIMO for NR)

Source:	Ad-hoc chair (Samsung)

### Maintenance of Enhancements on MIMO for NR

R1-2002648	Introduction of NR ATF measurements	Keysight Technologies UK Ltd

R1-2002649	Correction on SS-RSRPB measurement	Keysight Technologies UK Ltd

[100b-e-NR-38.215-RSRPB-ATF] Email discussion/approval of 38.213 CRs based on the following contributions

- R1-2002648 (agenda item 7.2.6)

- R1-2002649 (agenda item 7.2.6)

- R1-2002639 (agenda item 7.2)

- R1-2002231 (agenda item 7.1)

till 4/23 (Javier, Keysight)

Agreement

The text proposal in R1-2002231 is endorsed in R1-2002831 (TS38.215, Rel-15, CR#0022, Cat F) and R1-2002832 (TS38.215, Rel-16, CR#0023, Cat A).

Agreement

The extension of SS-RSRPB to FR1 and introduction of SS-RSARP are endorsed in R1-2002779 (TS38.215, Rel-16, CR#0021rev1, Cat B)

- Note: This CR is using a provisional WI code (NR_OTA_UEmeas) for a WI that is planned to be approved at RAN #88-e along with this CR.

#### CSI Enhancement for MU-MIMO Support

R1-2002138	Feature lead summary for MU-MIMO CSI	Moderator (Samsung)

R1-2001562	Remaining issues on MU-CSI in R16	Huawei, HiSilicon

R1-2001595	Maintenance of CSI enhancement for MU-MIMO	ZTE

R1-2001677	Discussion on remaining issues on MU CSI	vivo

R1-2001725	Text proposals for CSI enhancements for MU-MIMO support	OPPO

R1-2001912	Correction on enhanced Type II port selection codebook	LG Electronics

R1-2002089	Remaining issues on CSI Enhancement for MU-MIMO	CATT

R1-2002136	On maintenance of Rel.16 MU CSI enhancements	Samsung

R1-2002137	Summary of offline email discussion on [100e-NR-eMIMO-MUCSI-01] follow up	Moderator (Samsung)

R1-2002293	Maintenance on Rel-16 CSI enhancements	Nokia, Nokia Shanghai Bell

R1-2002336	Remaining issues for Rel-16 Type II CSI enhancement	Apple

R1-2002410	Remaining issues on MU-CSI Enhancements	Motorola Mobility, Lenovo

R1-2002550	Remaining issues on CSI enhancement for MU-MIMO support	Qualcomm Incorporated

R1-2002717	Feature lead summary#2 for MU-MIMO CSI	Moderator (Samsung)

R1-2002757	TP for eMIMO MU-CSI topic #2: correction on PMI subband size and basis subset selection	Moderator (Samsung)

[100b-e-NR-eMIMO-MUCSI-01] Proposal for agreement and conclusion with reference to issue H.0 (as described in R1-2002717) by 4/24 – Eko (Samsung)

Conclusion

On the support of CSI reporting for BWP size < 24 PRBs, there is no consensus in supporting Rel.15 Type-I multi-panel, Rel.15 Type-II, and Rel.16 eType-II codebooks for wideband CSI reporting

[100b-e-NR-eMIMO-MUCSI-02] The combined TP in section 3 of R1-2002717 from issues H.1, H.2, E.1, E.2, and E.3 for endorsement by 4/24 – Eko (Samsung)

Agreement

The text proposal in R1-2002757 is endorsed for TS38.214 (section 5.2.2.2.5). To be included as part of the editor’s CR for TS38.214.

#### Enhancements on Multi-TRP/Panel Transmission

R1-2002406	FL summary for Multi-TRP/Panel Transmission 	Moderator (OPPO)

R1-2001563	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

R1-2001596	Maintenance of multi-TRP enhancements	ZTE

R1-2001678	Discussion on remaining issues on multi TRP operation	vivo

R1-2001726	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

R1-2001822	Enhancements on Multi-TRP/Panel Transmission	MediaTek Inc.

R1-2001913	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

R1-2001971	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

R1-2002004	Corrections to multi-TRP	Intel Corporation

R1-2002052	TP on Multi-TRP/Panel Transmission	Futurewei

R1-2002090	Remaining issues on multi-TRP/panel transmission	CATT

R1-2002139	On Rel.16 multi-TRP/panel transmission	Samsung

R1-2002212	Remaining issues on multi-TRP/panel transmission	CMCC

R1-2002270	Discussion on remaining issues for multi-TRP operation	Spreadtrum Communications

R1-2002294	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

R1-2002337	Remaining issues for Multi-TRP enhancement	Apple

R1-2002364	Remaining issues on multi-TRP transmission	NEC

R1-2002378	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

R1-2002448	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

R1-2002472	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

R1-2002551	Multi-TRP Enhancements	Qualcomm Incorporated

R1-2002730	FL summary #2 for Multi-TRP/Panel Transmission	Moderator (OPPO)

[100b-e-NR-eMIMO-multiTRP-01] Email discussions to handle the following text proposals and editorial changes (details in R1-2002730) by 4/24 – Li (OPPO)

- TP #c-1 correcting one parameter typo of PDCCH monitoring in TS 38.213

- TP #c-2 Update the TCI-state activation in 38.214 according to RAN2 MAC CE design

- TP #c-3 Clarify DMRS table and entry {0,2,3} in TS 38.214

- TP #c-5 Correct the description on DAI bits in DCI format 1_2.

- TP #c-10 update one term in Section 5.1.5 of TS 38.214

- #a-2: To determine the default value of R used in PDCCH blind detection

Agreement

The TPs in R1-2002825, R1-2002826, R1-2002827 are endorsed for editor’s CRs on TS38.212, TS38.213, TS38.214, respectively.

[100b-e-NR-eMIMO-multiTRP-02] Email discussion to handle the issues related with multi-DCI based M-TRP (details in R1-2002730) by 4/24 and corresponding TP (if any) by 4/30 – Li (OPPO)

- #a-3: Clarify the CRS rate match behavior for multi-DCI based M-TRP

- #a-4: Define PDCCH monitoring occasion for Type-2 HARQ-ACK codebook in Multi-DCI M-TRP

- #a-6: Determine the default QCL for AP CSI-RS in multi-DCI based M-TRP

Agreement

- Add a new RRC parameter [crs-RateMatch-PerCORESETPoolIndex] that indicates the UE does rate match PDSCH around configured CRS for each TRP

- Adopt the following TP in R1-2002917 for editor’s CR on TS 38.214:

< Start of the text proposal > 5.1.4.2 PDSCH resource mapping with RE level granularity *** Unchanged text is omitted *** If the UE is configured by higher layer parameter PDCCH-Config with two different values of CORESETPoolIndex in ControlResourceSet and is also configured by the higher layer parameter LTE-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - If the UE is configured with [crs-RateMatch-PerCORESETPoolIndex], REs indicated by the CRS pattern(s) in lte-CRS-PatternList-r16 if the PDSCH is associated with CORESETPoolIndex = 0, or the CRS pattern(s) in lte-CRS-PatternListSecond-r16 if the PDSCH is associated with CORESETPoolIndex = 1CRSPatternList-CORESETPoolIndex for a UE supporting the capability of [separate-lte-CRS-ToMatchAround]; - Otherwise, REs indicated by CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig for a UE not supporting the capability of [separate-lte-CRS-ToMatchAround]. < End of the text proposal >

Agreement

Adopt the following TP in R1-2002917 for editor’s CR on TS 38.213.

< Start of the text proposal > 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel *** Unchanged text is omitted *** The set of PDCCH monitoring occasions for a DCI format scheduling PDSCH receptions or SPS PDSCH release is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells. PDCCH monitoring occasions are first indexed in an ascending order across serving cells indexes for a same start time of search space sets associated with DCI formats scheduling PDSCH receptions or SPS PDSCH release on the serving cells, and are then indexed in an ascending order of start times of the search space sets associated with a PDCCH monitoring occasion. For indexing within a serving cell for a same start time of search space sets, if the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and is provided ACKNACKFeedbackMode = JointFeedback for the active UL BWP, PDCCH monitoring occasions for the first CORESETs are indexed prior to PDCCH monitoring occasions for the second CORESETs.The cardinality of the set of PDCCH monitoring occasions defines a total number of PDCCH monitoring occasions. < End of the text proposal >

Agreement

Adopt the following TP in R1-2002917 for editor’s CR on TS 38.213:

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with the DCI formats is present up to the current serving cell and current PDCCH monitoring occasion, first in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index , where . If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the value of counter DAI is in the order of the first CORSETs and then the second CORESETs for the same serving cell index and same PDCCH monitoring occasion index. The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI formats is present, up to the current PDCCH monitoring occasion and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the {serving cell, PDCCH monitoring occasion}-pair(s) is counted for both the first CORESETs and the second CORESETs in the total DAI.

Agreement

Adopt the following TP in R1-2002917 for editor’s CR on TS 38.213:

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release on an active DL BWP of a serving cell , as described in Clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot based on - PDSCH-to-HARQ_feedback timing indicator field values for PUCCH transmission with HARQ-ACK information in slot in response to PDSCH receptions or SPS PDSCH release - slot offsets [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions or SPS PDSCH release and by pdsch-AggregationFactor or RepNumR16, when provided. --- end of TP ---

[100b-e-NR-eMIMO-multiTRP-03] Email discussion to handle the issues related with single-DCI based M-TRP by 4/24 and corresponding TP (if any) by 4/30 – Li (OPPO)

- #b-1: Determine default QCL for AP CSI-RS in single-DCI based M-TRP

- #b-8: Specify mapping type for PDSCH in scheme 3

- #b-9: PT-RS transmission in single-DCI based M-TRP

- #b-2: Default TCI-state for PDSCH of Scheme 3 and Scheme 4

Agreement

The following TP in R1-2002947 on PT-RS power boosting is endorsed for the editor’s CR on TS 38.214.

4.1 Power allocation for downlink ---- Unchanged text are omitted ---- When the UE is scheduled with a one or two PT-RS ports associated with the PDSCH, - if the UE is configured with the higher layer parameter epre-Ratio, the ratio of PT-RS EPRE to PDSCH EPRE per layer per RE for each PT-RS port () is given by Table 4.1-2 according to the epre-Ratio, the PT-RS scaling factor specified in clause 7.4.1.2.2 of [4, TS 38.211] is given by. - otherwise, the UE shall assume epre-Ratio is set to state '0' in Table 4.1-2 if not configured. Table 4.1-2: PT-RS EPRE to PDSCH EPRE per layer per RE () epre-Ratio; The number of PDSCH layers with DM-RS associated to the PT-RS port. 1; 2; 3; 4; 5; 6. 0; 0; 3; 4.77; 6; 7; 7.78. 1; 0; 0; 0; 0; 0; 0. 2; reserved. 3; reserved. ---- Unchanged text are omitted ----

Agreement

For the mapping type in Scheme 3, all PDSCH transmission occasions are mapping type B, UE does not expect the TDRA field of the DCI to indicate mapping type A.

Possible Agreement

The following TP in R1-200XXXX is endorsed for the editor’s CR on TS38.214.

5.1.2.1 Resource allocation in time domain ---- Unchanged text are omitted ---- When a UE is configured by the higher layer parameter RepSchemeEnabler set to 'TDMSchemeA' and indicated DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. - If two TCI states are indicated by the DCI field 'Transmission Configuration Indication', the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after symbols from the last symbol of the first PDSCH transmission occasion. If the value is not configured via the higher layer parameter StartingSymbolOffsetK, = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where applied respectively to the first and second TCI state. UE expects the PDSCH mapping type indicated by DCI field “Time domain resource assignment’ to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1. ---- Unchanged text are omitted ----

#### Enhancements on Multi-beam Operation

R1-2001915	FL summary#1 on Rel-16 MB1 maintenance	Moderator (LG Electronics)

R1-2002339	Feature Lead Summary #1 on L1-SINR and SCell BFR	Apple

R1-2002340	Feature Lead Summary #2 on L1-SINR and SCell BFR	Apple

R1-2001564	Remaining issues on multi-beam enhancements in R16	Huawei, HiSilicon

R1-2001597	Maintenance of multi-beam operation	ZTE

R1-2001679	Discussion on remaining issues on multi beam operation	vivo

R1-2001727	Remaining issues on Multi-beam Operation Enhancement	OPPO

R1-2001818	Remaining issues on multi-beam operation	Sony

R1-2001823	Remaining issues on multi-beam operation	MediaTek Inc.

R1-2001914	Remaining issues on multi beam operation	LG Electronics

R1-2001972	Remaining issues on Scell beam failure recovery	Lenovo, Motorola Mobility

R1-2002005	 Corrections to Multi-beam Operation 	Intel Corporation

R1-2002091	On Rel.16 support for multi-beam operation	CATT

R1-2002140	On Rel.16 support for multibeam operation	Samsung

R1-2002213	Remaining issues on multi-beam operation	CMCC

R1-2002271	Discussion on remaining issues on multi-beam operation	Spreadtrum Communications

R1-2002283	Enhancements on multi-beam operation	Fraunhofer IIS, Fraunhofer HHI

R1-2002295	Maintenance of Rel-16 Beam Management	Nokia, Nokia Shanghai Bell

R1-2002338	Remaining issues on beam management enhancement	Apple

R1-2002449	Remaining issues on multi-beam operation	NTT DOCOMO, INC

R1-2002498	On the timing for pathloss RS activation	Ericsson, Nokia

R1-2002552	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2002725	FL summary#2 on Rel-16 MB1 maintenance		Moderator (LG Electronics)

[100b-e-NR-eMIMO-MB1-01] Email discussion on whether/how to capture RAN1#100e agreements (Issue#1 in R1-2002725) by 4/24 – Jiwon (LG Electronics)

Agreement

The TP in R1-2002843 is endorsed for the editor’s CR on TS38.213.

Agreement

Send an LS to RAN4 to convey that RAN1 does not plan on specifying UE behavior when the PL-RS is not being maintained for the following cases

- Explicit MAC CE based PL-RS update

- Default PL-RS updated by MAC CE

The LS is endorsed in R1-200XXXX

[100b-e-NR-eMIMO-MB1-02] Email discussion on PL RS tracking behavior when default PL RS is enabled (Issue#2 in R1-2002725) by 4/24 and corresponding TP (if any) by 4/30 – Jiwon (LG Electronics)

Possible Agreement

The following TP in R1-200XXXX is endorsed for editor’s CR on TS38.213.

7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by IE SRS-Positioning-Config as described in Clause 7.3.1. If the number of RS resources configured by RRC for pathloss estimation for PUCCH, PUSCH and SRS is greater than 4, among those RS resources configured for the uplink channels and signals where the MAC-CE based activation of the RS resources for pathloss estimation is applicable as described in Clause 7.1.1, 7.2.1 and 7.3.1, UE is not required to track the RS resources which are not activated by MAC-CE.

--------------- Unchanged parts omitted -------------

[100b-e-NR-eMIMO-MB1-03] Email discussion on corrections for clarification (Issue#13, Issue#14, Issue#15, Issue#3, Issue#7 in R1-2002725) by 4/24 – Jiwon (LG Electronics)

- NOTE: Only proposals that does not require additional RAN1 agreements will only be discussed. In this regard, for Issue#7, only TP for Alt2 will be discussed.

Agreement

The TPs in R1-2002845 are endorsed for the editor’s CRs on TS38.213, TS38.214.

Conclusion

When the spatial relation for AP-SRS is updated by MAC-CE and the HARQ/ACK corresponding to the MAC-CE is transmitted in slot n, the updated spatial relation shall be active starting from slot n+3ms.

- The active spatial relation at the slot of AP SRS transmission is applied for the AP SRS transmission

[100b-e-NR-eMIMO-MB2-01] Email discussion on corrections proposed in section 3.1 of R1-2002340 by 4/24 – Yushu (Apple)

Agreement

The TPs in R1-2002823 are endorsed for the editor’s CRs on TS38.215, TS38.214.

[100b-e-NR-eMIMO-MB2-02] Email discussion on corrections proposed in section 3.5 of R1-2002340 by 4/24 – Yushu (Apple)

- Note: Check to make sure there are no overlapping/contradicting discussions between this email thread and those going on under NR_UE_pow_sav-WUS

Above email was canceled since the discussion was merged into [100b-e-NR-UE_pow_sav-WUS-02] Email discussion/approval on RAN1 and RAN2 alignment in section 1.2 and subsections in R1-2002698 till 4/24, with potential TP for endorsement till 4/29 (CATT, Fangchen)

[100b-e-NR-eMIMO-MB2-03] Email discussion on corrections proposed in section 4.1 of R1-2002340 by 4/24 – Yushu (Apple)

Agreement

The TPs in R1-2002824 are endorsed for the editor’s CR on TS38.213.

#### Full TX Power UL transmission

R1-2001719	Feature lead summary on ULFPTx	Moderator (vivo)

R1-2001565	Remaining issues on UL full power transmission in R16	Huawei, HiSilicon

R1-2001598	Maintenance of full power UL transmission	ZTE

R1-2001680	Discussion on remaining issues on UL full power transmission	vivo

R1-2001728	Text proposals for full TX power UL transmission	OPPO

R1-2001824	Remaining issues on UL full power transmission	MediaTek Inc.

R1-2001916	Text proposals on full Tx power UL transmission	LG Electronics

R1-2002006	Correction to Full Tx Power UL Transmission	Intel Corporation

R1-2002092	Remaining issues on UL full power transmission	CATT

R1-2002141	On UL full power transmission	Samsung

R1-2002214	Discussion on full TX power UL transmission	CMCC

R1-2002272	Discussion on remaining issues on full TX power for UL transmission	Spreadtrum Communications

R1-2002296	Maintenance of Rel-16 Full TX Power UL transmission	Nokia, Nokia Shanghai Bell

R1-2002341	Remaining issues for UL Full Power transmission enhancement	Apple

R1-2002368	Corrections for Full Power UL Transmission	Ericsson

R1-2002450	Remaining issue on full Tx power UL transmission	NTT DOCOMO, INC

R1-2002553	Remaining issues on full power UL transmission	Qualcomm Incorporated

R1-2002746	Summary of prep email discussion on ULFPTx	Moderator (vivo)

[100b-e-NR-eMIMO-ULFPTx-01] Email discussion on Issue #2 in R1-2002746: For 4Tx and partial-coherent UE in Mode 2, when the SRS resource set is configured with 2 ports SRS and 4 ports SRS, and the codebook subset associated with 2 port SRS resource. By 4/24 and corresponding TP (if any) by 4/30 – Rakesh (vivo)

Agreement

For a 4Tx UE configured with higher layer parameter codebookSubset set to partialAndNonCoherent in Mode 2, when the SRS resource set is configured with 2 ports SRS and 4 ports SRS, and the codebook subset associated with 2 port SRS resource:

- the codebook subset is nonCoherent

Note that this agreement does not introduce new RRC parameters.

Agreement

The following TP in R1-200XXXX is endorsed for the editor’s CR on TS328.214.

For codebook based transmission, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 and codebookSubset-ForDCIFormat0_2 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability. When higher layer parameter ul-FullPowerTransmission is set to ‘fullpowerMode2' and the higher layer parameter codebookSubset or the higher layer parameter codebookSubset-ForDCIFormat0_2 is set to 'partialAndNonCoherent', and when the SRS-resourceSet with usage set to “codebook” includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2 port SRS is ‘nonCoherent’. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 and maxRank-ForDCIFormat0_2 for PUSCH scheduled with DCI format 0_2.

[100b-e-NR-eMIMO-ULFPTx-02] Email discussion on Issue #3 in R1-2002746: UE feature/capability related issues. By 4/24 and corresponding TP (if any) by 4/30– Rakesh (vivo)

Conclusion

- A full coherent capable UE, depending on its capability, can also report Rel-16 UL full power transmission capability

- Discuss UE capability related issues under UE feature agenda

[100b-e-NR-eMIMO-ULFPTx-03] Email discussion on Issue #4 in R1-2002746: TP for power scaling for Mode2. By 4/24 and corresponding TP (if any) by 4/30 – Rakesh (vivo)

Discussion on TP until 4/30

#### Low PAPR RS

R1-2001566	Discussion on low PAPR sequence in R16	Huawei, HiSilicon

R1-2001599	Maintenance of low PAPR RS	ZTE

R1-2001681	Discussion on remaining issues on low PAPR RS	vivo

R1-2001729	Text proposals for low PAPR RS	OPPO

R1-2001812	Corrections in low PAPR RS for pi/2 BPSK PUSCH	Indian Institute of Tech (H)

Withdrawn

R1-2001825	Maintenance of low PAPR RS	MediaTek Inc.

R1-2001908	Remaining issues on low PAPR RS	Ericsson

R1-2002007	Low PAPR RS	Intel Corporation

R1-2002273	Discussion on remaining issues on Low PAPR DMRS sequence	Spreadtrum Communications

R1-2002297	Maintenance on Rel-16 Low PAPR RS	Nokia, Nokia Shanghai Bell

R1-2002554	Remaining issues on low PAPR RS	Qualcomm Incorporated

R1-2002726	Feature lead summary #2 of low PAPR RS	Moderator (Ericsson)

[100b-e-NR-eMIMO-lowPAPR-01] Email discussion on Issue #1 in R1-2002693: For PUCCH format 3 and 4, the used sequence does not depend on the cyclic shift α. The specification text in 38.211 does however describe the cyclic shift of the sequence, which is undefined. The open issue is whether to remove the text describing the dependence or alternatively define how cyclic shifts of the used sequence shall be applied. By 4/24 – Mattias (Ericsson).

Agreement

Remove specification text related to cyclic shifts for low PARP RSs. Text proposals in R1-200XXXX are endorsed for the editor’s CRs (TP#0 and TP#4 in the feature lead summary).

[100b-e-NR-eMIMO-lowPAPR-02] Email discussion on Issue #2 in R1-2002693: It should be clarified by RAN1 if the agreed π/2-BPSK DMRS in Rel-16 (when configured) is used for PUSCH scheduled by DCI 0_2 or not. In current specification it is not used for DCI format 0_2 based PUSCH and the proposal is to introduce it for DCI 0_2. By 4/24 – Mattias (Ericsson).

Agreement

The text proposal in R1-2002791 is approved for the editor’s CRs for TS38.211 and TS38.212.

#### Others

R1-2001730	Remaining issues on multi-TRP and panel Transmission	OPPO

R1-2001917	TP for the maximum number of CSI-RS resources for BM	LG Electronics

R1-2001918	Critical MIMO issue from observations in the field	Ericsson

R1-2002342	On further MIMO enhancement	Apple

R1-2002467	On limitations of PDSCH reliable transmission	vivo

R1-2002468	On limitations of PDSCH reliable transmission	vivo

Withdrawn

R1-2002469	On limitations of PDSCH reliable transmission	vivo

Withdrawn

R1-2002470	On limitations of PDSCH reliable transmission	vivo

Withdrawn

R1-2002471	On limitations of PDSCH reliable transmission	vivo

Withdrawn

R1-2002667	Other open issues for Rel-16 MIMO	Huawei, HiSilicon