3GPP TSG RAN WG1 Meeting #101-e	                                                                               R1-2003298

E-meeting, 25 May – 5 June, 2020

Agenda Item:	8.4.1.1

Source:	Huawei, HiSilicon

Title:	Baseline coverage performance for FR1

Document for:	Discussion and Decision

Introduction

In RANP#86 meeting, a new study item, i.e., study on NR coverage enhancement was approved for Release 17 [1]. The SI focuses on identifying target scenarios, services and channels for coverage enhancement and studying potential solutions for increasing coverage, some detailed objectives on baseline coverage performance for FR1 are copied as follows.

- The target scenarios and services include - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1 - Indoor scenario (indoor gNB serving indoor UEs) - TDD and FDD for FR1. - VoIP and eMBB service for FR1. - Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

In this contribution, we discuss the baseline coverage performance for FR1, including evaluation methodology, parameter settings and link level simulation performance.

Discussions

Evaluation methodology and revised parameters

For the three typical scenarios including urban, rural, rural with long distance, target data rates are specified as follows:

- Urban: 1Mbps for UL eMBB service, 12.2Kbps for UL VoIP service

- Rural: 100kbps for UL eMBB service, 12.2Kbps for UL VoIP service

- Rural with long distance: same as rural case

To identify the coverage bottleneck, the link budget evaluation is necessary and the methodology which was adopted in IMT-2020 self-evaluation for 3GPP submission [3] could be a start. The detailed evaluation procedure for available path loss and corresponding maximum coverage range, i.e., parameter setting and calculation formula, are cited as in the following table.

Table 1. Link budget template

| Parameter | Values |
| --- | --- |
| Scenario |  |
| Frame structure |  |
| Carrier frequency (Hz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability for control channel |  |
| Cell area reliability for data channel |  |
| Transmission bit rate for control channel (bit/s) |  |
| Transmission bit rate for data channel (bit/s) |  |
| Target packet error rate for the required SNR in item (19a) for control channel |  |
| Target packet error rate for the required SNR in item (19b) for data channel |  |
| Spectral efficiency (bit/s/Hz) |  |
| Pathloss model (select from LoS or NLoS) |  |
| UE speed (km/h) |  |
| Feeder loss (dB) |  |
| Transmitter |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (2) Maximal transmit power per antenna (dBm) |  |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) |  |
| (4) Transmitter antenna gain (dBi) |  |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) |  |
| (6) Control channel power boosting gain (dB) |  |
| (7) Data channel power loss due to pilot/control boosting (dB) |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm |  |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (11) Receiver antenna gain (dBi) |  |
| (11bis) Receiver array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, etc.) (dB) |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15a) Receiver interference density for control channel (dBm/Hz) |  |
| (15b) Receiver interference density for data channel (dBm/Hz) |  |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz |  |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz |  |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) |  |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) |  |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm |  |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm |  |
| (19a) Required SNR for the control channel (dB) |  |
| (19b) Required SNR for the data channel (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21a) H-ARQ gain for control channel (dB) |  |
| (21b) H-ARQ gain for data channel (dB) |  |
| (22a) Receiver sensitivity for control channel = (18a) ++ (19a) + (20) – (21a) dBm |  |
| (22b) Receiver sensitivity for data channel = (18b) ++ (19b) + (20) – (21b) dBm |  |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11bis) − (22a) dB |  |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11bis) − (22b) dB |  |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) |  |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) |  |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) – (12) dB |  |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) – (12) dB |  |
| Range/coverage efficiency calculation |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) |  |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) |  |

Proposal 1: Reuse the evaluation methodology of link budget in IMT-2020 self-evaluation for Rel-17 NR coverage enhancement at FR1.

Since IMT-2020 self-evaluation has already determined a full set of simulation parameters, we feel that the most convenient way is following the simulation assumptions in [3] with general parameters for FR1. Meanwhile, regarding differences in simulation cases, we need to revise some parameters, such as shadow fading margin and penetration margin, etc. Considering this early stage for evaluation, we reuse channel mode A as a start.

Following, we provide the general parameters and revised parameters, respectively

Table 2. Key general parameters for coverage enhancement evaluation

|  | Urban | Rural | Rural with long distance |
| --- | --- | --- | --- |
| System configuration | 4GHz TDD | 4GHz TDD/2GHz FDD | 700MHz FDD |
| SCS | 30kHz for TDD/ 15kHz for FDD |  |  |
| Physical channels | PUSCH/PUCCH/PDSCH/PDCCH/Msg3 |  |  |
| Channel model | TDL-C | TDL-C | TDL-E |
| DS | 300 ns |  |  |
| UE speed | 3 km/h | 120 km/h for outdoor UE 3 km/h for indoor UE | 120 km/h |
| Number of BS antennas & antenna configuration | 192* (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) |
| Number of TXRU for BS | 2 | 2 | 2 |
| Number of UE antennas | 2 | 2 | 1 |
| Number of TRXU for UE | 2 | 2 | 1 |
| Overhead assumption for required SNR simulation | DMRS: Type I, one DMRS symbol, no multiplexing with data | DMRS: - For 3km/h: Type I, one DMRS symbol, no multiplexing with data. - For 120km/h: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data | DMRS: For 120km/h: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |

*Note that: in ITU-2020 self-evaluation, 128 antennas at BS is used for urban scenario. However, in current commercial deployment, BS with 192 antennas are widely used.

Table 3. The revised parameters for coverage enhancement evaluation

| Parameters | Urban TDD | Rural NLoS TDD | Rural NLoS FDD | Rural with long distance FDD |
| --- | --- | --- | --- | --- |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) | 360000 (1*12*30*1000) | 360000 (1*12*30*1000) | 180000 (1*12*15*1000) | 180000 (1*12*15*1000) |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) | 10800000 eMBB (30*12*30*1000) 1440000 VoIP (4*12*30*1000) | 1440000 (4*12*30*1000) | 720000 (4*12*15*1000) | 720000 (4*12*15*1000) |
| Spectral efficiency(2) (bit/s/Hz) | 0.27 eMBB | 0.2 eMBB | 0.14 eMBB | 0.14 eMBB |
| (24) Lognormal shadow fading std deviation (dB) | 7(NLoS) | 8 (NLoS) | 8 (NLoS) | 6 (LoS) |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) | 7.56 | 10.45(O2O) 8.45(O2I) | 10.45(O2O) 8.45(O2I) | 6 |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) | 4.48 | 6.61(O2O) 5.13 (O2I) | 6.61(O2O) 5.13 (O2I) | 4.79 |
| (27) Penetration margin (dB) | 26.25 | 9(O2O) 14.53 (O2I) | 9(O2O) 12.5 (O2I) | 9 |

Notes for above specific values of revised parameters in coverage evaluation is provided as below:

On the determination of the number of PRB

Taking the eMBB service under urban scenario as an example, 1Mbps UL target data rate, 30 kHz SCS in TDD duplex mode with 7:3 DL-UL slot configuration, the time duration of a frame is 5 millisecond, hence, the minimum transmission block size requirement under 10% BLER for a half-frame could be calculated as:

1×106 bps0.9×5×10-3s≈5555 bits

For UEs at cell edge, a small index MCS is used to ensure the coverage, e.g., QPSK modulation with coding rate 251/1024, and one single symbol DMRS is configured within each slot, thus number of bits transmitted for each UL slot per PRB is

12×14-1×2×2511024≈76.5 bits

Given 7:3 DL-UL slot configuration, the number of bits per PRB in UL transmission in each half-frame is (76.5*3) bits

Hence, the minimum number of scheduled PRBs should be:

555576.5*3≈25,

Thus, 30 PRBs are scheduled to meet the above requirement. Following the similar principle, number of scheduled PRBs towards other services and scenarios are provided as well in Table 3.

On the determination of the DMRS symbol number

Given 3 km/h UE speed under outdoor to indoor scenario, one DMRS symbol is able to satisfy the requirement for channel estimation. When UE speed reaches 120km/h under outdoor to outdoor scenario, additional DMRS symbol should be configured to combat the large Doppler frequency shifts.

On the determination of margins

According to TS 38.901, we could find the recommended value and calculation formula for the aforementioned parameters such as shadow fading margin, penetration margin, respectively.

More detailed analysis for parameter determinations can be found in the Appendix.

Proposal 2: Adopt the revised values in Table 3 to the link budget template for the baseline link budget evaluation of Rel-17 coverage enhancement.

Link level simulation performances of coverage for FR1

On required SNR

As we can see from Table 1, the required SNR of the target channel is one key factors for identifying coverage bottleneck. According to above mentioned method of parameter determination, values of link level simulation parameters for different scenarios are attached in the Appendix, e.g. Table 12~14.  Based on these parameter settings, required SNR towards different channels are provided as follows:

Table 4 . Required SNR of PUSCH and PUCCH towards different scenario

| Required SNR (dB) | Uplink Channel |  |  |
| --- | --- | --- | --- |
|  | PUSCH (eMBB) | PUSCH (VoIP) | PUCCH |
| Urban TDD | 1.55 | 1 | -6.63 |
| Rural(O2I) TDD | 1.15 | 1 | -6.63 |
| Rural(O2O) TDD | -0.9 | 0.8 | -6 |
| Rural(O2I) FDD | -1.9 | 1.3 | -6.57 |
| Rural(O2O) FDD | -2 | 0.9 | -6.14 |
| Rural long distance FDD | -6.9 | -4.1 | -10.81 |

Table 5. Required SNR of other channels including Msg3, PDCCH and PDSCH

| Required SNR (dB) | Msg3 (10% BLER) | PDCCH (1% BLER) | PDSCH (10% BLER) |
| --- | --- | --- | --- |
| Urban TDD | -1.3 dB | -5.7 dB | -4.15 dB |
| Rural(O2I) TDD | -1.3 dB |  | -3.72 dB |
| Rural(O2O) TDD | -2 dB |  | -4.1 dB |
| Rural(O2I) FDD | -1.3 dB |  | -4 dB |
| Rural(O2O) FDD | -1.4 dB |  | -4.25 dB |
| Rural with long distance FDD | -1.3 dB |  | -5.64 dB |

On coverage performance of PUSCH and PUCCH

After acquiring the operation point (required SNR) for different channels, the available path loss can be deduced from aforementioned inputs for the link budget template, thus we could find out the bottleneck channel of coverage by comparing the Available pathloss and Target performance.  Note that:

- Available pathloss means the MCL at required SNR which can achieve the target data rate while the maximum coverage distance at the required SNR might not be achieved at the cell edge.

- Target performance means the MCL at the distance of being 23* ISD from the base station for hexagonal cells.

If the gap (Target performance- Available pathloss) is positive, then the coverage requirement is not achieved. As shown in Table 6~8, evaluation results for PUSCH and PUCCH are presented as follows:

Table 6. Coverage performance of PUSCH towards eMBB

| Scenario (ISD) | Available path loss (dB) | Target performance (dB) | Gap |
| --- | --- | --- | --- |
| Urban O2I TDD (500m) | 109.75 | 124.2 | 14.45 |
| Rural O2I TDD (1732m) | 125.19 | 133.99 | 8.8 |
| Rural O2O TDD (1732m) | 131.29 | 133.99 | 2.7 |
| Rural O2I FDD (1732m) | 130.28 | 127.97 | -2.31 |
| Rural O2O FDD (1732m) | 132.4 | 127.97 | -4.43 |
| Rural long distance O2O FDD (3km) | 139.12 | 128.07 | -11.05 |
| Rural long distance O2O FDD (12km) | 139.12 | 151.33 | 12.21 |

Table 7. Coverage performance of PUSCH towards VoIP

| Scenario (ISD) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- |
| Urban O2I TDD (500m) | 119.3 | 124.2 | 4.9 |
| Rural O2I TDD (1732m) | 125.34 | 133.99 | 8.65 |
| Rural O2O TDD (1732m) | 129.59 | 133.99 | 4.4 |
| Rural O2I FDD (1732m) | 127.08 | 127.97 | 0.89 |
| Rural O2O FDD (1732m) | 129.5 | 127.97 | -1.53 |
| Rural long distance O2O FDD (3km) | 136.12 | 128.07 | -8.05 |
| Rural long distance O2O FDD (12km) | 136.12 | 151.33 | 15.21 |

Table 8. Coverage performance of PUCCH

| Scenario (ISD) | Available path loss (dB) | Target performance (dB) | Gap (dB) |
| --- | --- | --- | --- |
| Urban O2I TDD (500m) | 126.05 | 124.2 | -1.85 |
| Rural O2I TDD (1732m) | 132.1 | 133.99 | 1.89 |
| Rural O2O TDD (1732m) | 135 | 133.99 | -1.01 |
| Rural O2I FDD (1732m) | 134.08 | 127.97 | -6.11 |
| Rural O2O FDD (1732m) | 135.15 | 127.97 | -7.18 |
| Rural long distance O2O FDD (3km) | 144.27 | 128.07 | -16.2 |
| Rural long distance O2O FDD (12km) | 144.27 | 151.33 | 7.06 |

Note that:

- A large coverage gap of PUSCH towards eMBB at urban scenario (14.45 dB) is observed, because a high uplink data rate as 1Mbps and a large ISD=500m are assumed. Furthermore, for outdoor gNB serving indoor UEs, a typical large penetration loss of 26.25 dB is used.

- In the calculation of required SNR of PUSCH towards VoIP service, 20ms packet transmission period is considered with repnum=1;

- In the above calculation of Target performance for data channels such as PUSCH, values of key related parameters including BS antenna height, number of scheduled PRBs, selected MCS and so on can be found in the Appendix.

Observation 1: According to the reference ISD in TR 38.913 for the concerning scenarios:

- Urban outdoor gNB serving indoor UE: under TDD duplex mode and 500m ISD, PUSCH cannot achieve coverage requirement for eMBB and VoIP service;

- Rural outdoor gNB serving indoor UE: under TDD duplex mode and 1732m ISD, PUSCH cannot achieve coverage requirement for eMBB and VoIP service, PUCCH cannot fulfil coverage requirement either;

- Rural outdoor gNB serving outdoor UE: under TDD duplex mode and 1732m ISD, PUSCH cannot achieve coverage requirement for eMBB and VoIP service;

- Rural with long distance outdoor gNB serving outdoor UE: under FDD duplex mode and 12km ISD, both PUSCH and PUCCH fail to achieve coverage requirement.

In conclusion, we have the following proposal.

Proposal 3: PUSCH should be prioritized for coverage enhancement.

On coverage performance of other channels

After acquiring the operation point of other channels in section 2.2.1 and following the same calculation method in section 2.2.2, the available pathloss and target performance for other channels including Msg3, PDCCH and PDSCH are illustrated in Table 9~11, respectively.

Table 9. Coverage performance of Msg3

| Scenario (ISD) | Available path loss (dB) | Target performance (dB) | Gap |
| --- | --- | --- | --- |
| Urban O2I TDD (500m) | 144.63 | 124.3 | -20.33 |
| Rural O2I TDD (1732m) | 142.63 | 134.03 | -8.59 |
| Rural O2O TDD (1732m) | 141.85 | 133.98 | -7.87 |
| Rural O2I FDD (1732m) | 142.63 | 127.96 | -14.67 |
| Rural O2O FDD (1732m) | 144.75 | 127.95 | -17.8 |
| Rural long distance O2O FDD (3km) | 146.47 | 128.07 | -18.4 |
| Rural long distance O2O FDD (12km) | 146.47 | 151.33 | 4.86 |

Table 10. Coverage performance of PDCCH

| Scenario (ISD) | Available path loss (dB) | Target performance (dB) | Gap |
| --- | --- | --- | --- |
| Urban O2I TDD (500m) | 131.65 | 124.15 | -7.5 |
| Rural O2I TDD (1732m) | 143.21 | 133.98 | -9.23 |
| Rural O2O TDD (1732m) | 144.71 | 133.98 | -10.73 |
| Rural O2I FDD (1732m) | 143.21 | 127.96 | -15.25 |
| Rural O2O FDD (1732m) | 144.71 | 127.96 | -16.75 |
| Rural long distance O2O FDD (3km) | 149.16 | 128.06 | -21.1 |
| Rural long distance O2O FDD (12km) | 149.16 | 151.32 | 1.16 |

Table 11. Coverage performance of PDSCH

| Scenario (ISD) | Available path loss (dB) | Target performance (dB) | Gap |
| --- | --- | --- | --- |
| Urban O2I TDD (500m) | 128.62 | 124.20 | -4.42 |
| Rural O2I TDD (1732m) | 150.09 | 133.99 | -16.1 |
| Rural O2O TDD (1732m) | 152.49 | 133.99 | -18.5 |
| Rural O2I FDD (1732m) | 150.65 | 127.97 | -22.68 |
| Rural O2O FDD (1732m) | 152.64 | 127.97 | -24.67 |
| Rural long distance O2O FDD (3km) | 155.85 | 128.07 | -27.78 |
| Rural long distance O2O FDD (12km) | 155.85 | 151.33 | -4.52 |

In the calculation of target performance for data channels such as PDSCH under different scenarios, values of key related parameters including BS antenna height, number of scheduled PRB, selected MCS can be found in the Appendix.

Observation 2: According to the reference ISD in TR 38.913, for the concerning scenarios:

- Msg3 can achieve the coverage requirement under urban, rural, rural with long distance (ISD=3km) scenarios for both FDD and TDD duplex modes, while there is a 4.86 dB coverage gap under rural with long distance (ISD=10km) scenario

- PDCCH can achieve the coverage requirement under urban, rural, rural with long distance (ISD=3km) scenarios for both FDD and TDD duplex modes, while there is a 1.16 dB coverage gap under rural with long distance (ISD=10km) scenario

- PDSCH can achieve the coverage requirement under all scenarios, including urban, rural, rural with long distance (ISD=3km) and rural with long distance (ISD=10km) scenarios.

In conclusion, we have the following proposal.

Proposal 4: For other channels including Msg3, PDCCH and PDSCH can achieve coverage requirements, no need of coverage enhancements.

Conclusions

In this contribution, the following observations and proposals are given:

Observation 1: According to the reference ISD in TR 38.913 for the concerning scenarios:

- Urban outdoor gNB serving indoor UE: under TDD duplex mode and 500m ISD, PUSCH cannot achieve coverage requirement for eMBB and VoIP service;

- Rural outdoor gNB serving indoor UE: under TDD duplex mode and 1732m ISD, PUSCH cannot achieve coverage requirement for eMBB and VoIP service, PUCCH cannot fulfil coverage requirement either;

- Rural outdoor gNB serving outdoor UE: under TDD duplex mode and 1732m ISD, PUSCH cannot achieve coverage requirement for eMBB and VoIP service;

- Rural with long distance outdoor gNB serving outdoor UE: under FDD duplex mode and 12km ISD, both PUSCH and PUCCH fail to achieve coverage requirement.

Observation 2: According to the reference ISD in TR 38.913, for the concerning scenarios:

- Msg3 can achieve the coverage requirement under urban, rural, rural with long distance (ISD=3km) scenarios for both FDD and TDD duplex modes, while there is a 4.86 dB coverage gap under rural with long distance (ISD=10km) scenario

- PDCCH can achieve the coverage requirement under urban, rural, rural with long distance (ISD=3km) scenarios for both FDD and TDD duplex modes, while there is a 1.16 dB coverage gap under rural with long distance (ISD=10km) scenario

- PDSCH can achieve the coverage requirement under all scenarios, including urban, rural, rural with long distance (ISD=3km) and rural with long distance (ISD=10km) scenarios.

Proposal 1: Reuse the evaluation methodology of link budget in IMT-2020 self-evaluation for Rel-17 NR coverage enhancement at FR1.

Proposal 2: Adopt the revised values in Table 3 to the link budget template for the baseline link budget evaluation of Rel-17 coverage enhancement.

Proposal 3: PUSCH should be prioritized for coverage enhancement.

Proposal 4: For other channels including Msg3, PDCCH and PDSCH can achieve coverage requirements, no need of coverage enhancements.

- References

- RP-191886, “Summary of email discussion on NR coverage enhancement”, RAN#85, Newport Beach, USA, September 16-19, 2019.

- RP-193240, “New SID on NR coverage enhancement”, RAN#86, Sitges, Spain, December 9 – 12, 2019.

RP-181814, “Consideration on IMT-2020 link budget”, RAN#81, Gold Coast, Australia, 10 - 13 September 2018

RP-191886, “Summary of email discussion on NR coverage enhancement”, China Telecom, Sep, 2019

- Appendix

- Link level evaluation assumption for PUSCH

Table 12. Evaluation assumptions for PUSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 4GHz TDD | 4GHz for TDD 2GHz for FDD | 700MHz FDD |
| Transmission bit rate for data channel (bit/s) | 1 Mbps for eMBB 12.2Kbps for VoIP | 100 Kbps for eMBB 12.2Kbps for VoIP | 100 Kbps for eMBB 12.2Kbps for VoIP |
| Frame structure | DDDSUDDSUU | DDDSUDDSUU for TDD UUUUU for FDD | UUUUU |
| Subcarrier spacing | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-E |
| Delay Spread | 300ns | 300ns | 300ns |
| Pathloss model (select from LoS or NLoS) | NLOS | NLOS | LOS |
| UE velocity | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| Occupied channel bandwidth | 30 PRBs for eMBB 4 PRBs for VoIP | 4 PRBs | 4 PRBs |
| Number of BS antennas & antenna configuration | 192 (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) |
| Number of TXRU for BS | 2 | 2 | 2 |
| Number of UE antennas | 2 | 2 | 1 |
| Number of TRXU for UE | 2 | 2 | 1 |
| Overhead assumption for required SNR simulation | DMRS: Type I, one DMRS symbol, no multiplexing with data | DMRS: - For 3km/h: Type I, one DMRS symbol, no multiplexing with data. - For 120km/h: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data | DMRS: For 120km/h: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |
| Target BLER | 10% |  |  |

- Link level evaluation assumption for PUCCH

Table 13. Evaluation assumptions for PUSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 4GHz TDD | 4GHz for TDD 2GHz for FDD | 700MHz FDD |
| Format type | Format 1 (long PUCCH with 14 OFDM symbols), 2bits | Format 1 (long PUCCH with 14 OFDM symbols), 2bits | Format 1 (long PUCCH with 14 OFDM symbols), 2bits |
| Subcarrier spacing | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-E |
| Pathloss model (select from LoS or NLoS) | NLOS | NLOS | LOS |
| Delay Spread | 300ns | 300ns | 300ns |
| UE velocity | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| Occupied channel bandwidth | 1 PRB | 1 PRB | 1 PRB |
| Number of BS antennas & antenna configuration | 192 (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) |
| Number of TXRU for BS | 2 | 2 | 2 |
| Number of UE antennas | 2 | 2 | 1 |
| Number of TRXU for UE | 2 | 2 | 1 |
| Target BLER | 1% |  |  |

- Link level evaluation assumption for other channels

Table 14. Evaluation assumptions for other channels

| Other channels | Msg3 | PDCCH | PDSCH |
| --- | --- | --- | --- |
| Key parameters | MCS: 0 (2,120/1024) Scheduled PRB: 2 Payload size: 56 bit Channel model: TDL-C Moving speed: 3km/h & 120km/h DMRS overhead: 1 front-loaded DMRS 1 front loaded DMRS + 1 additional DMRS | Aggregation level: 8 DCI payload size: 60 bit CRC length: 24 bit CORRESET PRB: 48 CORRESET symbols: 2 Channel model: TDL-C Moving speed: 3km/h & 120km/h DMRS overhead: 1 front-loaded DMRS 1 front loaded DMRS + 1 additional DMRS | MCS: 2 (2,251/1024) Scheduled PRB: Calculated Channel model: TDL-C Moving speed: 3km/h & 120km/h DMRS overhead: 1 front-loaded DMRS 1 front loaded DMRS + 1 additional DMRS Target data rate: 10Mbps for urban 1Mbps for rural |
| Target BLER | 10% | 1% | 10% |

More additional notes on the evaluation parameters and metrics:

- MCS selection:

Low MCS levels are selected to ensure the coverage, especially for uplink transmission, such as MCS 0 for Msg3 and MCS 2 for PDSCH.

- Determination of number of scheduled PRBs:

Following the determination method of number of PRB in section 2.2 and considering target data rates of 10 Mbps for urban scenario and 1Mbps for other scenarios with MCS 2, the number of scheduled PRBs for PDSCH are listed below:

| Parameters | Urban TDD | Rural(O2I) FDD | Rural(O2O) FDD | Rural(O2I) TDD | Rural(O2O) TDD | Rural long distance FDD |
| --- | --- | --- | --- | --- | --- | --- |
| Num. of PRB | 154 | 30 | 32 | 16 | 16 | 32 |
| TBS | 5640 | 1128 | 1128 | 608 | 552 | 1128 |

- Moving speed and DMRS overhead:

Following the same reason as explained in section 2.2, a moving speed of 3 km/h is assumed for NLoS O2I scenario with indoor UE inside a building while a moving speed of 120km/h is assumed for NLoS O2O scenario with outdoor UE in a car.

For low moving speed of being 3km/h, 1 front-loaded single-symbol DMRS is enough to estimate the fading channel; for high moving speed of being 120km/h, 1 front-loaded and 1 additional single-symbol DMRSs are used to estimate the fading channel.

- DCI payload size of PDCCH: a small payload size of PDCCH being 60 bits is used as a reference since usually a smaller payload size of PDCCH can achieve a better coverage.

- Parameter values of PUSCH Target Performance for eMBB

Table 15. Values of key related parameters of PUSCH Target Performance for eMBB services

| Scenarios | BS antenna height | Num of PRB | MCS selection | TBS | Operation point |
| --- | --- | --- | --- | --- | --- |
| Urban O2I TDD (500m) | 25 m | 30 | 3 (2, 251/1024) | 2280 | -12.9 dB |
| Rural O2I TDD (1732m) | 35 m | 4 | 1 (2, 157/1024) | 184 | -3.6 dB |
| Rural O2O TDD (1732m) | 35 m | 4 | 1 (2, 157/1024) | 176 | -7.65 dB |
| Rural O2I FDD (1732m) | 35 m | 4 | 0 (2, 120./1024) | 144 | 2.43 dB |
| Rural O2O FDD (1732m) | 35 m | 4 | 0 (2, 120./1024) | 128 | 0.41 dB |
| Rural long distance O2O FDD (3km) | 35 m | 4 | 0 (2, 120./1024) | 144 | 4.15 dB |
| Rural long distance O2O FDD (12km) | 35 m | 4 | 0 (2, 120./1024) | 128 | -19.11 dB |

- Parameter values of PDSCH Target Performance for eMBB

Table 16. Values of key related parameters of PDSCH Target Performance for eMBB services

| Scenarios | BS antenna height | Num of PRB | MCS selection | TBS | Operation point |
| --- | --- | --- | --- | --- | --- |
| Urban O2I TDD (500m) | 25 m | 154 | 0 (2, 120/1024) | 5640 | 0.27 dB |
| Rural O2I TDD (1732m) | 35 m | 16 | 0 (2, 120/1024) | 608 | 14.4 dB |
| Rural O2O TDD (1732m) | 35 m | 16 | 0 (2, 120/1024) | 552 | 12.38 dB |
| Rural O2I FDD (1732m) | 35 m | 30 | 0 (2, 120/1024) | 1128 | 20.43 dB |
| Rural O2O FDD (1732m) | 35 m | 32 | 0 (2, 120/1024) | 1128 | 18.68 dB |
| Rural long distance O2O FDD (3km) | 35 m | 32 | 0 (2, 120/1024) | 1128 | 22.14 dB |
| Rural long distance O2O FDD (12km) | 35 m | 32 | 0 (2, 120/1024) | 1128 | -1.12 dB |