3GPP TSG RAN WG1 #101		R1-2003354

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Transmit power of CSI-RS across different occasions

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the issue of transmission power of CSI-RS across different occasions in unlicensed band, as raised in LS [1] from RAN4.

- Discussion

In [1], RAN4 ask RAN1 to confirm whether a UE can assume that CSI-RS (for L1-RSRP, RLM, BFD and CBD) is transmitted with the same transmit power across different occasions during the measurement period in unlicensed band, and whether this also applies to SSB-based RRM measurements. The question comes from the agreements RAN1 made in RAN1#99, given as follows

Agreement: A UE shall not average CSI-RS measurements for channel estimation across different transmission bursts from the UE's perspective. FFS: Potential issues due to AGC

Due to the regulatory requirement, the transmission power for a node may be limited based on LBT outcome. The transmission power can be changed with different number of LBT subbands obtained. Therefore, UE can average the CSI-RS measurement within a transmission burst, but averaging across transmission burst is prohibited.

From RAN4 perspective, a measurement result provided by layer 1 is generated based on measurement on multiple RS occasions in a measurement period or evaluation period to guarantee the accuracy and meet the requirement, for RRM, RLM, BFD, L1-RSRP measurement, etc.. The measurement period or evaluation period is usually tens of hundreds of milliseconds, which may last for multiple transmission bursts in unlicenced band. If averaging between different CSI-RS occasions are not allowed due to power inconsistency, the aforementioned measurements can hardly meet the measurement requirement.

Although the transmission power in a LBT subband is not necessarily consistent across different transmission bursts, the network is able to keep the EPRE of CSI-RS consistent through proper implementation, e.g., the transmission power can be allocated to CSI-RS with highest priority.

Furthermore, the RRC signalling can expilictly indicated the purpose of the configured CSI-RS resource, which means the CSI-RS resources can be separately configured for different purposes. For example, the CSI-RS resource for CSI acquisition is configured by CSI-MeasConfig, CSI-RS for RLM/BFD is configured by RadioLinkMonitoringRS, CSI-RS for RRM is configured by CSI-RS-ResourceConfigMobility. When the CSI-RS resource is configured for these purposes other than CSI aquisition, it is up to network implementation to guarantee the EPRE constant, regardless of whether the resources are in the same transmission burst or not. UE always assume EPRE of the CSI-RS resources for RRM, RLM, BFD, L1-RSRP, CBD constant, once configured.

Observation 1: CSI-RS resources can be separately configured for different purposes.

Proposal 1: UE can assume EPRE of the CSI-RS resources for RRM, RLM, BFD, L1-RSRP, CBD constant once configured. It is up to network implementation to guarantee the EPRE constant across different occasions.

For SSB based RRM in unlicensed spectrum, the UE can still assume that SSBs transmitted with the same block index on the same center frequency location are quasi co-located with respect to average gain, which is the same as that in licensed spectrum.

Observation 2: For SSB based RRM in unlicensed spectrum, the UE can still assume that SSBs transmitted with the same block index on the same center frequency location are quasi co-located with respect to average gain.

Proposal 2: Send LS to RAN4 to inform above understandings.

- Conclusion

In this contribution, we discussed the transmission power in different occasions, and we have following observation and proposal.

Observation 1: CSI-RS resources can be separately configured for different purposes.

Proposal 1: UE can assume EPRE of the CSI-RS resources for RRM, RLM, BFD, L1-RSRP, CBD constant once configured. It is up to network implementation to guarantee the EPRE constant across different occasions.

- Send LS to RAN4 to inform above decision.

Observation 2: For SSB based RRM in unlicensed spectrum, the UE can still assume that SSBs transmitted with the same block index on the same center frequency location are quasi co-located with respect to average gain.

Proposal 2: Send LS to RAN4 to inform above understandings.

- References

- R1-2003273, “LS on transmit power of CSI-RS across different occasions”, RAN1 #101-e.