3GPP TSG RAN WG1 Meeting #101 					            R1-2003373

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Remaining issues on enhancements to configured grant

Agenda Item:	7.2.2.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on the remaining issues on enhancements to configured grant, including COT sharing indication and HARQ process conflicts between CG (configured grant) PUSCH and DG (dynamic grant) PUSCH, and UCI multiplexing.

- Discussion

  - COT sharing indication

The following agreements were made in RAN1  #98bis meeting [2] and #99 meeting [3].

Agreement #98bis: Sharing of a UE-initiated channel occupancy (either CG-PUSCH or scheduled UL) with gNB is supported, such that the gNB is allowed to transmit control/broadcast signals/channels for any UEs as long as the transmission contains transmissions for the UE that initiated the channel occupancy and/or DL signal ls/channels (PDSCH, PDCCH, reference signals) meant for the UE that initiated the channel occupancy. - The ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is configured by gNB (RRC signaling) - if ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is not configured, the transmission of the gNB in UE initiated COT may include only control/broadcast signals/channels transmissions of up to 2/4/8 OFDM symbols in duration for 15/30/60 kHz SCS Agreement #98bis: CG-UCI is included in every CG-PUSCH transmission (confirms working assumption from RAN1#98) Agreement #99: When ED threshold is configured, UCI indication for purpose of COT sharing: - UE provides a row index in a RRC configured table where D, O and CAPC are jointly encoded. - D: number of slots where DL transmissions can be assumed within UE initiated COT. - O: DL offset indicates starting slot of DL transmission indicated in number of slots from the end of the slot where the indicated D>0. - one row indicates no COT sharing information. When ED threshold is not configured, - COT sharing indication: 1 bit indicating if slot/symbol n+X is an applicable slot for UL to DL sharing). X is configured by the gNB as part of RRC configuration. - X is number of symbols from the end of the slot where the indication is enabled

The CG-UCI is included in every CG-PUSCH transmission. The CG-UCI within different CG-PUSCHs indicates the COT sharing information independently. The CG-UCI information in the later CG-PUSCH cannot override the information in the earlier ones.

Proposal 1. The CG-UCI indication carried by different PUSCHs indicates the COT sharing information independently, which means COT sharing information carried in later CG-PUSCH will not override the information in the earlier ones.

  - HARQ process conflicts between CG PUSCH and DG PUSCH

The HARQ process conflicts between CG PUSCH and DG PUSCH are specified in TS 38.214, as following:

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

However, in NR-U, the HARQ process id of configured grant PUSCH is selected by UE, which is not known by gNB before decoding the CG-UCI successfully. Therefore, above description in the specification is only applicable for the system operating in licensed spectrum, which should be clarified in the spec. To clarify following TP is proposed:

------------------------------------------- Start of Draft TP of 214 -------------------------------------

<Unchanged part omitted>

Except for operation with shared spectrum channel access, a A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

<Unchaged part omitted>

---------------------------------------------- end of Draft TP ---------------------------------------------

Proposal 2. The specification description in 38214 section 6.1 for HARQ process conflicts between CG PUSCH and DG PUSCH is only applicable for operation in licensed spectrum, adopt above TP.

  - Beta offset determination for CG-UCI

For CG-UCI multiplexing, some agreements have been achieved in previous RAN1 meetings.

Agreement #98: - CG-UCI, CSI-part1, CSI-part 2 can be sent on CG-PUSCH at least when CG-UCI and HARQ-ACK feedback is not multiplexed on a CG-PUSCH. Agreement #98bis: To determine the number of REs used for CG-UCI, the mechanism of beta-offset in Rel-15 NR for HARQ-ACK on CG-PUSCH is reused. - A new RRC parameter to configure the beta-offset for CG-UCI is defined. FFS: Value range Agreement #99: RRC configuration can be provided to the UE indicating whether to multiplex CG-UCI and HARQ-ACK - When configured for such multiplexing: In the case of PUCCH overlapping with CG-PUSCH(s) within a PUCCH group, the CG-UCI and HARQ-ACK are jointly encoded (CG-UCI is treated as the same type as a HARQ-ACK) - When not configured for such multiplexing: In the case of PUCCH overlapping with CG-PUSCH(s) within a PUCCH group and PUCCH carries HARQ ACK feedback, configured grant PUSCH is skipped Agreement #100e: The set of Beta-offset values used for CG-UCI is the same as the set of Beta-offset values used for HARQ-ACK - When CG-UCI is jointly encoded with HARQ, use the Beta-offset values configured for HARQ-ACK

It has been agreed in RAN1#98bis that to determine the number of REs used for CG-UCI, the mechanism of beta-offset in Rel-15 NR for HARQ-ACK on CG-PUSCH is reused.

In Rel-15 NR for HARQ-ACK on CG-PUSCH, the beta offset can be RRC configured as semi-static or dynamic. When it is configured as dynamic, at most 4 values are configured by RRC, and one of them is indicated by the activation DCI or determined from the first value of CG-UCI-OnPUSCH = 'dynamic' in case that the PUSCH is a configured grant Type 2 PUSCH activated by DCI format 0_0 . However, per the latest specification, beta offset for CG-UCI in CG-PUSCH can only be configured semi-statically. The motivation to support dynamic indication for beta offset is for the flexibiltity and this flexibiltity should also be supported for CG-UCI.

Proposal 3. The beta-offset values used for CG-UCI should be supported to be configured as dynamic. Adopt the following TP for TS 38.213 to capature the beta offset determination for CG-UCI correctly.

------------------------------------------- Start of Draft TP of 213 -------------------------------------

9.3	UCI reporting in physical uplink shared channel

Offset values are defined for a UE to determine a number of resources for multiplexing HARQ-ACK information and for multiplexing CSI reports in a PUSCH. Offset values are also defined for multiplexing CG-UCI [5, TS 38.212] in a CG-PUSCH. The offset values are signalled to a UE either by a DCI format scheduling the PUSCH transmission or by higher layers.

If a DCI format that does not include a beta_offset indicator field schedules the PUSCH transmission from the UE and the UE is provided betaOffsets = 'semiStatic', the UE applies the , βoffsetCG-UCI,, and  values that are provided by betaOffsets = 'semiStatic' for the corresponding HARQ-ACK information, CG-UCI, Part 1 CSI reports and Part 2 CSI reports.

If the PUSCH transmission is with a configured grant and the UE is provided CG-UCI-OnPUSCH= 'semiStatic', the UE applies the ,  βoffsetCG-UCI,, and  values that are provided by CG-UCI-OnPUSCH = 'semiStatic' for the corresponding HARQ-ACK information, CG-UCI, Part 1 CSI reports and Part 2 CSI reports.

If the PUSCH is scheduled by DCI format 0_0 and the UE is provided betaOffsets = 'dynamic', the UE applies the  , βoffsetCG-UCI, , and  values that are determined from the first value of  betaOffsets = 'dynamic'.

If the PUSCH is a configured grant Type 2 PUSCH activated by DCI format 0_0 and the UE is provided CG-UCI-OnPUSCH ='dynamic', the UE applies the ,  βoffsetCG-UCI,, and  values that are determined from the first value of CG-UCI-OnPUSCH = 'dynamic'.

HARQ-ACK information offsets  are configured to values according to Table 9.3-1. The betaOffsetACK-Index1, betaOffsetACK-Index2, and betaOffsetACK-Index3 respectively provide indexes , , and  for the UE to use if the UE multiplexes up to 2 HARQ-ACK information bits, more than 2 and up to 11 HARQ-ACK information bits, and more than 11 bits in the PUSCH, respectively.

CG-UCI offsets βoffsetCG-UCI are configured to values according to Table 9.3-1.

Part 1 CSI report and Part 2 CSI report offsets  and , respectively, are configured to values according to Table 9.3-2. The betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part2-Index1 respectively provide indexes  and  for the UE to use if the UE multiplexes up to 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. The betaOffsetCSI-Part1-Index2 and betaOffsetCSI-Part2-Index2 respectively provide indexes  or  for the UE to use if the UE multiplexes more than 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH.

If a DCI format that includes a beta_offset indicator field with one bit or two bits, as configured by uci-OnPUSCH, schedules the PUSCH transmission from the UE, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3} a set of two or four  indexes, by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of two or four  indexes, and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of two or four  indexes from Tables 9.3-1 and 9.3-2, respectively, for multiplexing HARQ-ACK information, CG-UCI, Part 1 CSI reports, and Part 2 CSI reports, respectively, in the PUSCH transmission. If a DCI format that includes a beta_offset indicator field with two bits, as configured by uci-OnPUSCH, schedules the PUSCH transmission from the UE, the UE is provided by each of { betaOffsetCG-UCI-Index-r16} a set of four IoffsetCG-UCI indexes. The beta_offset indicator field indicates a  value, a IoffsetCG-UCI value, a  value and a  value from the respective sets of values, with the mapping defined in Table 9.3-3 and in Table 9.3-3A.

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. Iif the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK.

Table 9.3-3: Mapping of four beta_offset indicator values to offset indexes

| beta_offset indicator | ( or or ), ( or ), ( or ), (IoffsetCG-UCI) |
| --- | --- |
| '00' | 1st offset index provided by higher layers |
| '01' | 2nd offset index provided by higher layers |
| '10' | 3rd offset index provided by higher layers |
| '11' | 4th offset index provided by higher layers |

---------------------------------------------- end of Draft TP ---------------------------------------------

- Conclusion

In the contribution, we provide our views on the enhancements to configured grant, and we propose that:

Proposal 1. The CG-UCI indication carried by different PUSCHs indicates the COT sharing information independently, which means COT sharing information carried in later CG-PUSCH will not override the information in the earlier ones.

------------------------------------------- Start of Draft TP of 214 -------------------------------------

<Unchanged part omitted>

Except for operation with shared spectrum channel access, a A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

<Unchaged part omitted>

---------------------------------------------- end of Draft TP ---------------------------------------------

Proposal 2. The specification description in 38214 section 6.1 for HARQ process conflicts between CG PUSCH and DG PUSCH is only applicable for operation in licensed spectrum, adopt above TP.

Proposal 3. The beta-offset values used for CG-UCI should be supported to be configured as dynamic. Adopt the following TP for TS 38.213 to capature the beta offset determination for CG-UCI correctly.

------------------------------------------- Start of Draft TP of 213 --------------------------------------

9.3	UCI reporting in physical uplink shared channel

Offset values are defined for a UE to determine a number of resources for multiplexing HARQ-ACK information and for multiplexing CSI reports in a PUSCH. Offset values are also defined for multiplexing CG-UCI [5, TS 38.212] in a CG-PUSCH. The offset values are signalled to a UE either by a DCI format scheduling the PUSCH transmission or by higher layers.

If a DCI format that does not include a beta_offset indicator field schedules the PUSCH transmission from the UE and the UE is provided betaOffsets = 'semiStatic', the UE applies the , βoffsetCG-UCI,, and  values that are provided by betaOffsets = 'semiStatic' for the corresponding HARQ-ACK information, CG-UCI, Part 1 CSI reports and Part 2 CSI reports.

If the PUSCH transmission is with a configured grant and the UE is provided CG-UCI-OnPUSCH= 'semiStatic', the UE applies the ,  βoffsetCG-UCI,, and  values that are provided by CG-UCI-OnPUSCH = 'semiStatic' for the corresponding HARQ-ACK information, CG-UCI, Part 1 CSI reports and Part 2 CSI reports.

If the PUSCH is scheduled by DCI format 0_0 and the UE is provided betaOffsets = 'dynamic', the UE applies the  , βoffsetCG-UCI, , and  values that are determined from the first value of  betaOffsets = 'dynamic'.

If the PUSCH is a configured grant Type 2 PUSCH activated by DCI format 0_0 and the UE is provided CG-UCI-OnPUSCH ='dynamic', the UE applies the ,  βoffsetCG-UCI,, and  values that are determined from the first value of CG-UCI-OnPUSCH = 'dynamic'.

HARQ-ACK information offsets  are configured to values according to Table 9.3-1. The betaOffsetACK-Index1, betaOffsetACK-Index2, and betaOffsetACK-Index3 respectively provide indexes , , and  for the UE to use if the UE multiplexes up to 2 HARQ-ACK information bits, more than 2 and up to 11 HARQ-ACK information bits, and more than 11 bits in the PUSCH, respectively.

CG-UCI offsets βoffsetCG-UCI are configured to values according to Table 9.3-1.

Part 1 CSI report and Part 2 CSI report offsets  and , respectively, are configured to values according to Table 9.3-2. The betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part2-Index1 respectively provide indexes  and  for the UE to use if the UE multiplexes up to 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. The betaOffsetCSI-Part1-Index2 and betaOffsetCSI-Part2-Index2 respectively provide indexes  or  for the UE to use if the UE multiplexes more than 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH.

If a DCI format that includes a beta_offset indicator field with one bit or two bits, as configured by uci-OnPUSCH, schedules the PUSCH transmission from the UE, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3} a set of two or four  indexes, by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of two or four  indexes, and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of two or four  indexes from Tables 9.3-1 and 9.3-2, respectively, for multiplexing HARQ-ACK information, CG-UCI, Part 1 CSI reports, and Part 2 CSI reports, respectively, in the PUSCH transmission. If a DCI format that includes a beta_offset indicator field with two bits, as configured by uci-OnPUSCH, schedules the PUSCH transmission from the UE, the UE is provided by each of { betaOffsetCG-UCI-Index-r16} a set of four IoffsetCG-UCI indexes. The beta_offset indicator field indicates a  value, a IoffsetCG-UCI value, a  value and a  value from the respective sets of values, with the mapping defined in Table 9.3-3 and in Table 9.3-3A.

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. Iif the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK.

Table 9.3-3: Mapping of four beta_offset indicator values to offset indexes

| beta_offset indicator | ( or or ), ( or ), ( or ), (IoffsetCG-UCI) |
| --- | --- |
| '00' | 1st offset index provided by higher layers |
| '01' | 2nd offset index provided by higher layers |
| '10' | 3rd offset index provided by higher layers |
| '11' | 4th offset index provided by higher layers |

---------------------------------------------- end of Draft TP ---------------------------------------------

- Reference

- Chairman Notes, 3GPP TSG RAN WG1 #100-e meeting

- Chairman Notes, 3GPP TSG RAN WG1 #98 bis meeting

- Chairman Notes, 3GPP TSG RAN WG1 #99 meeting

- Chairman Notes, 3GPP TSG RAN WG1 #101b-e meeting