3GPP TSG RAN WG1 #101		R1-2003387

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	PDCCH enhancements for URLLC

Agenda Item:	7.2.5.1

Document for:	Discussion and Decision

- Introduction

In the previous meeting, PDCCH enhancements for URLLC are discussed. Some agreements are achieved as following [1][2]:

In RAN1#99 meeting:

Working assumption:

When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

In this contribution, we share our view on remaining issues of PDCCH enhancements for URLLC.

- DCI format for URLLC

  - DCI size differentiation between DCI Format 0_2/1_2 and DCI Format 0_1/1_1

In DCI size alignment procedure, DCI size alignments between DCI Format 0_0 and DCI Format 1_0 in CSS and USS are performed in STEP 0 and STEP 1, respectively. Then DCI sizes of Format 0_1 and Format 1_1 are differentiated with DCI size of Format 0_0/1_0 in STEP 2, and DCI sizes of Format 0_2 and Format 1_2 are differentiated with DCI size of Format 0_0/1_0  in STEP 2A.

A UE can be configured with monitoring for DCI Format 0_1/1_1 and DCI Format 0_2/1_2 in USS. In that case, DCI size differentiation among Format 0_1, Format 1_1, Format 0_2 and Format 1_2 should be required. Because after the size differentiation operation, DCI sizes of Format 0_1 (or Format 1_1) and Format 0_2 (or Format 1_2) may be equal. For example, gNB can configure the different DCI sizes between Format 0_2 (or Format 1_2) and Format 0_0/1_0. If DCI size of Format 0_2 (or Format 1_2) is larger than DCI Format 0_0/1_0 with only 1 bit and if DCI size of Format 0_1 (or Format 1_1) equals DCI size of Format 0_0/1_0. After the size differentiation operation, DCI size of Format 0_1 (or Format 1_1) may equal the DCI size of Format 0_2 (or Format 1_2). In USS, UE may not determinate which DCI is scheduled if the same DCI size occurs.

Thus, DCI sizes differentiation among Format 0_1 (or Format 1_1) and Format 0_2 (or Format 1_2) should be added in DCI size alignment procedure as STEP 2B. If DCI size of Format 0_1 or Format 1_1 equals the DCI size of Format 0_2 or Format 1_2, 1 padding bit is added in Format 0_1 or Format 1_1 until any DCI sizes of Format 0_1 and Format 1_1 are is different with one of Format 0_2 and Format 1_2.

Proposal 1: STEP 2B is added in DCI size alignment for differentiating DCI size of Format 0_1 and Format 1_1 with DCI size of Format 0_2 and Format 1_2:

- If the size of DCI Format 0_2 monitored in a UE-specific search space equals that of a DCI Format 0_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 0_1.

- If the size of DCI Format 1_2 monitored in a UE-specific search space equals that of a DCI Format 1_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 1_1.

Text Proposal for 38.212

--------------------------------------------------------- Start of TP -----------------------------------------

- 7.3.1.0	DCI size alignment

……

STEP 2B:

- If the size of DCI Format 0_2 monitored in a UE-specific search space equals that of a DCI Format 0_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 0_1.

- If the size of DCI Format 1_2 monitored in a UE-specific search space equals that of a DCI Format 1_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 1_1

……

Step 4:

-	Otherwise

Step 4A:

-	Remove the padding bit (if any) introduced in step 2 and step 2A and step 2B above.

……

The UE is not expected to handle a configuration that, after applying the above steps, results in

-	the total number of different DCI sizes configured to monitor is more than 4 for the cell; or

-	the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_2 in another UE-specific search space; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_2 in another UE-specific search space.

-	the size of DCI format 0_2 in a UE-specific search space is equal to DCI format 0_1 in the same or another UE-specific search space; or

-	the size of DCI format 1_2 in a UE-specific search space is equal to DCI format 1_1 in the same or another UE-specific search space.

--------------------------------------------------------------------------------------------End of TP ------------------------------------------------------------

  - Field size alignment

In RAN1#99 meeting, the WA was achieved on the alignment of bit width of the some fields when a DCI format schedule PDSCH corresponding to the two HARQ-ACK codebooks.

It has been agreed that all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks. On the other hand, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK codebook priorities or PUSCH with different priorities.

Considering the PUCCH resources are also separately configured for HARQ-ACK codebooks. The bit width of PRI field needs to be aligned in a DCI format configured to support two HARQ-ACK codebooks. Thus, PRI also should be included in the WA.

The WA should be updated as following:

- When the UE is configured with two HARQ-ACK codebooks and for the case when any of DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

- PRI

Proposal 2: Confirm WA assumption with the following updates:

- When the UE is configured with two HARQ-ACK codebooks and for the case when any of DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

- PRI

  - How to determine the priority for UE without priority indication capability

In RAN1#99 meeting, the following working assumption and agreement are achieved

Working assumption: When a single PDSCH/PUSCH processing timeline is configured in the carrier, at least when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities. - 1-bit field in DCI can be configured as the PHY identification of the priority - No indication of different priorities by DCI formats 0_0/1_0 Agreement : When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities. - This feature is UE optional

Per the working assumption above, when UE is configured with only DCI format 0_1/1_1 or only DCI format 0_2/1_2 in USS, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in the DCI.

According to the agreement, when UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and the UE supports this optional capability, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in any of the DCI formats.

Observation 1: If a UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and supports the capability of dynamic indication of priorities via both DCI format 0_1/1_1 and DCI format 0_2/1_2, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in any of the DCI formats.

When the UE does NOT support this capability, there are following interpretations when the UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2.

- Interpretation 1: If a UE does not have the capability, all DCI formats cannot be used to indicate HARQ-ACK/PUSCH priority.

- Interpretation 2: If a UE does not have the capability, DCI format 0_2/1_2 can still be used to indicate HARQ-ACK/PUSCH priority, while DCI format 0_1/1_1 cannot be used to indicate the HARQ-ACK/PUSCH priority.

Based on the above different interpretations, different alternatives were proposed for UE not capable of supporting dynamic switching of HARQ-ACK/PUSCH priority via both DCI format 0_1/1_1 and 0_2/1_2.

- If a UE is NOT capable of supporting dynamic switching of HARQ-ACK/PUSCH priority via both DCI format 0_1/1_1 and 0_2/1_2, and the UE is configured with DCI format 0_1 / 1_1 and 0_2/1_2,

  - Alt-1 (based on Interpretation 1): The UE is expected to assume fixed priority by DCI format (i.e., low priority for DCI format 0_1/1_1, high priority for DCI format 0_2/1_2).

  - Alt-2 (based on Interpretation 2): The UE is expected to assume low priority for DCI format 0_1/1_1, and to follow the indicated priority (low or high) in the scheduling DCI format for DCI format 0_2/1_2.

For a UE configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 which does not support the capability in the above agreement, the UE is not capable of dynamic indication of priorities of HARQ-ACK/PUSCH via both DCI format 0_1/1_1 and DCI format 0_2/1_2. In such case, the UE is capable of supporting dynamic indication in either DCI format 0_1/1_1 or DCI format 0_2/1_2, since the capability in the agreement should be based on the capability in the working assumption. If a UE supports dynamic indication of priorities in either DCI format 0_1/1_1 or DCI format 0_2/1_2 according to the working assumption, there is no reason to assume that the UE can support dynamic indication of priorities in neither DCI format 0_1/1_1 nor DCI format 0_2/1_2, when the UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 but without the capability of supporting dynamic indication of priorities according to the agreement.

In case of a UE configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 but without the capability of supporting dynamic indication of priorities according to the agreement, one simple solution is that the priority of HARQ-ACK or PUSCH is fixed as low priority if it is scheduled by DCI format 0_1/1_1, while different priorities for HARQ-ACK or PUSCH can be scheduled by the PHY priority indication in DCI format 0_2/1_2.

Proposal 3: If a UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and does not have the capability of dynamic indication of priorities via both DCI format 0_1/1_1 and DCI format 0_2/1_2, the priorities of HARQ-ACK and PUSCH are fixed to low priority for DCI format 0_1/1_1, and are indicated by the priority indication (low or high) in the scheduling DCI for DCI format 0_2/1_2.

- Conclusion

In the contribution, we have some investigations on URLLC PDCCH enhancement, and propose that,

Observation 1: If a UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and supports the capability of dynamic indication of priorities via both DCI format 0_1/1_1 and DCI format 0_2/1_2, the UE can be scheduled with a PDSCH with different HARQ-ACK priorities or a PUSCH with different priorities by the PHY identification of the priority in any of the DCI formats.

Proposal 1: STEP 2B is added in DCI size alignment for differentiating DCI size of Format 0_1 and Format 1_1 with DCI size of Format 0_2 and Format 1_2:

- If the size of DCI Format 0_2 monitored in a UE-specific search space equals that of a DCI Format 0_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 0_1.

- If the size of DCI Format 1_2 monitored in a UE-specific search space equals that of a DCI Format 1_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 1_1

Text Proposal for 38.212

--------------------------------------------------------- Start of TP -----------------------------------------

- 7.3.1.0	DCI size alignment

……

STEP 2B:

- If the size of DCI Format 0_2 monitored in a UE-specific search space equals that of a DCI Format 0_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 0_1.

- If the size of DCI Format 1_2 monitored in a UE-specific search space equals that of a DCI Format 1_1 monitored in a UE-specific search space, one bit of zero padding shall be appended to DCI Format 1_1

……

Step 4:

-	Otherwise

Step 4A:

-	Remove the padding bit (if any) introduced in step 2 and step 2A and step 2B above.

……

The UE is not expected to handle a configuration that, after applying the above steps, results in

-	the total number of different DCI sizes configured to monitor is more than 4 for the cell; or

-	the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_2 in another UE-specific search space; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_2 in another UE-specific search space.

-	the size of DCI format 0_2 in a UE-specific search space is equal to DCI format 0_1 in the same or another UE-specific search space; or

-	the size of DCI format 1_2 in a UE-specific search space is equal to DCI format 1_1 in the same or another UE-specific search space.

--------------------------------------------------------------------------------------------End of TP ------------------------------------------------------------

Proposal2: Confirm WA assumption with the following updates:

- When the UE is configured with two HARQ-ACK codebooks and for the case when any of DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

- PRI

Proposal 3: If a UE is configured with both DCI format 0_1/1_1 and DCI format 0_2/1_2 and does not have the capability of dynamic indication of priorities via both DCI format 0_1/1_1 and DCI format 0_2/1_2, the priorities of HARQ-ACK and PUSCH are fixed to low priority for DCI format 0_1/1_1, and are indicated by the priority indication (low or high) in the scheduling DCI for DCI format 0_2/1_2.

------------------------------------------------------------------------------------------------------------------------------------

4. References

[1] Chairman notes RAN1 #99 meeting

[2] Chairman notes RAN1 #100bis e-meeting