3GPP TSG RAN WG1 #101		R1-2003420

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on UE features for power saving

Agenda Item:	7.2.11.7

Document for:	Discussion and Decision

- Introduction

This contribution addresses our further comments  on Rel-16 power saving UE features. The latest summary of UE power saving UE features is provided in [1].

- Discussion on UE Power saving features

19-1 DRX Adaptation

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |
| --- | --- | --- |

a) Whether or how to handle licensed/unlicensed differentiation for FG 19-1?

The PDCCH-based power saving signal/channel has no big difference to the PDCCH reception, thus no strong need to differentiate licensed/unlicensed.

Proposal 1: for 19-1,

- The PDCCH-based power saving signal/channel has no big difference to the PDCCH reception, thus no strong need to differentiate licensed/unlicensed.

19-2 Cross Slot Scheduling

| 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH |
| --- | --- | --- |

a) Prerequisite feature groups for FG 19-2

This feature is relevant to minimumSchedulingOffsetK0-r16 and minimumSchedulingOffsetK2-r16 . Considering minimumSchedulingOffsetK2-r16 configuration is for PUSCH, and PUSCH is not relevant to dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB configuration, we think it is not necessary to add dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB as  pre-requisite of 19-2.

b) Whether or how to handle licensed/unlicensed differentiation for FG 19-2?

No need to licensed/unlicensed differentiation.

Proposal 2: for 19-2,

- It is not necessary to add pre-requisite for 19-2.

- No need to licensed/unlicensed differentiation for 19-2

19-3 DL MIMO layer per BWP

| 19-3 | Maximum MIMO Layer Adaptation | (1) Support of maximum number of MIMO layer configuration per DL BWP |
| --- | --- | --- |

a) Type for 19-3

This feature is designed to save UE power by reducing baseband processing, reducing RF path and other implementation oriented approach. Furthermore, it is noted maximum MIMO layer configuration is band-specific configuration. Considering potential different RF implementations across different bands it should be per band.

b) Need of FR1/FR2 differentiation for FG 19-3

Considering different RF implementations across different FRx it should be FR1 and FR2 differential.

c) Whether or how to handle licensed/unlicensed differentiation for FG 19-3?

No need to licensed/unlicensed differentiation

Proposal 3: for 19-3,

- 19-3 is based on granularity of per band.

- 19-3 should be FR1 and FR2 differential

- No need to licensed/unlicensed differentiation

19-4a UE assistance information

| 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |
| --- | --- | --- |

a) Whether or how to handle licensed/unlicensed differentiation for FG 19-4a

The 19-4a is relevant to cross-slot scheduling and is NOT licensed/unlicensed differentiation similar to 19-2.

Proposal 4: for 19-4a,

- No need to licensed/unlicensed differentiation

- Conclusion

The following is proposed for power saving  features,

Proposal 1: for 19-1,

- The PDCCH-based power saving signal/channel has no big difference to the PDCCH reception, thus no strong need to differentiate licensed/unlicensed.

Proposal 2: for 19-2,

- It is not necessary to add pre-requisite for 19-2.

- No need to licensed/unlicensed differentiation for 19-2

Proposal 3: for 19-3,

- 19-3 is based on granularity of per band.

- 19-3 should be FR1 and FR2 differential

- No need to licensed/unlicensed differentiation

Proposal 4: for 19-4a,

- No need to licensed/unlicensed differentiation

- References

- R1-2003195, “Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — UE Power Saving Aspects”,	Moderator (AT&T)