3GPP TSG RAN WG1 #100	                                             R1-2003422

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on UE features for TEI 14-7

Agenda Item:	7.2.11.12

Document for:	Discussion and Decision

- Introdution

Following UE feature is yet to be agreed. Majority of companies expressed support to keep this feature in post RAN1#100b-e email discussion.

| 14. NR TEI | [14-7] | New capability for beamSwitchTiming values of 224 and 336 | [48 is used as the beam switching threshold for Ues reporting 224 or 336 When using sym224 and sym336, beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI.] | TBD [2-28] | Yes | N/A |  | Per band | N/A | N/A (FR2 only) | N/A | FFS: relationship with beamSwitchTiming for FG2-28 Agreements: ・48 is used as the beam switching threshold for Ues reporting 224 or 336 When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

We discuss potential implications if RAN1 agrees to include this UE feature.

- Discussion

In [1], we discussed 2 options to handle this issue.

If RAN1 agrees to introduce this UE feature, relevant changes to 38.331 and 38.306 are required, an LS should be sent to RAN2 informing the agreement. It is clear that Rel-15 UE will not report beamSwitchTiming values of 224 and 336 since the UE behavior in 38.214 is not defined.

However, with introduction of this UE feature, a Rel-16 UE may report Rel-15 capability beamSwitchTiming as one of the values of {14, 28, 48} and Rel-16 capability e.g. beamSwitchTiming-r16 as one the values of {224, 336}. In this scenario when DCI triggering of aperiodic CSI-RS resource configured with repetition ‘ON’ the Rel-16 beam switch timing applies. For all other cases of DCI triggering aperiodic CSI-RS resource the Rel-15 beam switch timing applies.

- Conclusion:

If RAN1 agrees to introduce the UE feature of “New capability for beamSwitchTiming values of 224 and 336”, an LS should be sent to RAN2 informing the RAN1 agreement.

- References

- R1-2001724, “Discussion on UE TEI feature 14-7”, vivo, RAN1#100bis emeeting