3GPP TSG RAN WG1 #101		R1-2003434

e-Meeting, May 25th – June 5th, 2020

Source:	vivo, Guangdong Genius

Title:	RRM relaxation for Reduced Capability NR devices

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introductions

In 3GPP TSG RAN Meeting #86, a new SID on support of reduced capability NR devices (RedCap) was agreed [1]. This contribution discusses the RRM relaxation for RedCap UEs.

- RRM relax for RedCap

As discussed in [2], idle mode power saving enhancements are critical for improving the battery life of RedCap UEs. There are two technical enhancements for idle mode can be considered. One is paging enhancement which will be discussed in R17 power saving WI. The other potential enhancement scheme is RRM relaxation as in RedCap SI objective.

In Rel-16, RRM relaxation for neighboring cells (both inter-/intra-frequency) are specified, while no relaxation was allowed for serving cell RRM. Actually, the RRM measurement relaxation for serving cell has been discussed and studied [3] in Rel-16, but not been specified due to limited scope. However, in most of RedCap use case (e.g. industrial sensors, and video surveillance), the UEs are stationary and have high SINR. In these use cases, the measurement relaxation for serving cell can be further considered for power saving. In the following, we present our views based on our evaluation of serving cell RRM relaxation in idle mode. The Rel-16 UE power models as in TR38.840 are reused in the evaluation. Furthermore, high SINR scenario and stationary UEs are assumed, thus the RRM measurements for both intra-frequency and inter-frequency neighboring cells are not considered.

  - Evaluation of serving cell RRM relaxation in idle mode

- Simulation Assumptions:

    - Paging rate = wake-up signal/channel (WUS) paging rate = 10%, refers to that there is 10% possibility for WUS indicating the UE to wake up and receive paging PDSCH.

    - The time-domain offset of SSB relative to PO is 10ms

    - No time gap between WUS and SSB

    - 𝑃WUS= 𝑃PO = 𝑃SSB =50 power units/slot, where 𝑃WUS, 𝑃PO and 𝑃SSB represent the relative power of WUS, PO, SSB respectively.

      - Scaled to a 20MHz receiving bandwidth from the 100MHz power model in TR38.840

    - TWUS= TSSB = 2ms, where TWUS and TSSB denote the duration of WUS and SSB.

    - One SSB per DRX cycle (paging cycle) is processed in high SINR case

- Simulation cases:

- Rel-15 paining monitoring without WUS

    - Case1: No serving cell RRM relax is assumed.

    - Case2: 4x serving cell RRM relax, i.e. only one SSB is measured every four paging cycles.

- WUS for paging monitoring

    - Case3: No serving cell RRM relax is assumed.

    - Case4: 4x serving cell RRM relax, i.e. only one SSB is measured every four paging cycles.

For Case 1, UE needs to receive SSB and monitor PO per paging cycle. For Case 2, with RRM relaxation, UE only perform SSB measurement every four paging cycles. As shown in Figure 1, in the last three paging cycles, power consumption is saved by skipping 75% of SSB processing and longer deep sleep duration appears.

Case 3 and Case 4 take WUS into account. Because the WUS paging rate is 10%, we only consider the case that WUS indicates UE not to receive PO. With RRM relaxation, only one SSB is measured every four paging cycles for Case 4. Longer deep sleep will take place by the reduced SSB measurement.

Figure 1 Illustration for four evaluation cases

According to the simulation assumption, the simulation results are given in Table 1. Case 2 can achieve 13.4% power saving gain over Case 1 thanks to the RRM relaxation. When WUS is configured, Case 4 reduces 3.6% power consumption over Case 3. Therefore, it is beneficial to support serving cell RRM relaxation for high SINR RedCap UE in idle mode.

Table 1 Simulation results of RRM relax in high SINR case

| Cases |  | Average relative power per slot | Power saving gain |
| --- | --- | --- | --- |
| w/o WUS | Case 1 | 1.6975 | 13.4% (case 2 over case 1) |
|  | Case 2 | 1.4709 |  |
| w/ WUS | Case 3 | 1.5367 | 3.6% (case 4 over case 3) |
|  | Case 4 | 1.4814 |  |

Observation 1: 13.4% power saving gain can be observed if serving cell RRM relaxation is introduced for high SINR UE.

Compared with Case1, for case 2, UE does not need to receive SSB every paging cycle, and the power saving gain is brought by skipping (N-1) SSB receptions every N paging cycle. Typically, UE receives SSB for synchronization to guarantee the reliable reception of paging PDCCH and PDSCH. However, the time drift caused by local oscillator is quite slow so that UE may still maintain good timing even if SSB receptions are skipped in several paging cycles.

Based on the time drift model as agreed in LTE eMTC/NB-IOT [4], we evaluated the time drift caused by oscillator after a number of DRX cycles, as shown in Table 2. It is shown feasible for UE to process SSB every 5.12 seconds in order to maintaining good time synchronization, if we assume 1/3 CP timing drift is acceptable.

Table 2 Time drift due to oscillator with 5ppm frequency error

| Sync periodicity(s) | 0.32 | 0.64 | 1.28 | 2.56 | 5.12 | 1/3 CP_length (30kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Time drift(s) | 2.56×10-9 | 1.02×10-8 | 4.10×10-8 | 1.64×10-7 | 6.55×10-7 | 7.81×10-7 |

To investigate the demodulation impacts due to 1/3 CP timing drift, we performed the link-level evaluation as the following.

In the evolution, we assume the receiver intercepts the time domain signal 1/3 CP ahead of the OFDM signal, as shown in Figure 2. Thus, even if UE suffers from time drift up to 1/3 CP length, the starting time of intercepted signal is still within CP duration, UE can still decode the DL transmission, e.g. paging PDCCH and paging PDSCH. We evaluated the performance of PDCCH and PDSCH with different starting time of intercepted signal, the evaluation assumptions are given in Table A-1 and A-2 in appendix, and the evaluation results are shown in Figure 3 and Figure 4.The results prove that a 1/3 CP timing drift is acceptable for paging reception.

Figure 2 Illustration of time domain signal interception for FFT transform at receiver

Figure 3 Performance of PDCCH with different starting time of intercepted signal

Figure 4 Performance of PDSCH with different starting time of intercepted signal

Note that, since NB-IoT UEs are low cost devices, the time drift caused by oscillator using the model in NB-IoT can be considered as the worst cases for Redcap UEs. Even if considering the worst case, UE are still allowed to receive SSB for synchronization every 5.12s with limited performance loss. Since the RRM relaxation can be applied to high SINR UEs, the performance loss due to sub-optimal timing is negligible.

Observation 2: It is feasible for RedCap UEs to process SSBs once per multiple paging cycles, assuming 1/3 CP timing drift tolerance and NB-IOT/eMTC timing drift model, it is possible for UE to process SSBs once per 5.12 seconds.

Proposal 1: Serving cell RRM relaxation for high SINR UE in idle state can be introduced for RedCap UE.

- Conclusion

In this contribution, we discuss the serving cell RRM relaxation for RedCap NR devices, and have the following observations and proposals:

Observation 1: 13.4% power saving gain can be observed if serving cell RRM relaxation is introduced for high SINR UE.

Observation 2: It is feasible for RedCap UEs to process SSBs once per multiple paging cycles, assuming 1/3 CP timing drift tolerance and NB-IOT/eMTC timing drift model, it is possible for UE to process SSBs once per 5.12 seconds.

Proposal 1: Serving cell RRM relaxation for high SINR UE in idle state can be introduced for RedCap UE.

References

- RP-193238, “New SID on support of reduced capability NR devices”, Ericsson, 3GPP TSG RAN Meeting #86

- R1-2003431, “Capability and complexity reduction for Reduced Capability NR devices”, vivo, Guangdong Genius, 3GPP TSG RAN WG1 #101.

- 3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR (Release 16)”.

- R1-1714993, “Assumptions for NB-IoT power consumption for power saving signal or channel”, RAN1#90, Ericsson.

Appendix: Evaluation assumptions for paging PDCCH and PDSCH

Table A-1 Evaluation assumptions for paging PDCCH

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C Low |
| Delay scaling | 100ns |
| UE speed | 3km/h |
| Subcarrier spacing | 30 kHz |
| Antenna configuration | 2T,2R |
| Channel estimation | Real MMSE |
| DCI size | 39 bits |
| CORESET resource | 48 RBs and 2 symbols |
| REG bundle size | 6 |
| Interleaver size | 2 |

Table A-2 Evaluation assumptions for paging PDSCH

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C Low |
| Delay scaling | 100ns |
| UE speed | 3km/h |
| Subcarrier spacing | 30 kHz |
| Antenna configuration | 2T,2R |
| Channel estimation | Real MMSE |
| Modulation | QPSK |
| Code rate | 1/3 or 2/3 |
| Data allocation | 12 symbols per slot, 12 RBs allocated |
| DMRS configuration | single symbol, 1 front-loaded+1 additional |