3GPP TSG-RAN WG1 Meeting #101	Tdoc R1-2003444

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.5.6

Source:	Ericsson

Title:	Remaining Issue of Enhancements to UL Configured Grant Transmission for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

This document is for the purposes of NR URLLC maintenance under the configured grant enhancement agenda item. In RAN1#100bis-e, we had the following agreement.

Agreements [1]: adopt following TP to TS 38.213 section 9.

9 UE procedure for reporting control information <Unchanged text is omitted> A PUSCH or a PUCCH, including repetitions if any, can be of priority index 0 or of priority index 1. If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0. For a configured grant PUSCH the priority index is determined based on the higher layer parameter priority, if provided. The priority of a PUCCH carrying HARQ-ACK feedback corresponding to SPS PDSCH reception or SPS PDSCH release is determined based on higher layer parameter harq-CodebookID, if provided. If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. <Unchanged text is omitted>

In this contribution, we discuss the remaining issues on UL CG enhancements.

# 2	Discussion

## 2.1 Priority Indication for Configured Grant

In the previous meeting, there was an agreement that the priority of configured grant is indicated by higher layer parameter priority. The term higher layer parameter is omitted in change request (CR) specification as stated as the follows.

## In CR 38.213 Section 9 stated that:

# 9	UE procedure for reporting control information

## *** Unchanged text is omitted ***

A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from priority, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index for harq-CodebookID, if provided. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0.

Following previous agreement, change text to “UE determines a priority index from higher layer parameter priority”, reflecting the that the parameter comes from higher layers.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Following previous agreement, change text to “UE determines a priority index from higher layer parameter priority”, reflecting the that the parameter comes from higher layers.

# References

[1] R1-2001796 Summary of email discussion [100bis-e-NR-L1enh_URLLC_eCG-02] on remaining issues on Enhancements to UL Configured Grant Transmission for NR URLLC, Vivo.