3GPP TSG-RAN WG1 Meeting #101	Tdoc R1-2003446

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.5

Source:	Ericsson

Title:	On UE Features for URLLC and IIoT

Document for:	Discussion, Decision

# 1	Introduction

Currently in RAN1, UE feature of NR is intensively discussed for Rel-16. After RAN1#101 e-meeting, R1-2003199 and R1-2003200 summarizes the outcome of NR UE feature discussion for URLLC and IIoT, respectively.

In this contribution, we further discuss several feature groups for NR_L1enh_URLLC and NR_IIoT, using R1-2003199 and R1-2003200 as the basis.

# 2	NR_L1enh_URLLC

## 2.1	Potential FG for Overlapping PUCCH

In last RAN1 e-meeting #100-bis, following agreements were made.

Agreements:

- If a UE is scheduled with a first PDSCH and a second PDSCH which is starting later than the first PDSCH on a given serving cell, the corresponding PUCCHs carrying HARQ-ACK with different priorities can overlap in time.

  - FFS: For supporting this feature, a new FG, separate from FG 12-1, will be introduced.

  - FFS: The PUCCH associated with the second PDSCH cannot be scheduled for transmission at or earlier than PUCCH associated with the first PDSCH.

Regarding whether a new FG other than FG 12-1 is needed, one needs to check whether a UE implements FG 12-1 would also implement the intended new FG. The potential new FG is to support PUCCHs carrying HARQ-ACK with different priorities, and it is a sub-case of “overlapping channel/signals with two priority levels” covered by FG 12-1 (see below). Specifically, component 3) of FG 12-1 fully covers the case of PUCCHs carrying HARQ-ACK with different priorities. Thus there is no need to introduce a new FG beyond FG 12-1.

| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) - [Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format] - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission |
| --- | --- | --- | --- |

- Do not introduce a new FG beyond FG 12-1 for overlapping PUCCHs carrying HARQ-ACK with different priorities.

## 2.2	Add FG 11-2b for Mixed Rel-15 and Rel-16 Monitoring Capabilities

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 11-2b | Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells | [Support Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells] - Capability on the number of CCs with Rel-15 PDCCH monitoring capability - Capability on the number of CCs with Rel-16 PDCCH monitoring capability | 11-2 |

In RAN1#100bis, further agreements have been made for mixed Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells. It is necessary to introduce FG 11-2b to reflect the RAN1 agreements.

Agreements: For the case with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells (i.e. case 3), UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) as UE capability.  If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16), - gNB configure which combination for UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

- Introduce FG 11-2b for separate capabilities for Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells.

- Do not introduce separate capabilities for mixed Rel-16 capability with Rel-15 PDCCH monitoring capability FG 3-1, FG 3-2, FG 3-5b on different serving cells.

## 2.3	FG 11-3

| 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | - Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. - Supported sub-slot configuration - [Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols] |
| --- | --- | --- |

Component 3) of feature group 11-3 should not be included. UE performs the sub-slot based HARQ-ACK transmission according to RRC configuration. There has never been any RAN1 agreement about the concept of combinations (A, B), and there is no need to introduce such combinations.

- Delete component 3 of FG 11-3 and the related text in “Note” column.

## 2.4	FG 11-4, 11-4a

| 11-4 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based + sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE | - Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. - Supports separate PUCCH configuration for different HARQ-ACK codebooks - Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. - [Supports a DCI format (from the formats 1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured per BWP] - Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. - [Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] |
| --- | --- | --- |
| 11-4a | Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE | - Supports two sub-slot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. - Supports separate PUCCH configuration for different HARQ-ACK codebooks - Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. - [Supports a DCI format (from the formats /1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP] - Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. - [Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] |

For FG 11-4 and 11-4a, component 6) (in bracket) should not be introduced. The slot / sub-slot configuration of HARQ-ACK codebook will define the maximum number of PUCCH transmissions for HARQ-ACK within a slot. There is no need to introduce component 6).

- For FG 11-4 and 11-4a, do not introduce a component for “Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot”.

## 2.6	FG 11-9

| 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | - Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations - [Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell] - [Supported maximum number of configured/active configured grant configurations across all serving cells] |
| --- | --- | --- |

For FG 11-9, there is a question if component 2) and 3) should be introduced. RAN1 has agreed that the maximum number of UL CG configurations per BWP of a serving cell is 12. Thus there is no clear need for the UE to additionally report components 2) and 3).

If it’s proven that FG 11-9 requires components 2), then the set of possible values should be limited. For example, for 2), supported maximum number of configured grant configurations in a BWP of a serving cell can be selected from {1, 2, 4, 8, 12}.

If it’s necessary to introduce component 3), it is reasonable to allow up to twice the number of UL CG configurations per cell group. This is similar to ‘multipleConfiguredGrants’ in Rel-15.

Preferably component 2) and 3) of FG 11-9 are not introduced.

If component 2) of FG 11-9 is introduced, the supported maximum number of configured grant configurations in a BWP of a serving cell is selected from {1, 2, 4, 8, 12}.

If Component 3) of FG 11-9 is introduced, component 3) is updated to: “3) Supported maximum number of configured grant configurations across all serving cells in a cell group is 24.”

# 3	NR_IIOT

## 3.1	FG [12-1a] and [11-4b]

| [11-4b] | [DL priority indication in DCI with mixed DCI formats] | [DL priority indication in DCI with mixed DCI formats] |
| --- | --- | --- |
| [12-1a] | [UL priority indication in DCI with mixed DCI formats] | [UL priority indication in DCI with DCI format 0_1 and 0_2] |

Currently FG [12-1a] and [11-4b] are still under discussion in RAN1. If it is eventually agreed that such capability reporting should be introduced, these FG should be combined into one under [12-1a].

If FG [12-1a] and [11-4b] are to be introduced, combine them into one FG [12-1a].

## 3.2	Other

Currently, in [2], value for the following columns are put in brackets and still not decided.

- Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)

- Need of FDD/TDD differentiation

- Need of FR1/FR2 differentiation

- Capability interpretation for mixture of FDD/TDD and/or FR1/FR2

In our view, the brackets should be removed.

For IIoT feature groups, set Type to ‘Per UE’.

For IIoT feature groups, no need of FDD/TDD differentiation.

For IIoT feature groups, no need of FR1/FR2 differentiation.

For IIoT feature groups, set ‘Capability interpretation for mixture of FDD/TDD and/or FR1/FR2’ to ‘N/A’.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not introduce a new FG beyond FG 12-1 for overlapping PUCCHs carrying HARQ-ACK with different priorities.

Proposal 2	Introduce FG 11-2b for separate capabilities for Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells.

Proposal 3	Do not introduce separate capabilities for mixed Rel-16 capability with Rel-15 PDCCH monitoring capability FG 3-1, FG 3-2, FG 3-5b on different serving cells.

Proposal 4	Delete component 3 of FG 11-3 and the related text in “Note” column.

Proposal 5	For FG 11-4 and 11-4a, do not introduce a component for “Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot”.

Proposal 6	Preferably component 2) and 3) of FG 11-9 are not introduced.

Proposal 7	If component 2) of FG 11-9 is introduced, the supported maximum number of configured grant configurations in a BWP of a serving cell is selected from {1, 2, 4, 8, 12}.

Proposal 8	If Component 3) of FG 11-9 is introduced, component 3) is updated to: “3) Supported maximum number of configured grant configurations across all serving cells in a cell group is 24.”

Proposal 9	If FG [12-1a] and [11-4b] are to be introduced, combine them into one FG [12-1a].

Proposal 10	For IIoT feature groups, set Type to ‘Per UE’.

Proposal 11	For IIoT feature groups, no need of FDD/TDD differentiation.

Proposal 12	For IIoT feature groups, no need of FR1/FR2 differentiation.

Proposal 13	For IIoT feature groups, set ‘Capability interpretation for mixture of FDD/TDD and/or FR1/FR2’ to ‘N/A’.

# References

R1-2003199, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_L1enh_URLLC,” 3GPP RAN1#100bis-e.

R1-2003200, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_IIoT,” 3GPP RAN1#100bis-e.