3GPP TSG RAN WG1 meeting #101   	                             R1-2003486

e-Meeting, May 25th – June 5th, 2020

Title: 	Remaining issues on WUS PDCCH

Source: 	ZTE

Agenda Item:	7.2.7.1

Document for:	Discussion and Decision

Introduction

The agreements on Rel-16 UE power saving in RAN1#100b-e were captured in [1]. The remaining issues on WUS PDCCH are discussed in this contribution.

Remaining issues of WUS PDCCH

To reduce power consumption of RRC connected mode UE, DCI format 2_6 (WUS PDCCH) was introduced. The functionality carried by WUS PDCCH includes wake-up indication and SCell dormancy indication.

L1 procedure of DCI format 2_6

In RAN1#100b-e meeting, the L1 procedure of of DCI format 2_6 detection was agreed as below[1].

Agreements: - L1 procedure of DCI format 2_6 detection - Successful decoding of DCI format 2_6 - L1 sends a positive indication to MAC when the value of wakeup indication bit is “1” - L1 sends a negative indication to MAC when the value of wakeup indication bit is “0” - Miss-detection - all CRC checks fails on DCI format 2_6 - L1 does not send any indication to MAC - All invalid monitoring occasions – following legacy behavior to wakeup - L1 sends a positive indication to MAC

The CR on TS 38.213 corresponding to the above agreements on UE power saving is shown as follows[2].

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331] - a PS-RNTI for DCI format 2_6 by ps-RNTI - a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1 - a payload size for DCI format 2_6 by SizeDCI_2-6 - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 <text omitted> If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [14, TS 38.321] for the next long DRX cycle. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report a value of the Wake-up indication bit to higher layers for the next long DRX cycle. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

According to the above CR, the interpretation of the value of wake-up indication is ambiguous, which would result in different understanding between network and UE. For example, when network indicate a value of ‘1’ via DCI format 2_6 , it is unclear for UE whether it denotes wake-up indication or no-wake-up indication.

Observation 1: In the CR on TS 38.213, the interpretation of the value of wake-up indication is ambiguous, which would result in different understanding between network and UE.

Moreover, it is specified in TS 38.321[3] that “if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213, start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe ”. As the PHY spec is not expected to specify UE behavior of starting the drx-onDurationTimer, the reported indication of wake-up or no-wake-up to higher layer should be clarified to avoid ambiguity.

Therefore, the interpretation of the value of the wake-up indication should be clarified in TS 38.213.

In addition, it may be unnecessary to specify UE to report the value of the wake-up indication from the physical layer to its higher layers. It is enough to specify that UE reports the wake-up indication or no-wake-up indication from the physical layer to the higher layers.

Proposal 1: The following TP of L1 procedure of the detection of DCI format 2_6 is proposed.

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331] - a PS-RNTI for DCI format 2_6 by ps-RNTI - a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1 - a payload size for DCI format 2_6 by SizeDCI_2-6 - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6, - the UE sends the no-Wake-up indication to higher layers for the next long DRX cycle when a value of the Wake-up indication bit is '0', and - the UE sends the Wake-up indication to higher layers for the next long DRX cycle when a value of the Wake-up indication bit is '1' <text omitted> If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report the information of the Wake-up indication to higher layers for the next long DRX cycle. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE reports the Wake-up indication to higher layers for the next long DRX cycle.

Periodic CSI/L1-RSRP report

In RAN1#100b-e, the description of periodic CSI/L1-RSRP report on Clause 5.1.6.1 and 5.2.2.5 of TS 38.214 has been corrected in[4].

Meanwhile, in RAN2#109b-e meeting, the agreements on the periodic CSI/L1-RSRP report have been reached as follows[5].

Agreements

1	RAN2 confirms that the flags ps-TransmitPeriodicL1-RSRP and ps-TransmitPeriodicCSI are defined per cell group

2	The flags ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP are independent, and it is possible to control UE to report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

3	The flag ps-TransmitPeriodicCSI is renamed to ps-TransmitOtherPeriodicCSI

Therefore, the name of flag ps-TransmitPeriodicCSI in Clause 5.1.6.1 and 5.2.2.5 of TS 38.214 needs to be updated.

Proposal 2: Adopt the following TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214.

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text>

5.2.2.5 CSI reference resource definition <omitted text> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

Conclusion

In this contribution, we discuss the remaining issues on WUS PDCCH and have the following observation and proposals.

Observation 1: In the CR on TS 38.213, the interpretation of the value of wake-up indication is ambiguous, which would result in different understanding between network and UE.

Proposal 1: The following TP of L1 procedure of the detection of DCI format 2_6 is proposed.

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331] - a PS-RNTI for DCI format 2_6 by ps-RNTI - a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1 - a payload size for DCI format 2_6 by SizeDCI_2-6 - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6, - the UE sends the no-Wake-up indication to higher layers for the next long DRX cycle when a value of the Wake-up indication bit is '0', and - the UE sends the Wake-up indication to higher layers for the next long DRX cycle when a value of the Wake-up indication bit is '1' <text omitted> If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report the information of the Wake-up indication to higher layers for the next long DRX cycle. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE reports the Wake-up indication to higher layers for the next long DRX cycle.

Proposal 2: Adopt the following TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214.

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text>

5.2.2.5 CSI reference resource definition <omitted text> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

Reference

- RAN1#100b-e, “RAN1 Chairman’s Notes”.

- R1-2003177, “Corrections on UE power savings” Samsung.

- 3GPP TS 38.321 V16.0.0 (2020-03), “NR; Medium Access Control (MAC) protocol specification(Release 16)”.

- R1-2003138, “Corrections of cross-slot scheduling restriction”, Nokia.

- R1-2003260, LS on RAN2 DCP Open Issues, InterDigital.

- 3GPP TS 38.213 V16.1.0 (2020-03), “NR; Physical layer procedures for control(Release 16)”.

- 3GPP TS 38.214 V16.1.0 (2020-03), “NR; Physical layer procedures for data(Release 16)”.