3GPP TSG RAN WG1 Meeting #101-e	R1-2003500

E-meeting, May 25 – June 5, 2020

Agenda Item:	7.2.4.7

Source:	Huawei, HiSilicon

Title:	Remaining details of NR Uu control for LTE sidelink

Document for:	Discussion and Decision

## Introduction

Regarding power control for NR SL, the following agreements were achieved in RAN1#97:

Agreements:

- For the SL open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss.

- When the SL open-loop power control is configured to use both DL pathloss and SL pathloss,

  - The minimum of the power values given by open-loop power control based on DL pathloss and the open-loop power control based on SL pathloss is taken.

    - (Working assumption) P0 and alpha values are separately (pre-)configured for DL pathloss and SL pathloss.

During RAN1#100bis-e meeting, it was proposed to clarify the power control for LTE sidelink when UE is controlled by NR Uu [1]. Basically, the clarification is that the pathloss used for LTE sidelink power control should be derived from NR Uu DL for the shared carrier to manage the interference to gNB.

Given the mechanism of pathloss derivation via NR Uu is quite different from that via LTE Uu, so it is worth to clarify it when the LTE SL is controlled by NR Uu. This contribution discusses how to clarify it.

## Discussion on the TP

The paper [1] proposes to include the TP (as shown in the Appendix) to clause 14.1.1.5 in TS 36.213 [2] regarding PSSCH power control. Actually, the pathloss for PSCCH power control also needs clarification. However, we think the change should be incorporated into TS 38.213 instead of TS 36.213.

- No need to worry the legacy LTE sidelink may not work in NR Uu control if the change is made in TS 38.213. This was actually also the reason we agreed on changing TS 38.213 to clarify the LTE sidelink transmission timing in the last e-meeting.

- We now have a precedent of changing TS 38.213 as in Clause 16.6 [3] to clarify the procedure for LTE sidelink transmission. Following the same logic makes better specification readability of TS 36.213 and TS 38.213.

- Easy specification of any future updates to cross-RAT control is desirable. Whatever change is needed for LTE sidelink transmission when it is controlled by NR Uu can be easily located in clause 16.6 in TS 38.213

So we have the text proposal to change TS 38.213 Clause 16.6 as follows:

- ----------------------------------------------- Start of text proposal for TS 38.213-----------------------------------------------

## 16.6	UE procedure for LTE sidelink transmission

If the UE detects a DCI format 3_1 with CRC scrambled by SL-L-CS-RNTI in slot n, the DCI format 3_1 activates or releases an LTE sidelink SPS configuration that is indicated by a SL SPS configuration index field [5, TS38.212]. If the DCI format 3_1 activates an SL SPS configuration, the UE procedure for transmitting a PSCCH and a PSSCH is as described in [13, TS36.213] except that a transmission starts no earlier than TDCI-NTA2×TC+X+4+m  ms, where TDCI is a start time of slot n, NTA and TC are defined in [4, TS 38.211], X is a value indicated by a Timing offset field in DCI format 3_1, and m is a value indicated by SL index field in DCI format 3_1 if the SL index field is present; otherwise, m=0.

If the UE is scheduled by DCI format 3_1, for sidelink transmission mode 3, the UE procedures for PSSCH and PSCCH power control are as described in [13, TS36.213] except that the pathloss is given by PL=PLb,f,c(qd) as described in clause 7.1.1 and  and are provided by higher layer parameters p0SL-V2V and alphaSL-V2V.

- ---------------------------------------------- End of text proposal for TS 38.213-------------------------------------------------

Proposal: Adopt the above TP to TS 38.213 section 16.6.

# Conclusions

This contribution discusses the clarification for LTE sidelink transmission regarding power control for PSSCH and PSCCH when UE is controlled by NR Uu and the corresponding TP is provided. The proposal is given as follows:

Proposal: Adopt the above TP to TS 38.213 section 16.6.

# References

R1-2002240, NR Uu controlling LTE sidelink transmissions, Ericsson, RAN1#100bis-e, April 20-30, 2020.

TS 36.213 Physical layer procedures (Release 16) V16.0.0.

TS 38.213 Physical layer procedures for control (Release 16) v16.1.0.

- ----------------------------------------------- Start of change in [1] to TS 36.213 from [1] -------------------------------------------------

#### 14.1.1.5	UE procedure for PSSCH power control

### < Unchanged parts are omitted >

For sidelink transmission mode 3, the UE transmit power  for PSSCH transmission is given by

[dBm] ,

- where  is defined in [6], and is the bandwidth of the PSSCH resource assignment expressed in number of resource blocks and  where is defined in Subclause 5.1.1.1.  and  are provided by higher layer parameters p0SL-V2V and alphaSL-V2V, respectively and that are associated with the corresponding PSSCH resource configuration. If a UE is scheduled by DCI Format 3_1  defined in 38.212, PL=PLb,f,c(qd) as described in Clause 7.1.1 of 38.213 and   and  are provided by higher layer parameters p0SL-V2V and alphaSL-V2V, respectively and that are associated with the corresponding PSSCH resource configuration.

- ----------------------------------------------- End of text proposal for TS 36.213-------------------------------------------------