3GPP TSG RAN WG1 Meeting #101-e	R1-2003504

E-meeting, May 25 – June 5, 2020

Agenda Item:	7.2.1.2

Source:	Huawei, HiSilicon

Title:	Corrections on procedure of 2-step RACH

Document for:	Discussion and Decision

Introduction

In RAN1#99, most discussion of 2-step RACH was completed, and the agreements have been incorporated in the 3GPP specifications [1][2][3][4]. In RAN1#100b-e, many corrections have been proposed and related TPs were endorsed [5]. In this contribution, we will provide some further corrections on the procedures of 2-step RACH.

Discussion

LSBs of SFN

As the MsgB window can be longer than 10ms, the UEs transmit MsgA in different SFNs may share the same MsgB-RNTI. If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI, the LSBs of SFN field in the DCI will indicate whether the MsgB is for the UE or not. Therefore, the LSBs of SFN should be further interpreted in TS 38.213.

Proposal 1: To complete the operations of MsgB reception, adopt TP#1 in the Appendix.

Conclusions

We propose to adopt the TP in the Appendix.

- References

3GPP TS 38.211, NR Physical channels and modulation, v16.1.0.

3GPP TS 38.212, NR Multiplexing and channel coding, v16.1.0.

3GPP TS 38.213, Physical layer procedures for control, v16.1.0.

3GPP TS 38.214, Physical layer procedures for data, v16.1.0.

3GPP RAN1 Chairman's Notes, RAN1#100b-e.

- Appendix

- Text proposal #1 for TS 38.213 Clause 8.3:

============================= Unchanged part omitted ===========================

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and the two LSB bits of the SFN corresponding to the PRACH occasion for PRACH transmission, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.3 when the UE detects a RAR UL grant, or

============================= Unchanged part omitted ===========================