3GPP TSG RAN WG1 #101							R1-2003571

e-Meeting, May 25th – June 5th, 2020

______________________________________________________________________Agenda item: 6.2.5.5

Source: LG Electronics

Title: Discussion on Rel-16 LTE UE features for 5G V2X

Document for: Discussion and decision

- Introduction

RAN1 LTE Rel-16 UE feature list was discussed and summarized [1], and this contribution provides discussions on the 5G V2X WI part.

- Discussion

As many of the FGs and their components are the same as those in NR UE feature list, it would be more efficient to discuss LTE UE FGs after stabilizing NR UE FGs first. Some issues specific to the LTE UE FGs are discussed as follows:

Basic feature group

It is noteworthy that the UEs considered in this discussion are NR UEs that support NR sidelink controlled by LTE eNB. So, if a UE reports a FG included in the list, the eNB knows that the UE supports NR sidelink including those defined as the basic feature group in the NR UE FG list. Therefore, the FGs already agreed as the basic FGs in the NR UE FG list do not need to be defined here again; it will be just a repetition of the same information. Additional basic FGs in the LTE UE FG list are not necessary as the cross-RAT sidelink control itself cannot be a basic FG for Rel-16 NR sidelink UEs.

The following FFS point in FG 5-1, 5-2, 5-3, 5-5, 5-12, 5-15 can be automatically decided based on the conclusion for the corresponding FGs in the NR UE feature list; for example, if RAN1 agrees that mode 1 TX is a basic FG for “sidelink in licensed spectrum where gNB is operating on or managing that spectrum,” FG 5-1 will be a basic FG for sidelink in licensed spectrum where eNB is operating on or managing that spectrum.”

FFS: This is the basic FG for sidelink in licensed spectrum where eNB is operating on or managing that spectrum and optional FG otherwise

Proposal 1: The basic FGs defined in the NR UE feature list do not need to be repeated in the LTE UE feature list.

FG 5-10 (eNB type synchronization source for NR sidelink)

As gNB based synchronization is the basic FG in Rel-16 NR sidelink and a UE indicating at least one FG in this list supports LTE Uu, it seems possible to make FG 5-10 as the prerequisite of FG 5-1 and 5-2. In other words, a UE supporting NR sidelink controlled by LTE Uu always supports the eNB based synchronization (in addition to other synchronization included as the basic FG in the NR UE FG list).

Proposal 2: FG 5-10 is a prerequisite of other FGs.

- Conclusions

This contribution discussed the remaining issues in the V2X part of RAN1 LTE UE feature list. The following proposals were made based on the discussion:

Proposal 1: The basic FGs defined in the NR UE feature list do not need to be repeated in the LTE UE feature list.

Proposal 2: FG 5-10 is a prerequisite of other FGs.

References

[1] R1-2003196, “Summary on email discussion [100b-e-LTE-UEFeatures-Remaining,” Moderators (AT&T, NTT DOCOMO, INC.)