3GPP TSG RAN WG1 #101                                      	R1-2003611

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Remaining issues on physical layer structure for NR sidelink

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

Introduction

In RAN1#100[1] and 100bis[2], the main remaining issues on physical layer structure are discussed, the following agreements are achieved:

Agreements: For 2nd SCI overhead in the TBS determination, the actual number of REs occupied by the 2nd SCI is used. Agreements: - For PSFCH overhead in the TBS determination, use the number of PSFCH symbols indicated by SCI. - For PSSCH DMRS overhead in the TBS determination, the reference number of REs occupied by PSSCH DMRS is used, where the reference number of REs is the average number of DMRS REs among (pre-)configured patterns. - For CSI-RS and PT-RS overheads in the TBS determination, a new higher layer parameter, e.g., sl-xOverhead, is introduced per resource pool. Agreements: The MCS table is indicated by 1st SCI, the number of MCS tables is (pre-) configured per resource pool. - 64QAM table is (pre-)configured as default. - Zero, one or two additional can be additionally (pre-)configured. Tables - Using the 256QAM and/or low-SE MCS tables - The number of bits in the 1st SCI for the indication is determined based on the number of MCS tables (pre)-configured for the resource pool - 0, 1, or 2 bits - Over-writing the (pre-)configured MCS table(s) by PC5-RRC is NOT supported - A UE is not required to decode the 2nd SCI or the PSSCH associated with a 1st SCI if the 1st SCI indicates an MCS table that the UE does not support Agreements: For derivation of the set of slots to be included in the resource pool, the baseline is the derivation with bitmap and periodicity based on Subclause 14.1.5 of TS36.213 with the following modifications. - FFS: Periodicity and L_bitmap value - The slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if serving cell timing reference is in use, or DFN 0 otherwise - The following procedure is used. - The set includes all the slots except the following slots: - Slots in which SLSS resource is configured, - (Working assumption) slots not having at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where - X is sl-LengthSymbols - Y is sl-StartSymbol - (Working assumption) reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213 Working assumption - For the number of PRBs for resource pool, allow configuration of all number of PRBs in a SL BWP. FFS until RAN1#100bis-e whether/how to deal with remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size.

In this contribution we continue to discuss the remaining issues on physical layer structure for NR sidelink:

- TBS determination

- Resource pool configuration

- Scrambling sequence generators

- Details on SCI design

- Frequency domain OCC design for PSCCH DMRS

- Initialization of PSCCH DMRS sequence generator

- The sequence generation of PSFCH

- Others

Discussion

TBS determination

It has been agreed that the actual number of REs occupied by the 2nd SCI is used for 2nd SCI overhead in TBS determination, but the circular dependency between 2nd SCI rate matching and TBS still needs to be resolved.

The number of coded modulation symbols for 2nd SCI transmission is determined by the TBS in the current version of TS 38.212. However, TBS determination also needs to be based on the number of REs occupied by 2nd SCI. For rate matching process of 2nd SCI, if 2nd SCI resource determination is not depending on TBS, and the number of modulation symbols for 2nd SCI is unchanged during initial transmission and re-transmissions, using the actual overhead of 2nd SCI for TBS determination will be perfectly reasonable. Besides, there are some issues on the Rate Matching of 2nd SCI in clause 8.4.4 of TS 38.212 which the modulation order of 2nd SCI and PSSCH may be different. The formula of REs calculation for 2nd SCI should be revised in the TP#1. The number of modulation symbols for 2nd SCI can be determined by code rate of PSSCH and the modulation order of the 2nd stage SCI. This will make the rate matching process independent of the TBS.

Proposal 1:  The number of modulation symbols for 2nd SCI are determined by code rate of PSSCH and the modulation order of the 2nd stage SCI, which is independent of the PSSCH TBS.

For PSFCH overhead in the TBS determination, it was agreed that the number of PSFCH symbols are indicated by SCI. If the 2nd SCI RE mapping is independent of TBS determination, it is reasonable to put the bits in 2nd SCI.

Regarding the PSFCH overhead indicated by SCI, there is no need for any dynamic signaling in cases of PSFCH periodicity N=0 and N=1, since PSFCH is always present or is always not present. For other cases of PSFCH periodicity N=2 and N=4, it is up to how many bits in 2nd SCI can be used to indicate the PSFCH overhead. In order to support flexible indication of the reference PSFCH overhead, 2 bits should be introduced to indicate the number of PSFCH symbols as [0,1,2,3]..

Proposal 2: For PSFCH overhead in the TBS determination,  the number of PSFCH symbols is indicated by 2nd SCI.

Proposal 3: PSFCH overhead indication is only enable when PSFCH periodicity N=2 or N=4, and 2 bits are used to indicate the overhead of PSFCH as [0,1,2,3] symbol(s).

Proposal 4: Adopt TP#1and 2 in section 4 for proposal 1&3.

Resource pool configuration

For the resource pool configuration, there are two remaining issues. The first one is the periodicity and L_bitmap value. The second one is how to deal with remaining PRBs if the configured PRBs for resource pool is not integer multiple of sub-channel size.

For the first issue, the periodicity refers to a time duration. The bitmap applies to the slots in this time duration. It is acceptable to reuse the LTE periodicity with a fixed value as 10240ms.

In LTE V2X, the bitmap length is determined with the consideration of traffic periodicity (e.g. typical periodicity value is 100ms for BSM). Then the [10, 20, 30, 40, 50, 60] is introduced for different TDD configuration, and 100ms is used for FDD configuration. Besides that 20ms is also introduced to support pre-crash service(typical periodicity value is 20ms), and it is also a factor of 100ms. 16ms is introduced only for the motivation of removing reserved slots.

For the length of bitmap in NR V2X, due to the flexible in NR Uu, a more flexible bitmap length should be used to support in NR V2X. In NR Uu, the TDD pattern will be repeated at least every 20ms. Considering SCS of 120kHz, there are 160 slots within 20ms. In summary, if the LTE principle is reused, the possible values of the bitmap length can be [1,2,3,…,160]. With the consideration of configuration flexibility, the bitmap length set for resource pool configuration can start from10 as [10,11,12,…,160].

Proposal 5: The resource pool periodicity is fixed as 10240ms, and the L_bitmap of NR sidelink is set to: [10,11,12,…,160].

The number of PRBs configured for a resource pool is preferred to be multiples of number of sub-channels to avoid the resource fragmentation. If the configured PRBs for resource pool are not multiple of number of sub-channels, there will be some remaining PRBs not to be used. In order to fully utilize the frequency resource, the remaining PRBs of the resource pool could be considered as the one sub-channel(indexed as last sub-channel), which has smaller number of PRBs than that of the normal sub-channels. Besides, PSCCH can’t be transmitted on the last sub-channel without the full number of PRBs.

Additionally, smaller sub-channel size is not preferred. The PSCCH duration is up to 3 symbols. If smaller sub-channel size is adopted, the number of REs for PSCCH might not be sufficient to maintain the PSCCH reliability. Introducing smaller sub-channel size should be avoided to maintain the PSCCH quality and prevent unnecessary specification effort.

Proposal 6: The last sub-channel consists of all the remaining PRBs, and PSCCH is not transmitted on the last sub-channel.

Scrambling sequence generators

The scrambling sequence for Rel-16 sidelink physical channel are not well discussed, the remaining details about the scrambling sequence generators for sidelink physical channels are discussed in following sections.

Scrambling sequence initialization for PSCCH

Because the 1st stage SCI transmitted on PSCCH is transparent for all UEs, the initialization of the scrambling sequence generator for PSCCH should be a fixed value. The scrambling sequence generator can be initialized with cinit=1024 at the start of every PSCCH slot, which can avoid potential scrambling sequence collision with PUCCH and PDCCH.

Proposal 7: The scrambling sequence generator can be initialized with cinit=1024 at the start of every PSCCH slot

Scrambling sequence initialization for 2nd stage SCI and PSSCH

It has been agreed that scrambling operation for the 2nd stage SCI is applied separately with PSSCH. In clause 8.3.1.1 of TS38.211 for the scrambling of 2nd stage SCI and data, following scrambling operation is defined:

bqi=bqi+cqi-M(q)mod 2

Where M(q) is different for 2nd stage SCI and data.

For the initialization of the scrambling sequence of 2nd stage SCI and data, we can reuse the same initialization computational formula by reference to the scrambling operation for NR PUSCH:

cinit=nRNTI∙215+nID

For PSSCH, nRNTI can be replaced with NID, and nID should be equal to 1024, where NID=NIDX mod 216 where the quantity NIDX equals the decimal representation of CRC on the PSCCH associated with the PSSCH according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212

Proposal 8: The scrambling sequence generator for PSSCH and 2nd SCI can be initialized with the CRC and scrambling initialization ID of PSCCH.

Proposal 9: Adopt TP#3 in section 4 for proposal 7, 8.

Details on SCI design

For HARQ feedback in groupcast and unicast, when PSFCH resource is (pre-)configured in the resource pool, SCI explicitly indicates whether HARQ feedback is used or not for the corresponding PSSCH transmission. This working assumption should be confirmed. With the consideration of overhead reduction in 1st SCI, we prefer to carry this indication in 2nd SCI. Another aspect is that the actual number of REs occupied by the 2nd SCI is used for TBS determination. If mixed transmission is supported, it is better to use same 2nd SCI format for multiple re-transmission of same TB. From this point of view, the indication of HARQ request should be present both in 2nd SCI format A and B. Besides, 1 bit HARQ type field used to indicate NACK-based or HARQ ACK/NACK based HARQ feedback should also be introduced in SCI format 2-A.

Proposal 10: If mixed transmission is supported, the indication of SL HARQ request is present both in 2nd SCI format A and B,

Proposal 11: 1bit HARQ type field should be introduced in SCI format 2-A

In RAN1 #100, the following options are provided to be down-selected [1]:

Agreements: - Down-select in the next meeting one of the following options - Option 1: There is no separate field in the first stage SCI indicating a resource index for the purpose of backward indication, i.e., backward indication is not supported - Option 2: When periodic reservations are enabled in a resource pool, a separate field of 1 bit in the first stage SCI indicates a resource index for the purpose of backward indication - Option 3: When periodic reservations are enabled in a resource pool, a separate field of ceil(log2(Nmax)) bit in the first stage SCI indicates a resource index for the purpose of backward indication

In the companion contribution, Option 3 is supported for the 1st SCI to indicate the resource index with backward indication when periodic reservation is enabled in the resource pool. The separate field of ceil(log2(Nmax) bits should be used for the relative timing position of the second transmission [3].

Proposal 12: When periodic reservations are enabled in a resource pool, a separate field of ceil(log2(NMAX)) bit in the 1st stage SCI indicates a resource index for the purpose of backward indication

At least 2 bits in 1st stage SCI should be used to indicate 2nd stage SCI formats for forward compatibility, some new 2nd stage SCI formats may be introduced.

Proposal 13: 2 bits in 1st stage SCI should be used to indicate 2nd stage SCI formats

Proposal 14: Adopt TP#4 in section 4 for proposal 2, 10~13.

The sequence generation of PSFCH

In RAN1#99, it has been agreed that:

- CDM between PSFCH transmissions from different UEs in the same PRB is supported as follows:

  - Base sequence is

    - (Pre-)configured per resource pool

The parameter hoppingID_PSFCH is introduced to generate the base sequence of the PSFCH used in the resource pool.

Proposal 15: Modify the sequence generation of PSFCH according to the following TP#5 in section 4.

Others

In this section, some minor typos in clause 8.4.1.2.2 of TS38.211 are corrected as the following TP.

Proposal 16: Modify the sequence generation of PSFCH according to the following TP#6 in section 4.

Conclusion

In this contribution, we provide some proposals on the NR V2X physical structure as follow:

Proposal 1:  The number of modulation symbols for 2nd SCI are determined by code rate of PSSCH and the modulation order of the 2nd stage SCI, which is independent of the PSSCH TBS.

Proposal 2: For PSFCH overhead in the TBS determination,  the number of PSFCH symbols is indicated by 2nd SCI.

Proposal 3: PSFCH overhead indication is only enable when PSFCH periodicity N=2 or N=4, and 2 bits are used to indicate the overhead of PSFCH as [0,1,2,3] symbol(s).

Proposal 4: Adopt TP#1and 2 in section 4 for proposal 1&3.

Proposal 5: The resource pool periodicity is fixed as 10240ms, and the L_bitmap of NR sidelink is set to: [10,11,12,…,160].

Proposal 6: The last sub-channel consists of all the remaining PRBs, and PSCCH is not transmitted on the last sub-channel.

Proposal 7: The scrambling sequence generator can be initialized with cinit=1024 at the start of every PSCCH slot

Proposal 8: The scrambling sequence generator for PSSCH and 2nd SCI can be initialized with the CRC and scrambling initialization ID of PSCCH.

Proposal 9: Adopt TP#3 in section 4 for proposal 7, 8.

Proposal 10: If mixed transmission is supported, the indication of SL HARQ request is present both in 2nd SCI format A and B,

Proposal 11: 1bit HARQ type field should be introduced in SCI format 2-A

Proposal 12: When periodic reservations are enabled in a resource pool, a separate field of ceil(log2(NMAX)) bit in the 1st stage SCI indicates a resource index for the purpose of backward indication

Proposal 13: 2 bits in 1st stage SCI should be used to indicate 2nd stage SCI formats

Proposal 14: Adopt TP#4 in section 4 for proposal 2, 10~13.

Proposal 15: Modify the sequence generation of PSFCH according to the following TP#5 in section 4.

Proposal 16: Modify the sequence generation of PSFCH according to the following TP#6 in section 4.

Text proposals

- TP#1: Text proposal for TS 38.212

8.4.4 Rate Matching For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols for 2nd-stage SCI transmission, decoted as QSCI2', is determined as follows: QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2R∙QmSCI2, αl=l1NsymbolPSSCH-1MscSCI2(l)+γ where - OSCI2 is the number of the 2nd-stage SCI bits - LSCI2 is the number of CRC bits for the 2nd-stage SCI, which is [xxx] bits. - βoffsetSCI2 is indicated in the corresponding 1st-stage SCI. CSL-SCH - MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers; - MscDMRS(l) is the number of subcarriers in OFDM symbol l that carries DMRS, in the PSSCH transmission. - MscPT-RS(l) is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PSSCH transmission. - MscCSI-RS(l) is the number of subcarriers in OFDM symbol l that carries CSI-RS, in the PSSCH transmission. - MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH, in PSSCH transmission and NsymbolPSSCH is the number of allocated symbols for the PSSCH except AGC symbol as defined in [6, TS 38.214]: - MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) - MscCSI-RS(l) - γ is the number of otherwise vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. - Kr is the r-th code block size for SL-SCH of the PSSCH transmission. - α is configured by higher layer parameter sl-Scaling. - R is the target code rate signalled by the MCS field in SCI format 0-1. - QmSCI2 is the modulation order of the 2nd stage SCI; <Unchanged part omitted>

- TP#2: Text proposal for TS 38.214

8.1.3.2 Transport block size determination The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB by - N'RE=NSCRB∙Nsymbsh-NDMRSPRB-NPSFCHPRB-NohPRB, where NSCRB=12 is the number of subcarriers in the frequency domain in a physical resource block, Nsymbsh Nsymbslotis the number of symbols L of the PSSCH allocation according to Clause 8.1.2.1 for scheduled PSSCH, NDMRSPRB is the average number of DMRS REs among (pre-)configured patterns, NPSFCHPRB is the overhead of PSFCH which can be determined by 2nd SCI, and NohPRB is the overhead of CSI-RS and PT-RS which is a new higher layer parameter introduced per resource pool. - A UE determines the total number of REs allocated for PSSCH (NRE) by NRE=min144, N'RE∙nPRB-NPSCCH- N2nd SCI, where nPRB is the total number of allocated PRBs for the UE, NPSCCH is the exact number of REs for PSCCH (including PSCCH DMRS), N2nd SCI=Q'SCI2 is the total number of REs for the 2nd stage SCI as given by Clause 8.4.4 in TS 38.212. - Next, proceed with steps 2-4 as defined in Clause 5.1.3.2 <Unchanged part omitted>

- TP#3: Text proposal for TS 38.211

- 8.3.2.1 Scrambling The block of bits b0,…,b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits b0,…,b(Mbit-1) according to bi=bi+cimod 2 where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with cinit=1024 <Unchanged part omitted> - 8.3.1.1 Scrambling For the single codeword q=0, the block of bits bq0,…,bqMbit(q)-1, where Mbit(q)=Mbit,SCI2(q)+Mbit,data(q) is the number of bits in codeword q transmitted on the physical channel as defined in [4, TS 38.212], shall be scrambled prior to modulation. Scrambling shall be done according to the following pseudo code set i=0 while i<Mbitq if b(q)i=x // SCI placeholder bits b(q)i=b(q)i-2 else b(q)i=b(q)i+cq(i-M(q)) mod 2 end if i = i + 1 end while where the scrambling sequence cq(i) is given by clause 5.2.1 and - The scrambling sequence generator shall be initialized with cinit=NID∙215+1024 - for 0≤i<Mbit,SCI2(q) - M(q)=0 cinit - for Mbit,SCI2(q)≤i< Mbit(q) - M(q)=Mbit,SCI2q cinit where NID=NIDX mod 216 where the quantity NIDX equals the decimal representation of CRC on the PSCCH associated with the PSSCH according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212 <Unchanged part omitted>

- TP#4: Text proposal for TS 38.212

8.3.1.1 SCI format 1-A SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH The following information is transmitted by means of the SCI format 1-A: - Priority – 3 bits as defined in clause 5.4.3.3 of [12, TS 38.287]. - Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.2.2 of [6, TS 38.214]. - Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.2.1 of [6, TS 38.214]. - Backward indication –Ceil(log2(NMAX)) bit in the 1st stage SCI indicates a resource index for the purpose of backward indication - Resource reservation period – log2(NreservPeriod) bits as defined in clause x.x.x of [6, TS 38.214], if higher parameter sl-MultiReserveResource is configured; 0 bit otherwise. - DMRS pattern – [x] bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], if more than one DMRS patterns are configured by higher layer parameter sl-PSSCH-DMRS-TimePattern; 0 bit otherwise. - 2nd-stage SCI format – [x] bits as defined in clause x.x.x of [6, TS 38.214]. - Beta_offset indicator – [2] bits as provided by higher layer parameter sl-BetaOffsets2ndSCI. - Number of DMRS port – 1 bit as defined in Table 8.3.1.1-1. - Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214]. - Additional MCS table indicator – as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl-MCS-Table; 0 bit otherwise. - Reserved – [2 - 4] bits as determined by higher layer parameter sl-NumReservedBits, with value set to zero. <Unchanged part omitted> - 8.4.1.1 SCI format 2-A SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, or when there is no feedback of HARQ-ACK information. The following information is transmitted by means of the SCI format 2-A: - HARQ request – 1 bit used for triggering the HARQ feedback process. - HARQ type – 1 bit used to indicate NACK-based or HARQ ACK/NACK based HARQ feedback - PSFCH overhead – 1 bit used to indicate the overhead of PSFCH in TBS determination - HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213]. - New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213]. - Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214]. - Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214]. - Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214]. - CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214]. - 8.4.1.2 SCI format 2-B SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK. The following information is transmitted by means of the SCI format 2-B: - HARQ request – 1 bit used for triggering the HARQ feedback process. - PSFCH overhead – 1 bit used to indicate the overhead of PSFCH in TBS determination - HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213]. - New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213]. - Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214]. - Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214]. - Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214]. - Zone ID – 12 bits as defined in clause x.x.x of [9, TS 38.331]. - Communication range requirement – 4 bits as defined in clause x.x.x of [9, TS 38.331] <Unchanged part omitted>

- TP#5: Text proposal for TS 38.211

8.3.4.2.1 Sequence generation <Unchanged part omitted> The pseudo-random sequence generator shall be initialized with . <Unchanged part omitted>

- TP#6: Text proposal for TS 38.211

8.4.1.2.2 Mapping to physical resources The UE shall transmit phase-tracking reference signals only in the resource blocks used for the PSSCH, and only if the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used. The PSSCH PT-RS shall be mapped to resource elements according to ak,lpo,μ⋮ak,lpρ-1,μ=βPT-RSWr(p0)(2n+k')⋮r(pυ-1)(2n+k') k=4n+2k'+Δconfiguration type 16n+k'+Δ[configuration type 2] when all the following conditions are fulfilled - l is within the OFDM symbols allocated for the PSSCH transmission; - resource element k,l is not used for sidelink CSI-RS, PSCCH, nor DM-RS associated with PSSCH; - k' and Δ correspond to p0, …, pυ-1 The precoding matrix W is given by clause 8.3.1.4. The quantity βPTRS is an amplitude scaling factor to conform with the transmit power specified in clause XXX of [6, TS 38.214]. The set of time indices l defined relative to the start of the PSSCH allocation is defined by 1. set i=0 and lref=0 2. if any symbol in the interval max lref+i-1LPT-RS+1, lref,…,lref+iLPT-RS overlaps with a symbol used for DM-RS according to clause 8.4.1.1.2 - set i=1 - set lref to the symbol index of the DM-RS symbol - repeat from step 2 as long as lref+iLPT-RS is inside the PSSCH allocation 3. add lref+iLPT-RS to the set of time indices for PT-RS 4. increment i by one 5. repeat from step 2 above as long as lref+iLPT-RS is inside the PSSCH allocation where LPT-RS∈1,2,4 is defined in Table XXXX of [6, TS 38.214]. <Unchanged part omitted>

References

  - 3GPP TSG RAN WG1 Meeting RAN1 #100-e Chairman’s Notes, Mar. 2020.

  - 3GPP TSG RAN WG1 Meeting RAN1 #100bis-e Chairman’s Notes, April. 2020.

  - R1-2003613, “Remaining issues on Mode 2 resource allocation in NR V2X”, CATT

  - R1-1913601, “Agreement of 5G V2X”, November 2019

  - 3GPP TS 38.211, “Physical channels and modulation”, V16.0.0.

  - 3GPP TS 38.212, “Multiplexing and channel coding”, V16.0.0.

  - 3GPP TS 38.213, “Physical layer procedures for control”, V16.0.0.

  - 3GPP TS 38.214, “Physical layer procedures for data”, V16.0.0.