3GPP TSG RAN WG1 #101		R1-2003788

e-Meeting, May 25th – June 5th, 2020

Agenda item:	6.2.5.1

Source: 	Qualcomm Incorporated

Title: 	UE features for NB-IoT

Document for:	Discussion and Decision

# Background

In this document we present our comments to the current NB-IoT feature list (from R1-2003196). The proposed changes are provided with change marks with respect to the current version.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-1 | UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | Rel-15 NWUS | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-2 | UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | 2-1 | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal and group resource alternation. | Per UE | FDD only | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured. | Optional with capability signalling |
|  | 2-3 | Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) | - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) |  | Yes | N/A | UL data transmission will use EDT or connected mode instead of PUR | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 2-4 | PUR with serving cell RSRP for TA validation | - PUR with serving cell RSRP for TA validation | 2-3 | Yes | N/A | PUR will not use serving cell RSRP for TA validation | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. TA validation mechanisms based on ‘Serving cell changes’, ‘TA timer for idle mode’ and ‘TA always valid’ are mandatory for PUR UEs | Optional with capability signalling |
|  | 2-5 | PUR with L1 ACK | - PUR with L1 ACK | 2-3 | Yes | N/A | PUR will not use L1 ACK | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 2-6 | Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-7 | Multi-TB scheduling for unicast in DL with a single DCI (Non-interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Non-interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (non-interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-8 | Multi-TB scheduling for unicast in UL with a single DCI (Interleaved transmission) | - Multi-TB scheduling for unicast in UL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in UL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-9 | Multi-TB scheduling for unicast in UL with a single DCI(Non-interleaved transmission) | - Multi-TB scheduling for unicast in UL with a single DCI(Non-interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in UL with a single DCI (non-interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-10 | Multi-TB scheduling for unicast in DL in a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | - Multi-TB scheduling for unicast in DL in a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | 2-6 | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-11 | Multi-TB scheduling for SC-MTCH | 1. Scheduling of multiple transport blocks for SC-MTCH in a single DCI 2. Scheduling of multiple transport blocks for SC-MTCH in a single DCI with scheduling gaps | SC-PTM | No | N/A | The network cannot schedule transmission of multiple TBs with a single DCI | Per UE | FDD only | N/A | The basic multicast (SC-PTM) functionality was introduced for LTE-M/NB-IoT in Rel-14 without capability signaling. | Up to RAN2 |
|  | 2-12 | DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-12a | DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | 2-12 | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-13 | UL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - UL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-13a | UL resource reservation with slot-level and symbol(s)-level granularity of NB-IoT non-anchor carriers. | - UL resource reservation with slot-level and symbol(s)-level granularity of NB-IoT non-anchor carriers. | 2-13 | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-14 | Quality report in Msg3 for non-anchor access in idle | - Quality report in Msg3 for non-anchor access in IDLE mode | multicarrier-NPRACH-r14 | No | N/A | The network cannot receive the quality report for non-anchor carriers in IDLE mode | Per-UE | FDD only | N/A |  | Optional without capability signalling |
|  | 2-15 | Quality report in connected mode | 1. Quality report in connected mode other than Msg3 for anchor and non-anchor carriers |  | Yes | N/A | The network cannot receive the quality report other than in Msg3 | Per-UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-16 | NRS on a non-anchor carrier for paging | 1. Presence of NRS on a set of subframes on a non-anchor carrier when no paging NPDCCH is transmitted | Paging non-anchor carrier | No | N/A | UE does not know whether NRS is transmitted on a non-anchor carrier when paging is not present | Per UE | FDD only | N/A |  | Optional without capability signalling |