3GPP TSG RAN WG1 #101		R1-2003789

e-Meeting, May 25th – June 5th, 2020

Agenda item:	6.2.5.3

Source: 	Qualcomm Incorporated

Title: 	UE features for MIMO

Document for:	Discussion and Decision

# Background

The current UE features for MIMO (from email thread [100e-b-LTE-UEFeatures-Remaining]) have the following FFS in component 3-1:

| 3-1 | Additional SRS symbols within normal UL subframes without frequency hopping | 1. Support of additional 1~13 SRS symbols within normal UL subframes with repetitions, |  | Yes | N/A | Network cannot utilize additional SRS symbols within normal UL subframes | Per UE | TDD only | N/A | FFS: How to capture the limitation that a UE may support additional SRS in cells with PUSCH, but not in PUSCH-less SCells. This may be realized by additional capability signaling (including new FG), or change the “additional SRS” (3-1) to per BoBC | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In our companion contribution [1] we provide some remaining details on SRS carrier switching (CS) with type 2 SRS. In this contribution, we present our views on how to introduce a capability for SRS CS + type 2 SRS.

# New capability for SRS carrier switching

Current SRS CS capability is given by retuningTimeInfoBandList-r14, where the UE indicates, for a band pair in the band combination, whether the UE supports SRS carrier switching.

In our view, it is necessary to introduce a separate capability for simultaneous additional SRS and SRS CS:

- SRS CS is not widely deployed today. It is unclear if the set of operators that will deploy SRS CS will also implement additional SRS (and vice-versa). Thus, there may not be IODT opportunities to test these two features together.

- The complexity of additional SRS (from RF point of view) is further exacerbated by the combination with SRS CS. For example, in the case of intra-band contiguous CS, the impact of retuning is reduced, and thus it may be easier to implement the remaining RF operations (e.g. FH/AS) than the case of inter-band CS, where the retuning is requiring an almost complete reconfiguration of the front-end. In general, the complications related to SRS CS are band-pair specific.

Due to these reasons, we propose to follow the SRS CS capability (retuningTimeInfoBandList) to indicate support for additional SRS per band pair of band combination.

Proposal: Introduce a new capability (per band pair of band combination) of whether SRS carrier switching can be used with additional SRS.

| 3-x | Additional SRS symbols within normal UL subframes with SRS carrier switching | 1. Support of additional 1~13 SRS symbols within normal UL subframes with SRS carrier switching | 3-1, SRS carrier switching | Yes | N/A | Network cannot utilize additional SRS symbols within normal UL subframes with SRS carrier switching | Per Band Pair of Band combination | N/A | N/A | For each band pair for which the UE reports SRS carrier switching capability (retuningTimeInfoBandList), the UE indicates whether additional SRS within normal UL subframes can be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

[1] R1-2003784 Additional SRS symbols, Qualcomm Incorporated