3GPP TSG RAN WG1 Meeting #101                     R1- 2003799

e-Meeting, May 25th – June 5th, 2020

Source: 	ZTE Corporation

Title: 	Discussion on LTE Rel-16 UE Features for eMTC

Agenda item:	6.2.5.1

Document for:	Discussion/Decision

Introduction

After RAN1#100b-e meeting, it has been declared all the Rel-16 RAN1-led WIs as finished from RAN1 perspective. To facilitate the UE features discussion, two email discussions (i.e., 100e-b-NR-UEFeature and 100e-b-LTE-UEFeature) on NR and LTE Rel-16 UE features were initiated to collect companies’ input.

In this contribution, remailing detail discussion for LTE Rel-16 eMTC UE features are presented.

Discussions

Multi-TB unicast UL early termination

Currently, it is suggested that 1-25 'subframe level resource reservation for UL in CEmodeA'  and 1-26  'subframe level resource reservation for UL in CEmodeB' could be the prerequisite of UL early termination for multi-TB unicast scheduling. However, UE early termination should be independent of resource reservation

Proposal 1: For 1-17 'Multi-TB unicast UL early termination' remove 1-25/26 in the prerequisite fields.

MPDCCH performance improvement with reciprocity-based candidates in TDD

For FG 1-34, currently it is suggest to FFS whether it can apply to CE mode B. From practical deployment scenario, for majority of CE mode B UE, the SNR of the measurement channel is low therefore the measurement accuracy is usually the problem. Moreover, with long repetition for CE mode B, the delay in CSI measurement is large. Considering also the UE complexity it is suggested not to support this FG in CE mode B.

Proposal 2:  FG 1-34 'MPDCCH performance improvement with reciprocity-based candidates in TDD' only applies to CE mode A.

- Add 'in CEMode A' in FG name

- Remove FFS

- Remove 1-32 in the prerequisite field

Conclusion

In this contribution, proposals on LTE Rel-16 UE features for eMTC are introduced.

Proposal 1: For 1-17 'Multi-TB unicast UL early termination' remove 1-25/26 in the prerequisite fields.

Proposal 2: FG 1-34 'MPDCCH performance improvement with reciprocity-based candidates in TDD' only applies to CE mode A.

- Add 'in CEMode A' in FG name

- Remove FFS

Reference

Chairman note RAN1#99.

Chairman’s note RAN1#94bis.

Chairman’s note RAN1#AH1901.