3GPP TSG RAN WG1 #101-e			R1-2003853

e-Meeting, May 25th – June 5th, 2020

Title:	[Draft] Response LS on NR-U SSB monitoring capabilities

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	[Samsung] RAN WG1

To:	RAN WG4

Cc:	-

Contact Person:

Name:	Hongbo Si

E-mail Address:	Hongbo_dot_Si_at_samsung_dot_com

1. Overall Description:

In R4-2005418 [1], RAN WG4 asked RAN WG1 to provide feedback on aspects regarding the SSB monitoring capabilities for NR-U, and RAN WG1 provides the feedback as follows.

[Q] Provide feedback whether monitoring within a given discovery burst transmission window all candidate SS/PBCH block indexes corresponding to the same SS/PBCH block index is mandatory for UEs.

[A] As a UE capability, it is mandatory for the UEs to monitor all SS/PBCH blocks with candidate indexes corresponding to the same SS/PBCH block index within a given discovery burst transmission window. It can be up to UE’s implementation to stop monitoring if an SS/PBCH block is detected among the SS/PBCH blocks with candidate indexes corresponding to the same SS/PBCH block index within a given discovery burst transmission window.

[Q] Provide feedback on the values of N1 and N2, considering the impact on the network performance if UEs are not monitoring all candidate positions.

[A] N1 and N2 are not applicable, since the UE is monitoring all SS/PBCH blocks with candidate indexes corresponding to the same SS/PBCH block index within a given discovery burst transmission window.

[Q] Provide feedback on whether differentiation is needed for UEs operating in FBE and LBE modes.

[A] There is no need to differentiate UEs operating in FBE and LBE modes.

[Q] Provide feedback for the case when Q is not provided to the UE

[A] For RLM/BFD/CBD, Q is always provided to the UE. More details of the indication of Q can be found in R1-2003044 [2].

2. Actions:

To RAN WG4.

ACTION: RAN WG1 kindly asks RAN WG4 to take above responses into account.

3. References

[1] R4-2005418, “LS on NR-U SSB monitoring capabilities”, Nokia, RAN4

[2] R1-2003044, “LS on Signaling of Q Parameter for NR-U”, Charter Communications, RAN1

4. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #102				24th – 28th August 2020		e-Meeting

TSG-WG1 Meeting #102bis			12th – 16th October 2020		CN