3GPP TSG RAN WG1 #101	        		             R1-2003865

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.5.1

Source: 	Samsung

Title: 				    Remaining issues for PDCCH enhancements

Document for:	Discussion and Decision

# Introduction

This contribution focuses on open issues on the DL control signaling design for the completion of the Rel-16 URLLC specifications considering the guidance and the limitations of the e-meeting.

# Remaining issues on DL control for URLLC

“Aligned” and “unaligned” spans

The following is currently captured in [1] for TS 38.213.

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - TBD, otherwise where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

Some optimizations may be possible if “the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y” does not hold (i.e. for the TBD in the text above). However, as CA is not a typical use-case for Rel-16 (span-based) PDCCH monitoring and as “unaligned” spans across cells is a further atypical configuration, reverting to Rel-15 operation is preferred to avoid introducing additional mechanisms to address “unaligned” spans and to maintain operational simplicity, especially if the end result is slot-based partitioning of PDCCH candidates/non-overlapping CCEs. In other words, if a configuration of search space sets in one or more cells would result to “unaligned” spans, the UE expects to be configured for Rel-15 PDCCH monitoring on those cells. This also avoids one cell with combination X,Y resulting to unaligned spans for all cells with combination X,Y for a same SCS.

Proposal 1: Update the specification text in [1] as follows to address the TBD

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE - expects that the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,X,Y,μ downlink cells results to PDCCH monitoring according to the combination X,Y, and - is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cellsNcells,r16DL,(X,Y),μX,Y where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

It was previously proposed (e.g. [2]), together with a corresponding TP, to extend the expressions for the partition of PDCCH candidates and non-overlapping CCEs to multi-TRP operation since URLLC is one of the main use cases for multi-TRP in Rel-16. For brevity, the TP is not repeated here as it is a direct extension of the current specifications for slot-based/Rel-15 PDCCH monitoring to span-based/Rel-16 PDCCH monitoring. The only aspect to resolve is whether to define separate RRC parameters/UE capability for Rel-16 PDCCH monitoring (UE capability for the factor R and BDFactorR) or to assume that a same reported UE capability and BDFactorR are applicable to both Rel-15 and Rel-16 PDCCH monitoring.

Proposal 2: Capture multi-TRP operation for Rel-16 PDCCH monitoring.

Order between determination of X,Y combination and span duration

One aspect identified during the email discussion from RAN1#100bis-e is the determination for the order of the following two operations:

Operation 1 (determination of span duration)

The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot.

Operation 2 (determination of X,Y combination)

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

Given the supported X,Y combinations, the ordering for “Operation 1” and “Operation 2” needs to be clarified when dCORESET,max=2 or dCORESET,max=1. Clearly, if dCORESET,max=3 and if search space sets are not misconfigured, a separation of every two consecutive PDCCH monitoring spans should be at least X=4 symbols and then it does not matter which of the above operations is performed first.

For dCORESET,max=2 or dCORESET,max=1, if “Operation 1” is performed first and if the UE reports X,Y=(2, 2), then dspan=2. Then, assuming that Y=dspan in “Operation 2”, only X,Y=(2, 2) is possible even if the configuration of search space sets allows for e.g. X,Y=(4, 3). However, such an event could in principle be avoided by the network by not configuring dCORESET,max=2 or dCORESET,max=1 if the search space configuration is such that it could result to e.g. X,Y=(4, 3). However, this is an unnecessary restriction and can have other side effects such as reduced coverage. If “Operation 2” is performed first, then if the result is X,Y=(4, 3) or X,Y=(7, 3), dspan=3 regardless of dCORESET,max or Ymin. If the result is X,Y=(2, 2), then dspan=2 regardless of dCORESET,max or Ymin as dCORESET,max=3 would be a misconfiguration for X,Y=(2, 2). Therefore, if “Operation 2” is performed first,  dspan=maxdCORESET,max,Ymin is irrelevant. Overall, there is less specification ambiguity and there is no network restriction if “Operation 2” is performed first. Then, the Rel-15 component of dspan=maxdCORESET,max,Ymin is unnecessary for Rel-16 PDCCH monitoring and the specifications can be simplified since the duration of a span is the value of Y resulting from “Operation 2”.

Observation 1: If a UE first determines a combination X,Y for PDCCH monitoring as the combination resulting to a separation of every two consecutive PDCCH monitoring spans that is at least X and has the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ, the duration of a span is Y.

Proposal 3: Update the following text in [1] as

If a UE monitors PDCCH on a cell according to combination X,Y, the duration of a span is Y dspan=maxdCORESET,max,YmindCORESET,maxYminYX,Y. A last span in a slot can have a shorter duration than other spans in the slot.

Last span in a slot can have shorter duration

Another issue is the following statement that was inherited from FG 3-5b in Rel-15.

A last span in a slot can have a shorter duration than other spans in the slot.

The above may be beneficial for example as it allows for X,Y=(4, 3) PDCCH monitoring to start at the beginning of a slot and have Y=1 at the 13th symbol of the slot. However, in that case, the following statement is not applicable for the “including across slots”.

The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots.

Proposal 4: RAN1 to clarify when and how both following statements are applicable. If no such case, remove the second statement from the specifications as otherwise the UE behavior is ambiguous.

The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots.

and

A last span in a slot can have a shorter duration than other spans in the slot.

Determination of combination X,Y

The following is captured in [1] for a UE to determine the (X, Y) combination for PDCCH monitoring in case the UE declares support for multiple (X, Y) combinations.

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

The UE behavior is not defined for the case that a configuration of search space sets for PDCCH monitoring results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for only one of the multiple combinations X,Y. This is a trivial issue (it is understood that the UE monitors PDCCH on the cell according to that one combination X,Y), but the specifications are actually incomplete as the UE behavior is not defined.

Proposal 5: Update [1] as follows to complete the description of the UE behavior for determining a combination (X, Y) for PDCCH monitoring.

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more combinations X,Y of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y from the one or more combinations X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

DCI format related issues

A few aspects relating to DCI formats were identified and/or discussed in RAN1#100-e including the following.

- Need for specification support to distinguish DCI formats 0_2/1_2 from DCI formats 0_0/1_0

Nearly all fields of DCI formats 0_2/1_2 are configurable. A network can always choose a size of one (or more) of the fields such that a size of DCI formats 0_2/1_2 is different than a size of DCI formats 0_0/1_0. The network can do so while getting improved performance (e.g. RV field of 1 bit instead of 0 bits) and/or with/without using the increased functionality (e.g. TDRA field of 3 bits instead of 2 bits and it is up to the NW whether or not to use the additional entries). In any case, implementation mechanisms are always preferable to introducing a (useless) padding bit and any specification support is unnecessary.

Observation 2: There is no need to specify any mechanism to distinguish DCI formats 0_2/1_2 and DCI formats 0_0/1_0.

- Mapping of 1 RV bit to RV values

It was identified in RAN1#100-e that, in case of an RV field with 1 bit, the values map to 0 and 3 instead of 0 and 2. Although simple to fix, this issue was deprioritized from the email discussions [2]. Clearly, as the objective is to increase data rates and, unlike CG-PUSCH, there is no point for self-decodable retransmissions, the values should be 0 and 2 and not 0 and 3. A same conclusion was reached in NR-U where the RV field of 1 bit maps to values of 0 and 2.

Proposal 6: Correct the mapping for the RV field with 1 bit from 0 and 3 to 0 and 2 for DCI formats 0_2/1_2.

# Conclusions

This contribution considered open issues related to DL control signaling for Rel-16 URLLC and proposes the following.

Proposal 1: Update the specification text in [1] as follows to address the TBD

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE - expects that the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,X,Y,μ downlink cells results to PDCCH monitoring according to the combination X,Y, and - is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cellsNcells,r16DL,(X,Y),μX,Y where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

Proposal 2: Capture multi-TRP operation for Rel-16 PDCCH monitoring.

Proposal 3: Update the following text in [1] as

If a UE monitors PDCCH on a cell according to combination X,Y, the duration of a span is Y dspan=maxdCORESET,max,YmindCORESET,maxYminYX,Y. A last span in a slot can have a shorter duration than other spans in the slot.

Proposal 4: RAN1 to clarify when and how both following statements are applicable. If no such case, remove the second statement from the specifications as otherwise the UE behavior is ambiguous.

The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots.

and

A last span in a slot can have a shorter duration than other spans in the slot.

Proposal 5: Update [1] as follows to complete the description of the UE behavior for determining a combination (X, Y) for PDCCH monitoring.

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more combinations X,Y of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y from the one or more combinations X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

Proposal 6: Correct the mapping for the RV field with 1 bit from 0 and 3 to 0 and 2 for DCI formats 0_2/1_2.

In addition, the following observations are made.

Observation 1: If a UE first determines a combination X,Y for PDCCH monitoring as the combination resulting to a separation of every two consecutive PDCCH monitoring spans that is at least X and has the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ, the duration of a span is Y.

Observation 2: There is no need to specify any mechanism to distinguish DCI formats 0_2/1_2 and DCI formats 0_0/1_0.

# References:

- R1-2003176, “Corrections on Ultra Reliable Low Latency Communications Enhancements”

- R1-2003176, “Remaining issues for PDCCH enhancements”, Samsung