3GPP TSG RAN WG1#101-e			R1-2003883

e-Meeting, 25th May – 5th June, 2020

Agenda item:	7.2.6.4

Source: 	Samsung

Title: 	On UL full power transmission

Document for:	Discussion and Decision

# Introduction

This contribution discusses some issues related to the full power UL transmission in Rel. 16 NR_eMIMO.

# On TPMI groups for mode 2

The following agreement was made in RAN1#99.

Agreement For 4 ports, number of bits to indicate TPMI(s) which can deliver UL full power: - Non Coherent 2 bits - Partial coherent 4 bits - Additional entries on top of existing entries may be added to table 1 and table 2 - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions Table 1. 4Tx, nonCoherent; 4Tx, partial coherent (4bit). G0; G0. G1; G1. G2; G2. G3; G3. G4. G5. G6. Definition of G0~G6 can be found in the table below. Table 2.

There have proposals from several companies to include additional TPMI groups for 4Tx, partial-coherent case. In our view, additional TPMI groups are not needed due not the following reasons.

- The issue is not critical (non-essential) since the current specification is not broken. Hence, such specification change is beyond the scope of normative maintenance work.

- The agreed 7 TPMI groups can cover most of the relevant PA combinations. This is shown in Figure 1 wherein it has shown that 76 out of 81 PA combinations can be covered by the agreed groups.

- The performance benefits of any additional TPMI group, if there is any, when compared with the agreed TPMI groups is likely to be marginal.

Figure 1

Observation: additional TPMI groups for mode 2, 4Tx, partial-coherent UEs is not needed.

- The issue is not critical (non-essential) since the current specification is not broken.

- The agreed 7 TPMI groups can cover most of the relevant PA combinations.

- The performance benefits of any additional TPMI group, if there is any, when compared with the agreed TPMI groups is likely to be marginal.

Since significant amount of discussion on this topic has already happened in last two e-Meetings, to save RAN1 meeting time and repeated discussions, in our view, there is no need for any further discussion in future meetings. We therefore propose the following.

Proposal 1: For mode, 4 ports, partial-coherent UEs,

- Number of bits to indicate TPMI(s) which can deliver UL full power is reduced to 3 bits

- Any additional TPMI group on top of agreed TPMI groups is not supported.

UE capability

The outcome of UE feature discussion during RAN1#100-b-e [1] includes the following codebook-related feature groups (FGs).

| 16-5a | UL full power transmission [mode 0] | - Supported UL full power transmission [mode 0] |
| --- | --- | --- |
| 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} - FFS: New UL codebook set(s) per supported Tx |
| 16-5c | [UL full power transmission mode 2] | - [Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port] |

Based on the email discussion “Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects” [2], FG 16-5b was agreed to be revised as follows.

| 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - [Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx}] - [New UL codebook set(s) per supported Tx] |
| --- | --- | --- |

In this section, three remaining FFS items are discussed.

The first item is the bracket around mode 0 in FG 16-5a. In our view, the bracket should be removed since it adds to the clarity and improves spec-readability.

The second item is the component 2 in FG 16-5b. The technical motivation of having 2Tx_4Tx is unclear. If a UE reports 4Tx, then it should be able to support 2Tx also. We fail see the reason why a UE supporting 4Tx can’t support 2Tx. So, in our view, 2Tx_4Tx is redundant and not needed.

The third item is related to FG 16-5c. There has been extensive discussion during previous e-Meeting whether to keep FG 16-5c as one row or to split it into two rows. The proponents of two rows consider the two features for mode 2 (multiple SRS resources with different number of SRS ports and TPMI group signaling) as separate since they may require two different UE implementations. We prefer two rows since it adds more flexibility/choices to a UE. Regarding 2Tx_4Tx in component 2, we prefer to remove it due to the reason explained above. FFS in component 3 is not needed since we have component 2 for number of Tx. Component 4 and 5 are not needed. As discussed, component 6 should be a row or FG group.

Proposal 2: Regarding UE capability for full power UL transmission,

- Remove bracket around mode 0 in FG 16-5a

- Remove 2Tx_4Tx from component 2 of FG 16-5b

- Split FG 16-5c into two rows with component 6 being a separate row or FG

  - Remove 2Tx_4Tx from component 2 of FG 16-5c

  - Remove FFS from component 3 of FG 16-5c

  - Remove component 4 and 5 of FG 16-5c

Conclusion

In this contribution, the following observation and proposals are given:

Observation: additional TPMI groups for mode 2, 4Tx, partial-coherent UEs is not needed.

- The issue is not critical (non-essential) since the current specification is not broken.

- The agreed 7 TPMI groups can cover most of the relevant PA combinations.

- The performance benefits of any additional TPMI group, if there is any, when compared with the agreed TPMI groups is likely to be marginal.

Proposal 1: For mode, 4 ports, partial-coherent UEs,

- Number of bits to indicate TPMI(s) which can deliver UL full power is reduced to 3 bits

- Any additional TPMI group on top of agreed TPMI groups is not supported

Proposal 2: Regarding UE capability for full power UL transmission,

- Remove bracket around mode 0 in FG 16-5a

- Remove 2Tx_4Tx from component 2 of FG 16-5b

- Split FG 16-5c into two rows with component 6 being a separate row or FG

  - Remove 2Tx_4Tx from component 2 of FG 16-5c

  - Remove FFS from component 3 of FG 16-5c

  - Remove component 4 and 5 of FG 16-5c

# References

- R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E,” AT&T/NTT DOCOMO, INC.

- R1-2003194, “Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — MIMO Enhancement Aspects,” Moderator (AT&T).