3GPP TSG RAN WG1 #101	R1-2003906

e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.2.1

Source: 	Samsung

Title: 	Additional scenarios for evaluation

Document for:	Discussion and Decision

# Introduction

A new study item on “NR Positioning Enhancements” was approved in RAN#86 and the objectives are as follows [1].

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

In this contribution, we discuss the additional scenarios for evaluation for NR positioning evaluation.

# Use cases and prioritized scenarios

In order to facilitate the evolution of future factories, new use cases, i.e., IIOT use cases with the associated performance requirements are defined to identify the performance gap and the simulation scenarios for the IIOT use cases with associated parameters. Eight positioning service levels are defined in [2] as below, each corresponding to a specific use case.

| Position service level | Horizontal accuracy | Availability | Heading | Latency for position estimation of UE | UE Mobility | Use case reference |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | < 1 m | [99.9%] | < 0,54 rad | < [1 s] | N/A | Factories of the Future 6.5 |
| 2 | < [5 m] | [90%] | N/A | < [5 s] | N/A | Factories of the Futur6 6.7 |
| 3 | < [1 m] | [99%] | < [0,17 rad] | < [15 ms] | < [10 km/h] | Factories of the future 10.8 |
| 4 | < [1 m] | [90%] | N/A | < [2 s] | < [30 km/h] | Factories of the Future 13.3 |
| 5 | < [30 cm (if supported by further sensors like camera, GNSS, IMU)] | [99.9%] | N/A | [10 ms] | < [30 km/h] | Factories of the Future15.5 |
| 6 | < [20 cm] | [99%] | N/A | < [1 s] | < [30 km/h] | Factories of the Future15.6 |
| 7 | < [50 cm] | [99%] | N/A | [1 s] | < [30 km/h] | Factories of the Future18.19 |
| 8 | < [1m (relative positioning)] | [99%] | N/A | [1 s] | < [30km/h] | Factories of the Future18.20 |

Table-1 Positioning Service Performance Requirements

The first two service levels correspond to mobile control panels with safety functions. Due to the criticality of these safety functions, it is crucial that these two service levels should be supported. The last service level, i.e., relative positioning, corresponds to the use case of flexible, modular assembly area, where a modular production environment adaptable to the different variants of product orders is up and running. In order to achieve high production flexibility and versatility, this service level should also be supported. Therefore, the target positioning requirements should be defined following the IIoT use cases with positioning level 1,2 and 8 in Table 8.1.7 in TR 22.804. In addition, table 7.3.2.2-1 of TS 22.261 should also be considered.

Proposal 1: The target positioning requirements should be defined following the IIoT use cases with positioning level 1, 2 and 8 in Table 8.1.7 in TR 22.804 and Table 7.3.2.2-1 of TS 22.261.

For the evaluation of positioning performance with the target IIOT scenarios, the channel models of 5 Indoor Factory (InF) scenarios are defined in TR 38.901 as follows,

- InF-SL	Indoor Factory with Sparse clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-DL	Indoor Factory with Dense clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-SH	Indoor Factory with Sparse clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-DH	Indoor Factory with Dense clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-HH	Indoor Factory with High Tx and High Rx (both elevated above the clutter)

Among above five scenarios, the LOS condition of InF-SH is the best and it should be a baseline scenario for comparing and calibrating the evaluation results from multiple sources. In addition, InF-DH can be considered as additional scenario.

Proposal 2: InF-SH should be considered as baseline scenario for evaluation.

For evaluation metrics, positioning accuracy should be the baseline metric including relative positioning accuracy. In addition, latency, signaling overhead and UE power consumption can be considered as the metrics for evaluation. It should be noted that not all metrics can be evaluated in a quantitive manner.

Proposal 3: Positioning accuracy including relative positioing accuracy should be the baseline metric for evaluation. Latency, signalling overhead and UE power consumption can be considered additionally as metrics for evaluation in an analytical manner.

# Evaluation parameters

## Evaluation parameters

At the beginning and during the stage of Rel-16 NR positioning study, the evaluation parameters including carrier frequency, BW, gNB and UE models, etc., are defined for three scenarios: indoor office, UMi and Uma as below.

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table-2 Common scenario parameters applicable for all scenarios

The above table and the IIOT channel models defined in TR 38.901 should be a starting point for evaluation parameters.

Proposal 4: Evaluation parameters in the below table can be a starting point with addtional consideration to include IIoT channel model in TR 38.901.

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

## PRS configurations

In order to compare results from different sources in a fair manner, PRS configuration should also be defined. However, a small set should be defined to achieve reasonably comprehensive evaluation results without increasing simulation load significantly. Based on previous email discussion, the following table can be a starting point.

| Parameters | Values |
| --- | --- |
| DL-PRS-Periodicity | 10 ms |
| DL-PRS-ResourceRepetitionFactor | 1 |
| DL-PRS-ResourceTimeGap | 1 |
| DL-PRS-SFN0-Offset | 0 |
| DL-PRS-CombSizeN | N=6 |
| DL-PRS-ReOffset | 0,3,1,4,2,5 |
| DL-PRS-ResourceSymbolOffset | 0 to 13 |
| DL-PRS-NumSymbols | 6 |
| DL-PRS-SubcarrierSpacing | 15 kHz for FR1 120 kHz for FR2 |
| DL-PRS-CyclicPrefix | Normal |
| DL-PRS-ResourceBandwidth | [50/100 MHz] with [15/30]kHz SCS for FR1 400 MHz with [120]kHz SCS for FR2 |
| DL-PRS-expectedRSTD | +/- 1 us |
| DL-PRS-expectedRSTD-uncertainty | FR1/FR2: +/- 1 us FR2: +/- 1 us |
| DL–PRS- PositioningFrequencyLayer | 1 |
| NumDL-PRS-RSTD-MeasurementsPerTRPPair | 1 |
| NumPositioningFrequencyLayers | 1 |
| NumTrpPerPositioningFrequencyLayer | 19 |
| NumDL-PRS-ResourceSetsPerTRPFrequencyLayer | 1 |
| NumDL-PRS-ResourcesPerSet | Configured as nedded for the beamforming in the simulation evaluation |

Table-3 PRS configuration

Proposal 5: The below table can be a starting point for PRS configuration for evaluation.

| Parameters | Values |
| --- | --- |
| DL-PRS-Periodicity | 10 ms |
| DL-PRS-ResourceRepetitionFactor | 1 |
| DL-PRS-ResourceTimeGap | 1 |
| DL-PRS-SFN0-Offset | 0 |
| DL-PRS-CombSizeN | N=6 |
| DL-PRS-ReOffset | 0,3,1,4,2,5 |
| DL-PRS-ResourceSymbolOffset | 0 to 13 |
| DL-PRS-NumSymbols | 6 |
| DL-PRS-SubcarrierSpacing | 15 kHz for FR1 120 kHz for FR2 |
| DL-PRS-CyclicPrefix | Normal |
| DL-PRS-ResourceBandwidth | [50/100 MHz] with [15/30]kHz SCS for FR1 400 MHz with [120]kHz SCS for FR2 |
| DL-PRS-expectedRSTD | +/- 1 us |
| DL-PRS-expectedRSTD-uncertainty | FR1/FR2: +/- 1 us FR2: +/- 1 us |
| DL–PRS- PositioningFrequencyLayer | 1 |
| NumDL-PRS-RSTD-MeasurementsPerTRPPair | 1 |
| NumPositioningFrequencyLayers | 1 |
| NumTrpPerPositioningFrequencyLayer | 19 |
| NumDL-PRS-ResourceSetsPerTRPFrequencyLayer | 1 |
| NumDL-PRS-ResourcesPerSet | Configured as nedded for the beamforming in the simulation evaluation |

# Conclusions

In this contribution, we discussed the additional scenarios for evaluation for NR positioning evaluation. Our proposals are summarized below.

Proposal 1: The target positioning requirements should be defined following the IIoT use cases with positioning level 1, 2 and 8 in Table 8.1.7 in TR 22.804.

Proposal 2: InF-SH should be considered as baseline scenario for evaluation.

Proposal 3: Positioning accuracy including relative positioing accuracy should be the baseline metric for evaluation. Latency, signalling overhead and UE power consumption can be considered additionally as metrics for evaluation in an analytical manner.

Proposal 4: Evaluation parameters in the below table can be a starting point with addtional consideration to include IIoT channel model in TR 38.901.

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Proposal 5: The below table can be a starting point for PRS configuration for evaluation.

| Parameters | Values |
| --- | --- |
| DL-PRS-Periodicity | 10 ms |
| DL-PRS-ResourceRepetitionFactor | 1 |
| DL-PRS-ResourceTimeGap | 1 |
| DL-PRS-SFN0-Offset | 0 |
| DL-PRS-CombSizeN | N=6 |
| DL-PRS-ReOffset | 0,3,1,4,2,5 |
| DL-PRS-ResourceSymbolOffset | 0 to 13 |
| DL-PRS-NumSymbols | 6 |
| DL-PRS-SubcarrierSpacing | 15 kHz for FR1 120 kHz for FR2 |
| DL-PRS-CyclicPrefix | Normal |
| DL-PRS-ResourceBandwidth | [50/100 MHz] with [15/30]kHz SCS for FR1 400 MHz with [120]kHz SCS for FR2 |
| DL-PRS-expectedRSTD | +/- 1 us |
| DL-PRS-expectedRSTD-uncertainty | FR1/FR2: +/- 1 us FR2: +/- 1 us |
| DL–PRS- PositioningFrequencyLayer | 1 |
| NumDL-PRS-RSTD-MeasurementsPerTRPPair | 1 |
| NumPositioningFrequencyLayers | 1 |
| NumTrpPerPositioningFrequencyLayer | 19 |
| NumDL-PRS-ResourceSetsPerTRPFrequencyLayer | 1 |
| NumDL-PRS-ResourcesPerSet | Configured as nedded for the beamforming in the simulation evaluation |

# References

- RP-193237, “NR Positioning Enhancements,” Qualcomm Incorporated.

- 3GPP TR 22.804, " Study on Communication for Automation in Vertical Domains ".