3GPP TSG RAN WG1 #101	R1-2003907

e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.2.2

Source: 	Samsung

Title: 	Evaluation of achievable positioning accuracy and latency

Document for:	Discussion and Decision

# Introduction

A new study item on “NR Positioning Enhancements” was approved in RAN#86 and the objectives are as follows [1].

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

In this contribution, we discuss the evaluation of achievable positioning accuracy and latency.

# Use Cases and Requirements

In this section, we present our views on the use cases and review the general requirements for evaluation. In order to facilitate the evolution of future factories, new use cases, i.e., IIOT use cases with the associated performance requirements are defined to identify the performance gap and the simulation scenarios for the IIOT use cases with associated parameters. In our companion contribution [2], we have identified the following three specific use cases with positioning level 1, 2 and 8 for target requirement of evaluation.

- Mobile control panels with safety functions;

- Flexible, modular assembly area.

Three requirements including horizontal positioning accuracy, availability and latency are proposed in [3] and in our view, horizontal positioning accuracy < 1m and latency < 1s should be the main requirements. In addition, for IIOT devices, they are expected to operate for a long time without human intervene. In this regard, power consumption is an important requirement, which relates to many aspects, such as signaling overhead, etc.

Proposal 1: The target requirements for NR positioning enhancement should be

- horizontal positioning accuracy < 1m

- latency < 1s.

In addition, power consumption can also be considered which can be reflected as aspects such as signaling overhead.

# Evaluation parameters and preliminary simulation results

In our companion contribution [], we have presented the following proposals for evaluation parameters.

Proposal 2: Evaluation parameters in the below table can be a starting point with addtional consideration to include IIoT channel model in TR 38.901.

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Proposal 3: The below table can be a starting point for PRS configuration for evaluation.

| Parameters | Values |
| --- | --- |
| DL-PRS-Periodicity | 10 ms |
| DL-PRS-ResourceRepetitionFactor | 1 |
| DL-PRS-ResourceTimeGap | 1 |
| DL-PRS-SFN0-Offset | 0 |
| DL-PRS-CombSizeN | N=6 |
| DL-PRS-ReOffset | 0,3,1,4,2,5 |
| DL-PRS-ResourceSymbolOffset | 0 to 13 |
| DL-PRS-NumSymbols | 6 |
| DL-PRS-SubcarrierSpacing | 15 kHz for FR1 120 kHz for FR2 |
| DL-PRS-CyclicPrefix | Normal |
| DL-PRS-ResourceBandwidth | [50/100 MHz] with [15/30]kHz SCS for FR1 400 MHz with [120]kHz SCS for FR2 |
| DL-PRS-expectedRSTD | +/- 1 us |
| DL-PRS-expectedRSTD-uncertainty | FR1/FR2: +/- 1 us FR2: +/- 1 us |
| DL–PRS- PositioningFrequencyLayer | 1 |
| NumDL-PRS-RSTD-MeasurementsPerTRPPair | 1 |
| NumPositioningFrequencyLayers | 1 |
| NumTrpPerPositioningFrequencyLayer | 19 |
| NumDL-PRS-ResourceSetsPerTRPFrequencyLayer | 1 |
| NumDL-PRS-ResourcesPerSet | Configured as nedded for the beamforming in the simulation evaluation |

One of the main challenge for IIOT is the NLOS channel. With increased NLOS probability, the current positioning scheme, e.g., OTDOA, will fail since the basic assumption for OTDOA is LOS link between the cells and the UE. We have provided some preliminary simulation results for indoor UEs to show the performance degradation due to increased NLOS probability in the below table.

| LOS probability | 100% | 95% | 90% | 85% | 80% |
| --- | --- | --- | --- | --- | --- |
| 90% accuracy | 0.2m | 2.1m | 4.8m | 6.7m | 8m |

Table-1 Accuracy v.s. LOS probability

As we can see from the figure, when NLOS is increased to 20%, the 90% positioning accuracy is degraded to 8m, which is even lower than the Rel-16 accuracy requirement for indoor scenario. Therefore, NLOS links should be identified and removed from measurement and reporting.

Observation 1: With increased NLOS probability, positioning accuracy degrades significantly.

# Conclusions

In this contribution, we discussed the additional scenarios for evaluation for NR positioning evaluation. Our proposals are summarized below.

Observation 1: With increased NLOS probability, positioning accuracy degrades significantly.

Proposal 1: The target requirements for NR positioning enhancement should be

- horizontal positioning accuracy < 1m

- latency < 1s.

In addition, power consumption can also be considered which can be reflected as aspects such as signaling overhead.

Proposal 2: Evaluation parameters in the below table can be a starting point with addtional consideration to include IIoT channel model in TR 38.901.

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Proposal 3: The below table can be a starting point for PRS configuration for evaluation.

| Parameters | Values |
| --- | --- |
| DL-PRS-Periodicity | 10 ms |
| DL-PRS-ResourceRepetitionFactor | 1 |
| DL-PRS-ResourceTimeGap | 1 |
| DL-PRS-SFN0-Offset | 0 |
| DL-PRS-CombSizeN | N=6 |
| DL-PRS-ReOffset | 0,3,1,4,2,5 |
| DL-PRS-ResourceSymbolOffset | 0 to 13 |
| DL-PRS-NumSymbols | 6 |
| DL-PRS-SubcarrierSpacing | 15 kHz for FR1 120 kHz for FR2 |
| DL-PRS-CyclicPrefix | Normal |
| DL-PRS-ResourceBandwidth | [50/100 MHz] with [15/30]kHz SCS for FR1 400 MHz with [120]kHz SCS for FR2 |
| DL-PRS-expectedRSTD | +/- 1 us |
| DL-PRS-expectedRSTD-uncertainty | FR1/FR2: +/- 1 us FR2: +/- 1 us |
| DL–PRS- PositioningFrequencyLayer | 1 |
| NumDL-PRS-RSTD-MeasurementsPerTRPPair | 1 |
| NumPositioningFrequencyLayers | 1 |
| NumTrpPerPositioningFrequencyLayer | 19 |
| NumDL-PRS-ResourceSetsPerTRPFrequencyLayer | 1 |
| NumDL-PRS-ResourcesPerSet | Configured as nedded for the beamforming in the simulation evaluation |

# References

- RP-193237, “NR Positioning Enhancements,” Qualcomm Incorporated.

- R1-2003906, “Additional scenarios for evaluation,” Samsung.

- 3GPP TR 22.804, " Study on Communication for Automation in Vertical Domains ".