3GPP TSG RAN WG1 #101	R1-2003908

e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.2.3

Source: 	Samsung

Title: 	Potential positioning enhancements

Document for:	Discussion and Decision

# Introduction

A new study item on “NR Positioning Enhancements” was approved in RAN#86 and the objectives are as follows [1].

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

In this contribution, we discuss the potential positioning enhancements for NR positioning.

# PRS pattern enhancement

In this section, we present our views on the enhancement of PRS pattern. In Rel-16, the following combination of comb sizes and number of symbols are supported.

|  | 2 symbols | 4 symbols | 6 symbols | 12 symbols |
| --- | --- | --- | --- | --- |
| Comb-2 | {0,1} | {0,1,0,1} | {0,1,0,1,0,1} | {0,1,0,1,0,1,0,1,0,1,0,1} |
| Comb-4 | NA | {0,2,1,3} | NA | {0,2,1,3,0,2,1,3,0,2,1,3}} |
| Comb-6 | NA | NA | {0,3,1,4,2,5} | {0,3,1,4,2,5,0,3,1,4,2,5} |
| Comb-12 | NA | NA | NA | Working assumption: {0,6,3,9,1,7,4,10,2,8,5,11} |

Table-1 Supported comb size and number of symbols

This pattern can guarantee that the PRS can be distributed in the frequency domain as uniform as possible. However, such pattern can also cause collision of PRS from multiple cells if the starting symbols of PRS from multiple cells are different.

For example, in Fig. 1 (a) comb-4 and 4 symbols are illustrated starting from the first symbol of a slot in cell 1. In cell 2, the same comb size and number of symbols are applied but the starting symbol is the second symbol of a slot.

(a)

(b)

Fig. 1 (a) Cell 1 PRS with initial offset 0 (b) Cell 2 PRS with initial offset 2

As we can see from the figure, collision will always happen between cell1 PRS and cell 2 PRS if the initial offset of PRS from cell 2 is 2 or 3. When collsion happens, PRS from different cells will interfere with each other and the positioning accuracy is degraded. However, such collision can be avoided using the diagonal pattern shown in the below figure.

Fig. 2 Diagonal PRS pattern

For IIOT scenario, the density of cells can be quite high and in order to avoid collision as much as possible, diagonal PRS pattern should be supported as well.

Proposal 1: New PRS pattern should be studied to avoid collision between multiple TRPs.

# IDLE/INACTIVE positioning

For IIOT devices, power consumption is of paramount importance since these devices are expected to operate for a long time without human intervene. In this regard, IIOT devices should be kept in IDLE/INACTIVE state as much as possible to reduce power consumption. In LTE, UE needs to be in RRC-CONNECTED state to receive positioning reference signals and perform positioning function. In NR, due to the fact that some devices are of low complexity and require low energy consumption, RRC-IDLE state positioning is discussed. Since PRS can be periodic, the wakeup periodicity of the UE can be matched with PRS periodicity so that when UE wakes up, it can potentially measure PRS without entering RRC-CONNECTED state.

However, positioning in RRC-IDLE state has following challenges. Firstly, there is no UE reporting in RRC-IDLE state since UL link is not established. It has two implications: 1) only UE based positioning is feasible since network based positioning requires UE reporting of the measurements; and 2) network cannot know the UE position without UE reporting. In [2], the following use cases are defined.

Table 6.1-1 – Use cases synthesis

| Use cases |  | Potential requirements per use cases |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Environment of Use | Position Accuracy | Velocity | Avail. | Update rate or interval | TTFF | Latency | Other KPI |
| 5.2.1 | Bike sharing | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s |  |
|  |  | Enhanced positioning area - Outdoor | 0.2m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.2.2 | Augmented Reality | Outdoor - 5G positioning service area | 1-3m Horizontal 0.1-3m Vertical | 2 m/s 10deg. | 80 % | 1 - 10 Hz | 10s | 1s | Low Energy |
| 5.2.3 | Wearables | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 90 % | 30s - 300s | 10s |  | Power saving mode |
|  |  | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 99 % | 1s - 30s | 10s | 1s | Normal mode |
| 5.2.4 | Advertisement push | 5G positioning service area - -Outdoor/Indoor | 3m Horizontal 3m Vertical |  | 90 % |  |  | 60s |  |
| 5.2.5 | Flow management | Enhanced positioning- Outdoor/Indoor | 10m Horizontal |  | 80 % | 10s | 10s |  |  |
| 5.3.1 | Person and medical equipment location in Hospital | Enhanced positioning- Outdoor/Indoor | 3m Horizontal 2m Vertical |  | 99 % |  |  | 60s |  |
| 5.3.2 | Patient location (outside Hospital) | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 99 % |  |  |  |  |
| 5.3.3 | Trolley | Enhanced positioning- Outdoor/Indoor | 0.5m Horizontal 1-3m Vertical |  | 99 % |  |  | 20ms |  |
| 5.3.4 | Waste management | 5G positioning service area - Outdoor | 3m Horizontal |  | 99 % | 2h - 1 day |  | 60s | Very low energy (15 years) |
| 5.4.1 | Emergency call | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical |  | 95 % |  | 30s | 60s | Reliability/ Confidence |
| 5.4.2 | Accurate Positioning for First Responders | Outdoor | 1m Horizontal, 0.3 m Vertical |  | 98 % |  | 10s | 5s | MCX Confidence Event-triggered report |
|  |  | Indoor | 1m Horizontal, 2 m Vertical |  | 95 % |  | 10s | 1s |  |
| 5.4.3 | Alerting nearby emergency responders | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical (floor) |  | 99% |  | 10s |  | Privacy, scalability, cross operator |
| 5.4.4 | Emergency equipment loc. outside hospitals | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 95% |  | 10s |  | Extended sleep periods |
| 5.5.1 | Traffic Monitoring & Control | 5G positioning service area - Outdoor | 1-3m Horizontal 2.5m Vertical |  | 95 % | 10 Hz | 10s | 30ms | Antispoofing Antitampering |
| 5.5.2 | Road User Charging | 5G positioning service area - Outdoor Enhanced positioning-Tunnels | <1m (across track) 3m (along track) | 2 m/s | 99 % | 1 Hz | 10s |  | Antispoofing Antitampering |
| 5.6.1 | Asset tracking and management | 5G positioning service area - Outdoor | 10-30m Horizontal | 5 m/s | 99 % | 300s-1day |  |  | 20 mJ/fix (average), Antispoofing, Antitampering, support for "out of coverage" positioning |
|  |  | Enhanced positioning - Outdoor | 1m Horizontal |  | 99 % | 1s | 1s in enhanced positioning area |  |  |
| 5.7.1 | UAV (Data analysis) | 5G positioning service area - Outdoor | 0.1m Horizontal 0.1m Vertical | 0.5 m/s 2 deg. | 99 % |  | 10s |  | Low Energy, Antispoofing, Antitampering |
| 5.7.2 | UAV (Remote control) | 5G positioning service area - Outdoor | 0.5m Horizontal 0.3m Vertical |  | 99 % |  |  | 150ms | Antispoofing Antitampering |
|  |  | Enhanced positioning area - Outdoor | 0.5m Horizontal 0.1m Vertical |  | 99.9 % |  |  | 150ms | Antispoofing Antitampering |
| 5.8.1 | Support multiple different location service | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s | Management of different KPI and positioning services |
|  |  | Enhanced positioning area - Indoor | 0.1m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.8.2 | Support location method negotiation | 5G positioning service area Outdoor/Indoor |  |  |  |  |  |  | Support + negotiation of positioning methods (incl. hybrid) |
| Note: most use cases also feature potential requirements on modes of operation, intended for the UE, the Network or for the 5G system. |  |  |  |  |  |  |  |  |  |

Among all use cases, the use cases requiring low energy or power saving mode include AR, wearable, waste management and UAV. For all these four use cases, it is clear that the devices need to report its position to the networks. Taking waste management as an example, the connected bins can be configured to send a message when they are full and trucks and vehicles in charge of the waste collection are equipped with positioning to locate these bins. Therefore, reporting measurement results is one of the main challenges for IDLE/INACTIVE state positioning. Secondly, as aforementioned, only UE based positioning can be applied without UE reporting, positioning algorithm needs to be implemented in UE side and such algorithms can be quite complicated and require high energy consumption. Finally, in RRC-INACTIVE state the context information is maintained by the UE and it is expected that the UE remains in the same location or with very low mobility.

Proposal 2: IDLE/INACTIVE state positioning should be studied considering the challenges of measurement reporting, low mobility requirement, etc.

# Angle based NLOS identification

As shown in our companion contribution [3], with increase NLOS probability the positioning accuracy is degraded significantly. In this regard, NLOS links should be differentiated from LOS links to avoid such performance degradation.

It is not always possible to have LOS link between the gNB and the UE. When NLOS link is considered as shown in the below figure, in addition to the UE location, reflector location is also unknown. In total, we have 5 unknown values including UE location (x,y), reflector location (x1,y1) and UE orientation. If only AoA or AoD at one side is measured, it is clearly not possible to locate the UE. Here we also consider signal strength in terms of RSRP, RSRQ or SINR and such information together with path loss information can be used to estimate the path length of the singal traveled from the gNB to the UE or the other way around. All measurements can be analysed jointly to identify the NLOS links.

gNB1

θ1

Reflector

θ2

φ1

Fig. 3 NLOS links

Proposal 3: Angle based LOS/NLOS differentiation with joint measurement should be studied.

# Uplink Transmission-Based Relative Positioning

In smart factories, with objects tightly stored, and moving tools (e.g. forklifts) trying to locate and move objects, it is important to accurately locate objects to avoid moving a wrong object. In our companion contribution [4], we discuss uplink transmission-based positioning in more details. The channel conditions (e.g. LOS vs NLOS) depend on the distance between two devices (e.g. base station and UE) as shown in [4], the larger the distance, the higher the NLOS probability and accordingly the less accurate the positioning estimate. While objects (e.g. object to be moved and forklift) whose position is being measured are relatively close to each other, they could be much further away from a base station. Furthermore, it is the relative position of these objects relative to each other, rather than the absolute position of each, that is of relevance. Hence, it would see natural to use relative positioning to determine the position of the objects relative to one another.

As stated in NOTE 1 of WID [1], “Sidelink is not part of this objective”. However, downlink and uplink channels on the NR Uu interface can be used for positioning. Uplink transmissions can be monitored and received by other UEs in the vicinity of the UE transmitting the uplink signal to assist in relative positioning.

Proposal 4: Uplink transmission-based relative positioning should be studied.

# Conclusions

In this contribution, we discussed the potential positioning enhancements for NR positioning. Our proposals are summarized below.

Proposal 1: New PRS pattern should be studied to avoid collision between multiple TRPs.

Proposal 2: IDLE/INACTIVE state positioning should be studied considering the challenges of measurement reporting, low mobility requirement, etc.

Proposal 3: Angle based LOS/NLOS differentiation with joint measurement should be studied.

Proposal 4: Uplink transmission-based relative positioning should be studied.

# References

- RP-193237, “NR Positioning Enhancements,” Qualcomm Incorporated.

- 3GPP TR 38.855, “Study on NR positioning support”

- R1-2003906, “Additional scenarios for evaluation,” Samsung.

- R1-2003909, “Uplink Transmission Based Relative Positioning”, Samsung, 3GPP RAN1#101e, May 2020.