3GPP TSG RAN WG1 #101-e	R1-2003915

# e-Meeting, May 25th – June 5th, 2020

Agenda Item:		8.4.1.2

Source:				Samsung

Title:					Scenarios and simulation assumptions for coverage enhancement in FR2

Document for:	Discussion

Introduction

In RAN#86 meeting, NR coverage enhancement SI was approved and the objective of coverage enhancement SI is listed below [1]:

The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows. - The target scenarios and services include - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1 - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2. - TDD and FDD for FR1. - VoIP and eMBB service for FR1. - eMBB service as first priority and VoIP as second priority for FR2. - LPWA services and scenarios are not included. - Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation - UL channels (including PUSCH and PUCCH) are prioritized for FR1. - Both DL and UL channels for FR2. - Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services - The target channels include at least PUSCH/PUCCH - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions) - Study the additional enhanced solutions for FR2 if any - Evaluate the performance of the potential solutions based on link level simulation.

As listed in the objective, link-level simulations are to be used for the identification of baseline coverage performance for both DL and UL for the various scenarios and services. This contribution mainly focuses on the simulation assumptions and preliminary results for FR2.

Evaluation methodology and simulation assumption

Evaluation methodology

As mentioned in our Tdoc regarding the evaluation assumption and preliminary results for FR1 [2], available path loss in link budget was chosen as the metric to evaluate the coverage for various scenarios in Rel-15 IMT 2020 self-evaluation. In order to take into account the different scenarios, the link budget methodology employed in IMT 2020 self-evaluation for 3GPP submission is also preferred for FR2. The detailed evaluation procedure for available path loss and corresponding maximum range is provided in the following table in Table A-1. The calculated maximum range would provide evidence to identify the coverage bottleneck of transmission channels in both UL and DL.

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR2 to take into account the different scenarios and channel.

Simulation assumptions for FR2

In TR 38.802 [3], there are link-level simulation assumptions for evaluation related to MIMO scheme including the parameters for FR 2. To decide the simulation parameters for FR2, in the beginning phase, most simulation parameters can be followed based on the parameters used in [3]. Similar to FR1, since the target scenarios and services focused in this SI are different, the transmission bit rate and the occupied channel bandwidth, BS and UE antenna configurations can be changed. In detail, general simulation parameters are summarized in below in Table 1 for the two scenarios, such as urban scenario and indoor scenario, respectively.

Table 1. General simulation parameters for FR2

|  | FR2 |  |
| --- | --- | --- |
|  | Urban | Indoor |
| System configuration | NR [30] GHz TDD | NR [30] GHz TDD |
| Physical channel | DL: PDCCH, PDSCH; UL: PUCCH, PUSCH |  |
| Channel state | NLOS O-to-I, NLOS | NLOS |
| Channel model | NLOS: CDL-A | NLOS: CDL-A |
|  | Delay spread: 100ns |  |
| SCS | [120] kHz | [120] kHz |
| UE speed | NLOS O-to-O: 30km/h, [120]km/h NLOS O-to-I: 3km/h | NLOS: 3km/h |
| # of gNB Ant. | [256] | [64] |
| # of TXRUs for gNB | 2 | 2 |
| # of UE Ant. | 4(DL) / 2(UL), [8 (DL/UL)] | 4 (DL) / 2 (UL), [8 (DL/UL)] |
| # of TxRUs for UE | 2 | 2 |

In detail, channel specific parameters are summarized in below in Table 2 for the four physical channel, such as PDSCH, PDCCH, PUSCH, and PUCCH, respectively.

Table 2. Channel specific parameters for FR2

|  | DL control channel | DL data channel | UL control channel | UL data channel |
| --- | --- | --- | --- | --- |
| Transmission bit rate (bit/s) | DCI format 1-0/0-0; DCI size = 68 bit; QPSK, aggregation level = 16 CCE | eMBB: [25Mbps] / [1Mbps] VoIP: 12.2kbps (304bits: 244 + 60 (header for RoHC compress)) | PUCCH Format 1 w/ 14 OFDM symbols, UCI 2 bit | eMBB: [5Mbps] / [100kbps] VoIP: 12.2kbps (304bits: 244 + 60 (header for RoHC compress)) |
| Occupied channel BW | 50MHz | 50MHz | 1 PRB | 4 PRB/30 PRB (for high data rate) |
| Performance target | 1% BLER | eMBB: 10% iBLER VoIP: 2% rBLER | 1% BLER | eMBB: 10% iBLER VoIP: 2% rBLER |
| Overhead assumption for DMRS | DMRS: - For 3km/h or 30km/h: Type I, one DMRS symbol, no multiplexing with data. - For 120km/h: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data. |  |  |  |

In detail, LLS parameters are summarized in below in Table 3 such as TXRU mapping, beam selection method, antenna configurations, and so on.

Table 3. LLS parameters for FR2

|  | FR2 |
| --- | --- |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| BS antenna configurations | (M,N,P,Mg,Ng) = [(8,16,2,1,1) for Urban, (4,8,2,1,1) for indoor] for 1 panel, [(4,8,2,2,2) for Urban] for multiple panels, (dV,dH) = (0.5, 0.5)λ |
| BS antenna element radiation pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng) = (1,1,2,1,1) for UL & (1,2,2,1,1) for DL; [(2,2,2,1,1) or (4,1,2,1,1)] if # of UE Ant. =8, (dV,dH) = (0.5, 0.5)λ. |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| BF scheme | Analog BF based on beam selection + Digital BF |

Proposal 2. Consider  link-level simulation assumptions of TR38.802 including Table 1, 2, and 3 as the starting point for coverage enhancement evaluation for FR2 case.

Preliminary results

The following link budget results using link budget template in Table A-1 have been performed based on the simulation parameter values in Table 1, 2, and 3. In addition, the required SNR in the link budget template is calculated based on the link-level simulation at performance target for each channel in Table 2. The baseline performance in dB and coverage distance in meter are defined as available path-loss (29) and maximum range (30) in link budget template in Table A-1, respectively. ISD in meter is evaluated based on the coverage distance in meter. The target performance uses the expected ISD for each scenario (e.g., 500m for urban and 20m for indoor) to compute the target path loss from the path-loss model in [Section 7.4.1, 4] and corresponding performance gap represented as: Gap = target path loss – available path loss in baseline performance.

Preliminary results for urban scenario at 30GHz

In this section, baseline performance is provided for urban scenario with 30GHz carrier frequency and TDD system. We assume that BS antenna configuration is (M,N,P,Mg,Ng) = (8,16,2,1,1), UE antenna configuration is (M,N,P,Mg,Ng) = (1,1,2,1,1) for UL, and # of UE antenna elements is 2. In addition, we consider UE speed is 30km/h in NLOS O-to-O channel. For urban scenario, the target ISD is 500m and thus the target performance for ISD=500m is calculated based on the path loss model in [Section 7.4.1, 4] as 139.24 dB in terms of path loss.

Table 4. Preliminary results for urban scenario at FR2

| Carrier | Scenario | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Baseline Performance (dB) | Coverage distance/ISD (m) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | ISD= 500m | Gap |
| 30GHz | Urban | 5Mbps for eMBB | DDDSUDDSUU | NLOS(O-to-I) | 30 | -8.3 | 107.38 | 44.2/76.5 | 139.24 | 31.86 |
|  |  |  |  | NLOS(O-to-O) |  | -8.9 | 129.68 | 164.2/284.4 |  | 9.56 |
|  |  |  | DDDSU | NLOS(O-to-I) |  | -7.3 | 106.38 | 41.6/72.1 |  | 32.86 |
|  |  |  |  | NLOS(O-to-O) |  | -7.8 | 128.58 | 153.9/266.6 |  | 10.66 |

As observed in Table 4, there are 31.86 dB and 9.56 dB gap from target ISD=500m for PUSCH with 5Mbps eMBB service in case of TDD configuration with DDDSUDDSUU for NLOS O-to-I and NLOS O-to-O channel, respectively. In addition, there are 32.86 dB and 10.66 dB gap from target ISD=500m for PUSCH with 5Mbps eMBB service in case of TDD configuration with DDDSU for NLOS O-to-I and NLOS O-to-O channel, respectively. For urban scenario with NLOS O2I/O2O channel, coverage distance of PUSCH with eMBB is about 42/158m (ISD=74/275 m). Based on the results, it is observed that for urban scenario for FR2, PUSCH enhancement in eMBB is significantly needed with ISD=500m.

Observation 1. For urban scenario for FR2, PUSCH enhancement in eMBB is significantly needed with ISD=500m.

Preliminary results for indoor scenario at 30GHz

In this section, baseline performance is provided for indoor scenario with 30 GHz carrier frequency and TDD system. We assume that BS antenna configuration is (M,N,P,Mg,Ng) = (4,8,2,1,1), UE antenna configuration is (M,N,P,Mg,Ng) = (1,1,2,1,1) for UL, and # of UE antenna elements is 2. In addition, we consider UE speed is 30km/h in NLOS O-to-O channel. For indoor scenario, the target ISD is 20m and thus the target performance for ISD=20m is calculated based on the path loss model in [Section 7.4.1, 4] as 94.77 dB in terms of path loss.

Table 5. Preliminary results for indoor scenario at 30GHz

| Carrier | Scenario | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Baseline Performance (dB) | Coverage distance/ISD (m) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | ISD=20m | Gap |
| 30GHz | Indoor | 5Mbps for eMBB | DDDSUDDSUU | NLOS | 30 | -5.5 | 132.83 | 197.7/342.4 | 94.77 | -38.06 |
|  |  |  | DDDSU |  |  | -4.5 | 131.83 | 186.4/322.8 |  | -37.06 |

As observed in Table 5, there are -38.06 dB and -37.06 dB gap from target ISD=20m for PUSCH with 5Mbps eMBB service in case of TDD configuration with DDDSUDDSUU and DDDSU, respectively. For indoor scenario with NLOS channel, coverage distance of PUSCH with eMBB is about 190m (ISD=330m). Based on the results, it is observed that for indoor scenario at 30 GHz, PUSCH enhancement in both eMBB is not needed with ISD=20m.

Observation 2. For indoor scenario at 30 GHz, PUSCH enhancement in eMBB service is not needed with ISD=20m.

Conclusion

This contribution discusses the evaluation methodology and simulation assumption for FR2 to identify the baseline coverage performance. In addition, we provide the preliminary results for target scenarios. Following observations and proposals are made:

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR2  to take into account the different scenarios and channel.

Proposal 2. Consider  link-level simulation assumptions of TR38.802 as the starting point for coverage enhancement evaluation for FR2 case.

Observation 1. For urban scenario for FR2, PUSCH enhancement in eMBB is significantly needed with ISD=500m.

Observation 2. For indoor scenario at 30 GHz, PUSCH enhancement in eMBB service is not needed with ISD=20m.

- References

- RP-193240, New SID on NR coverage enhancement, RAN#86, Dec. 2019.

- R1-2003914, Evaluation methodology and assumption for FR1, Samsung, RAN1#101-e, May 2020.

- TR 38.802, Study on new radio access technology Physical layer aspects (Release 14).

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16).

- Appendix

Table A-1. Link budget template in [2]

| Parameter | Values |
| --- | --- |
| Scenario |  |
| Frame structure |  |
| Carrier frequency (Hz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability for control channel |  |
| Cell area reliability for data channel |  |
| Transmission bit rate for control channel (bit/s) |  |
| Transmission bit rate for data channel (bit/s) |  |
| Target packet error rate for the required SNR in item (19a) for control channel |  |
| Target packet error rate for the required SNR in item (19b) for data channel |  |
| Spectral efficiency (bit/s/Hz) |  |
| Pathloss model (select from LoS or NLoS) |  |
| UE speed (km/h) |  |
| Feeder loss (dB) |  |
| Transmitter |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (2) Maximal transmit power per antenna (dBm) |  |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) |  |
| (4) Transmitter antenna gain (dBi) |  |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) |  |
| (6) Control channel power boosting gain (dB) |  |
| (7) Data channel power loss due to pilot/control boosting (dB) |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm |  |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (11) Receiver antenna gain (dBi) |  |
| (11bis) Receiver array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, etc.) (dB) |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15a) Receiver interference density for control channel (dBm/Hz) |  |
| (15b) Receiver interference density for data channel (dBm/Hz) |  |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz |  |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz |  |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) |  |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) |  |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm |  |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm |  |
| (19a) Required SNR for the control channel (dB) |  |
| (19b) Required SNR for the data channel (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21a) H-ARQ gain for control channel (dB) |  |
| (21b) H-ARQ gain for data channel (dB) |  |
| (22a) Receiver sensitivity for control channel = (18a) ++ (19a) + (20) – (21a) dBm |  |
| (22b) Receiver sensitivity for data channel = (18b) ++ (19b) + (20) – (21b) dBm |  |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11bis) − (22a) dB |  |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11bis) − (22b) dB |  |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) |  |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) |  |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) – (12) dB |  |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) – (12) dB |  |
| Range/coverage efficiency calculation |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) |  |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) |  |