3GPP TSG RAN WG1 #101	   	 	  	                	      R1-2003928

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.6.2

Source:	LG Electronics

Title:	Text proposals on enhancements on multi-TRP/panel transmission

Document for:	Discussion and Decision

Introduction

In this contribution, we share our proposals on the maintenance for enhancements for multi-TRP/panel transmission based on the previous agreements.

Discussion

Clarification of default beam assumption for multi-DCI based multi-TRP/panel transmission

Based on current TS38.214, if a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE assumes two default TCIs, i.e. one for each CORESETPoolIndex. However, this default beam assumption can be supported only for the UE which is capable of supporting two default TCIs, e.g. UEs with two Rx panels that is capable of buffering with two different Rx beams simultaneously. As this feature requires a specific UE implementation which would consume UE battery faster, it was also agreed in RAN1#99 that single default TCI should be applied if the UE reports gNB that it is not capable of supporting two default TCIs regardless of the number of CORESET pools, i.e. buffering with single Rx beam, as highlighted below.

Agreement@#99

For multi-DCI based multi-TRP/panel transmission, if CORESETPoolIndex is configured,

- If the time offset between the reception of the PDCCH and the corresponding PDSCH is less than a threshold, UE could assume that the DM-RS ports of PDSCH are QCL-ed with the RS(s) with respect to the QCL parameter(s) used for PDCCH of the lowest CORESET index among CORESETs configured with the same value of CORESETPoolIndex,

  - in the respective latest slot in which one or more CORESETs associated with each of CORESETPoolIndex within the active BWP of the serving cell are monitored by the UE

    - The support of this feature is indicated by UE capability

    - If the UE does not support the above feature, Rel-15 behavior is reused regardless of CORESETPoolIndex

If this is not reflected into the specification, the specification would mandate all UEs to support two different default beams which is a huge burden for UE implementation. In addition, in RAN1#100bis-e meeting, it was agreed to introduce the related UE capability in FG 16-2a-6. Therefore, default beam assumption for those UEs should also be defined. Based on the previous agreement, we propose the following text proposal.

Proposal #1: Adopt the following TP to clause 5.1.5 of TS38.214.

### 5.1.5 Antenna ports quasi co-location ---- Unchanged text are omitted ---- Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if all the TCI codepoints are mapped to a single TCI state and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, if the UE is capable of supporting two default TCIs, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE, and if the UE is not capable of supporting two default TCIs, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. If the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. ---- Unchanged text are omitted ----

Configuration for MTRP based URLLC transmission schemes

According to the following agreement, which is highlighted in yellow, Condition 4 is agreed for FDMSchemeA, FDMSchemeB, or TDMSchemeA in order not to support dynamic switching with TDMSchemeB, i.e., scheme 4 and not to support mixed transmission scheme with TDMSchemeB.

Agreement

Following TCI state and joint schemes are supported

|  | TCI states | CDM groups | URLLCRepNum | URLLCSchemeEnabler | UE Behavior |
| --- | --- | --- | --- | --- | --- |
| 0 (in spec draft) | 1 | >=1 | Not applicable | Not applicable | Rel 15 |
| A (one scheme) | 1 | 1 | Condition 1 | Configured or not configured | "Scheme 4" with repetition from the same TRP Limitations agreed for Scheme 4 apply |
| A’ (one scheme) | 1 | >=1 | Condition 2 | Not configured | Rel 15 |
| B (in spec draft) | 2 | 1 | Condition 1 | Not configured | Scheme 4 |
| C (in spec draft) | 2 | 2 | Condition 2 | Not configured | 1a/NCJT |
| E (in spec draft) | 2 | 2 | Condition 4 | Not configured | 1a/NCJT |
| F (in spec draft) | 2 | 1 | Condition 4 | Configured | Scheme 2a/2b/3 |
| D’’ (one scheme) | 2 | 2 | Condition 4 | Configured | 1a/NCJT |
| G’ (one scheme) | 1 | >=1 | Condition 2 | Configured | Rel 15 |
| G (one scheme) | 1 | >=1 | Condition 4 | Configured | Rel 15 |

Note:

- Condition 1: indicates one entry in pdsch-TimeDomainAllocationList containing URLLCRepNum (>1) in TDRA by DCI

- Condition 2: indicates one entry in pdsch-TimeDomainAllocationList having no URLLCRepNum by DCI, but at least one entry having URLLCRepNum

- Condition 4: None of entry in TDRA contains URLLCRepNum

However, Condition 4 is not captured in the current specification. As a result, FDMSchemeA, FDMSchemeB, or TDMSchemeA can be applied even when at least one entry in TDRA contains URLLCRepNum so that both TDMSchemeB and one of FDMSchemeA, FDMSchemeB, or TDMSchemeA can be applied at the same time.

For example, according to the current specification, conditions for TDMSchemeA and TDMSchemeB can be satisfied at the same time when # of TCI states=2, # of CDM groups=1, RepSchemeEnabler= TDMSchemeA and RepNumR16 is configured, which is highlighted in yellow. However, the agreement does not support mixing TDM A and B and dynamic switching between TDM A, and B. For this purpose we have agreed Condition 4 for FDMSchemeA, FDMSchemeB, or TDMSchemeA. Based on this background, Condition 4 should be captured.

5.1.2.1 Resource allocation in time domain When a UE is configured by the higher layer parameter RepSchemeEnabler set to ‘TDMSchemeA’ and indicated DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’, the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Subclause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Subclause 5.1.2.1. When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the same SLIV is applied for all PDSCH transmission occasions, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Subclause 5.1.2.1. (Omit the remaining parts)

Based on the previous agreement, we propose the following text proposal.

Proposal #2: Adopt the following TP to clause 5.1 of TS38.214.

## 5.1 UE procedure for receiving the physical downlink shared channel ---- Unchanged text are omitted ---- When a UE is configured by higher layer parameter RepSchemeEnabler set to one of 'FDMSchemeA', 'FDMSchemeB', 'TDMSchemeA' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) withina one CDM group in the DCI field "Antenna Port(s)". - When two TCI states are indicated in a DCI and the UE is set to 'FDMSchemeA', the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to 'FDMSchemeB', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to 'TDMSchemeA', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Clause 5.1.2.1. ---- Unchanged text are omitted ---- #### 5.1.2.1 Resource allocation in time domain ---- Unchanged text are omitted ---- When a UE is configured by the higher layer parameter RepSchemeEnabler set to 'TDMSchemeA' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16 and indicated DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. - If two TCI states are indicated by the DCI field 'Transmission Configuration Indication', the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1. ---- Unchanged text are omitted ---- #### 5.1.2.3 Physical resource block (PRB) bundling ---- Unchanged text are omitted ---- For a UE configured by the higher layer parameter RepSchemeEnabler set to 'FDMSchemeA' or 'FDMSchemeB' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", - If is determined as "wideband", the first nPRB2 PRBs are assigned to the first TCI state and the remaining nPRB2 PRBs are assigned to the second TCI state, where nPRB is the total number of allocated PRBs for the UE. - If is determined as one of the values among {2, 4}, even PRGs within the allocated frequency domain resources are assigned to the first TCI state and odd PRGs within the allocated frequency domain resources are assigned to the second TCI state. - The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For a UE configured by the higher layer parameter RepSchemeEnabler set to 'FDMSchemeB' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", each PDSCH transmission occasion shall follow the Clause 7.3.1 of [4, TS 38.211] with the mapping to resource elements determined by the assigned PRBs for corresponding TCI state of the PDSCH transmission occasion, and the UE shall only expect at most two code blocks per PDSCH transmission occasion when a single transmission layer is scheduled and a single code block per PDSCH transmission occasion when two transmission layers are scheduled. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 are applied to the first and second TCI state, respectively. ---- Unchanged text are omitted ---- #### 5.1.3.1 Modulation order and target code rate determination ---- Unchanged text are omitted ---- For a UE configured with FDMSchemeB and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the determined modulation order of PDSCH transmission occasion associated with the first TCI state is applied to the PDSCH transmission occasion associated with the second TCI state. ---- Unchanged text are omitted ---- #### 5.1.3.2 Transport block size determination ---- Unchanged text are omitted ---- For a UE configured with FDMSchemeB and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16 and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the TBS determination follows the steps 1-4 with the following modification in step 1: a UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs corresponding to the first TCI state. and the determined TBS of PDSCH transmission occasion associated with the first TCI state is also applied to the PDSCH transmission occasion associated with the second TCI state. ---- Unchanged text are omitted ---- #### 5.1.6.3 PT-RS reception procedure ---- Unchanged text are omitted ---- When a UE configured by the higher layer parameter RepSchemeEnabler set to 'FDMSchemeA' or 'FDMSchemeB' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the UE shall receive a single PT-RS port which is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH, a PT-RS frequency density is determined by the number of PRBs associated to each TCI state, and a PT-RS resource element mapping is associated to the allocated PRBs for each TCI state. ---- Unchanged text are omitted ----

Conclusion

In this contribution, we discuss our views for maintenance for enhancements for multi-TRP/panel transmission based on the previous agreements, and propose the followings based on the discussion.

Proposal #1: Adopt the following TP to clause 5.1.5 of TS38.214.

### 5.1.5 Antenna ports quasi co-location ---- Unchanged text are omitted ---- Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if all the TCI codepoints are mapped to a single TCI state and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, if the UE is capable of supporting two default TCIs, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE, and if the UE is not capable of supporting two default TCIs, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. If the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. ---- Unchanged text are omitted ----

Proposal #2: Adopt the following TP to clause 5.1 of TS38.214.

## 5.1 UE procedure for receiving the physical downlink shared channel ---- Unchanged text are omitted ---- When a UE is configured by higher layer parameter RepSchemeEnabler set to one of 'FDMSchemeA', 'FDMSchemeB', 'TDMSchemeA' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) withina one CDM group in the DCI field "Antenna Port(s)". - When two TCI states are indicated in a DCI and the UE is set to 'FDMSchemeA', the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to 'FDMSchemeB', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to 'TDMSchemeA', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Clause 5.1.2.1. ---- Unchanged text are omitted ---- #### 5.1.2.1 Resource allocation in time domain ---- Unchanged text are omitted ---- When a UE is configured by the higher layer parameter RepSchemeEnabler set to 'TDMSchemeA' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16 and indicated DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. - If two TCI states are indicated by the DCI field 'Transmission Configuration Indication', the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1. ---- Unchanged text are omitted ---- #### 5.1.2.3 Physical resource block (PRB) bundling ---- Unchanged text are omitted ---- For a UE configured by the higher layer parameter RepSchemeEnabler set to 'FDMSchemeA' or 'FDMSchemeB' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", - If is determined as "wideband", the first nPRB2 PRBs are assigned to the first TCI state and the remaining nPRB2 PRBs are assigned to the second TCI state, where nPRB is the total number of allocated PRBs for the UE. - If is determined as one of the values among {2, 4}, even PRGs within the allocated frequency domain resources are assigned to the first TCI state and odd PRGs within the allocated frequency domain resources are assigned to the second TCI state. - The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For a UE configured by the higher layer parameter RepSchemeEnabler set to 'FDMSchemeB' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", each PDSCH transmission occasion shall follow the Clause 7.3.1 of [4, TS 38.211] with the mapping to resource elements determined by the assigned PRBs for corresponding TCI state of the PDSCH transmission occasion, and the UE shall only expect at most two code blocks per PDSCH transmission occasion when a single transmission layer is scheduled and a single code block per PDSCH transmission occasion when two transmission layers are scheduled. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 are applied to the first and second TCI state, respectively. ---- Unchanged text are omitted ---- #### 5.1.3.1 Modulation order and target code rate determination ---- Unchanged text are omitted ---- For a UE configured with FDMSchemeB and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the determined modulation order of PDSCH transmission occasion associated with the first TCI state is applied to the PDSCH transmission occasion associated with the second TCI state. ---- Unchanged text are omitted ---- #### 5.1.3.2 Transport block size determination ---- Unchanged text are omitted ---- For a UE configured with FDMSchemeB and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16 and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the TBS determination follows the steps 1-4 with the following modification in step 1: a UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs corresponding to the first TCI state. and the determined TBS of PDSCH transmission occasion associated with the first TCI state is also applied to the PDSCH transmission occasion associated with the second TCI state. ---- Unchanged text are omitted ---- #### 5.1.6.3 PT-RS reception procedure ---- Unchanged text are omitted ---- When a UE configured by the higher layer parameter RepSchemeEnabler set to 'FDMSchemeA' or 'FDMSchemeB' and configured by the higher layer parameter PDSCH-config that indicates none of entries in pdsch-TimeDomainAllocationList contain RepNumR16, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the UE shall receive a single PT-RS port which is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH, a PT-RS frequency density is determined by the number of PRBs associated to each TCI state, and a PT-RS resource element mapping is associated to the allocated PRBs for each TCI state. ---- Unchanged text are omitted ----