3GPP TSG RAN WG1 #101                          	                                 R1-2003947

e-Meeting, May 25th - June 5th, 2020

Agenda Item:	7.2.1.2

Title:	Remaining issue on 2-step RA procedure triggered by BFR

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In this contribution, we would address the remaining issue on 2-step RA procedure triggered by BFR.

- Beam failure recovery through 2-step RACH

In Rel-16, the issue of 2-step RACH has been discussed in RAN1 and RAN2.

According to the latest version of TS 38.321 [1], RRC configures the parameters for 2-step RA procedure, for example:

-	msgA-prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for MSGA in 2-step RA type;

-	msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type. If the Random Access procedure is initiated for beam failure recovery, msgA-RSRP-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to msgA-RSRP-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	msgA-RSRP-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 2-step RA type. If the Random Access procedure is initiated for beam failure recovery, msgA-RSRP-ThresholdCSI-RS is equal to msgA-RSRP-ThresholdSSB in BeamFailureRecoveryConfig IE;

The parameters msgA-RSRP-ThresholdSSB and msgA-RSRP-ThresholdCSI-RS are related to the threshold for candidate beam selection if the procedure is initiated for beam failure recovery.

According to the parameters captured in TS 38.321, BFR can trigger a 2-step RA procedure. However, the related parameters for BFR through 2-step RACH are not captured in TS 38.213. Hence, we propose the following Text proposal in TS 38.213.

------------------------------------------Start of Text Proposal to TS 38.213 v16.1.0---------------------------------- ### 6 Link recovery procedures ### --------------- Unchanged parts omitted ------------- The thresholds Qout,LR and Qin,LR correspond to the default value of rlmInSyncOutOfSyncThreshold, as described in [10, TS 38.133] for Qout, and to the value provided by rsrp-ThresholdSSB or msgA-RSRP-ThresholdSSB or rsrp-ThresholdSSBBFR, respectively. ### --------------- Unchanged parts omitted ------------- ### --------------------------------------- End of Text Proposal ------------------------------------

- Conclusion

In this contribution, remaining issue on 2-step RA procedure triggered by BFR has been discussed. Based on the discussion, we have the following Text proposal in TS 38.213 :

Proposal 1: The following changes should be captured in TS 38.213

------------------------------------------Start of Text Proposal to TS 38.213 v16.1.0---------------------------------- ### 6 Link recovery procedures ### --------------- Unchanged parts omitted ------------- The thresholds Qout,LR and Qin,LR correspond to the default value of rlmInSyncOutOfSyncThreshold, as described in [10, TS 38.133] for Qout, and to the value provided by rsrp-ThresholdSSB or msgA-RSRP-ThresholdSSB or rsrp-ThresholdSSBBFR, respectively. ### --------------- Unchanged parts omitted ------------- ### --------------------------------------- End of Text Proposal ------------------------------------

References

- 3GPP TS 38.321 v16.0.0: "NR; Medium Access Control (MAC) protocol specification"