3GPP TSG RAN WG1 Meeting #101  			    			            R1-2003973

e-Meeting, May 25th – June 5th, 2020

# Source:	ETRI

# Title:		Remaining issues on initial access procedure for NR-U

# Agenda item:	7.2.2.2.2

# Document for:	Discussion/Decision

Introduction

In this contribution, we introduce our view on a remaining issue on initial access procedure for NR-U.

NR-U initial access procedures

PDSCH rate-matching around candidate SSBs

Regarding the relationship between SSB index and candidate SSB index was discussed and agreed, and the impacts of specifications were summarized in [1]. Most of those have been captured in the related specification. However, the index #22 is still missed as shown in Appendix, even though couple of contributions mentioned on the issue in the last meeting [2][3]. Therefore, it is proposed to adopt the following TP in order to address the remaining issue for indexing SSB for NR-U.

Proposal 1: The following TP should be adopted to address the remaining issue for indexing SSBs.

TS 38.214 Subclause 5.1.4, Start of Text Proposal ---------------------------------------------

### 5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents candidate SS/PBCH blocks corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213].

TS 38.214 Subclause 5.1.4, End of Text Proposal ---------------------------------------------

Summary

In this contribution, we made the following proposal for the remaining issue on initial access procedure for NR-U.

Proposal 1: The following TP should be adopted to address the remaining issue for indexing SSBs.

TS 38.214 Subclause 5.1.4, Start of Text Proposal ---------------------------------------------

### 5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents candidate SS/PBCH blocks corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213].

TS 38.214 Subclause 5.1.4, End of Text Proposal ---------------------------------------------

References

- 3GPP, TS38.213 v16.1.0, Mar. 2020.

- R1-1913538, “Summary of discussion on SS/PBCH block indexing,” Samsung

- R1-2001936, “Remaining issues of initial access and mobility for NR-U”, LG Electronics

- R1-2002248, “Remaining issues on initial access procedure for NR-U”, ETRI

- Appendix

| Index | Spec | Session | Topic | Detailed description in Rel-15 | RAN1 consensus | Other comments |
| --- | --- | --- | --- | --- | --- | --- |
| 22 | 38.214 | 5.1.4 | PDSCH resource mapping | When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. | RAN1’s intention is, the UE assumes SS/PBCH block transmission according to candidate SS/PBCH block index(es) corresponding to SS/PBCH block index(es) provided by ssb-PositionsInBurst. |  |